3GPP TSG-RAN5 Meeting #90-e						R5-210260

Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0141 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections and clarifications to default MBS, WLAN and Sensor assistance data in clause 5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | PCTEST Engineering Laboratory |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The MBS, WLAN and Sensor Assistance Data Elements in clauses 5.4.1.3, 5.4.1.4 and 5.4.1.5 are confusing and in the case of WLAN, incorrect. 1. In all cases there are two Tables, however the introductions only mention one Table and it is not clear how the other Table should be used. 2. The Table titles are confusing and inconsistent. 3. In the Value/remark columns, some IEs are missing necessary explanations. 4. The wlan-AP-Identifier is a mandatory IE and therefore should not be listed as a conditional IE in Table 5.4.1.4-1. 5. The IEs period-v1520 and area-v1520 are missing from the Sensor-AssistanceDataList. 6. In clauses 5.4 and 8.4 the references for sensor-ProvideAssistanceData, tbs-ProvideAssistanceData and wlan-ProvideAssistanceData are confusing as they only refer to one (of the two) Tables in clause 5.4.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. In all cases the introductions are modified for clarity. 2. The Table titles are clarified. 3. In the Value/remark columns, missing explanations are added 4. The wlan-AP-Identifier is deleted in Table 5.4.1.4-1. 5. The IEs period-v1520 and area-v1520 are added. 6. The references for sensor-ProvideAssistanceData, tbs-ProvideAssistanceData and wlan-ProvideAssistanceData are clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of the MBS, WLAN and Sensor Assistance Data Elements will remain confusing and in the case of WLAN, incorrect. This could lead to incorrect coding and failing of compliant UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4, 5.4.1.3, 5.4.1.4, 5.4.1.5, 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of Modified Section >

## 5.4	Default LPP message and information elements contents

This clause contains the default values of LPP messages and information elements used, unless indicated otherwise in specific clauses of this specification.

-	LPP REQUEST CAPABILITIES

Table 5.4-1: RequestCapabilities

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| Initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestCapabilities SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestCapabilities-r9 SEQUENCE { |  |  |  |
| commonIEsRequestCapabilities SEQUENCE { | Present | Rel-14 onwards |  |
| lpp-message-segmentation-req-r14 | 00 | Server is not able to send segmented LPP messages. Server is not able to receive segmented LPP messages. Rel-14 onwards |  |
| } |  |  |  |
| a-gnss-RequestCapabilities SEQUENCE { |  |  |  |
| gnss-SupportListReq | TRUE |  |  |
| assistanceDataSupportListReq | TRUE |  |  |
| locationVelocityTypesReq | TRUE |  |  |
| } |  |  |  |
| otdoa-RequestCapabilities SEQUENCE { | Present |  |  |
| } |  |  |  |
| ecid-RequestCapabilities SEQUENCE { | Present |  |  |
| } |  |  |  |
| epdu-RequestCapabilities | Not present |  |  |
| sensor-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| tbs-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| wlan-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| bt-RequestCapabilities-r13 SEQUENCE { | Present | Rel-13 onwards |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	LPP PROVIDE ASSISTANCE DATA

Table 5.4-2: ProvideAssistanceData

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { | Dependent on test case. |  |  |
| initiator |  |  |  |
| transactionNumber |  |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 7 and 15 only; and as defined in Table 5.4.1.1-1. |
| gnss-CommonAssistData SEQUENCE { |  |  |  |
| gnss-ReferenceTime | As defined in 37.571-5 [12] |  |  |
| gnss-ReferenceLocation | As defined in 37.571-5 [12] |  |  |
| gnss-IonosphericModel | As defined in 37.571-5 [12] |  |  |
| gnss-EarthOrientationParameters | Not present |  |  |
| } |  |  |  |
| gnss-GenericAssistData(SIZE(1..4))OF{ | SIZE is dependent on the number of GNSSs supported by the UE. If one GNSS supported by the UE, SIZE = 1 If two GNSSs supported by the UE, SIZE = 2 If three GNSSs supported by the UE, SIZE = 3 If four GNSSs supported by the UE, SIZE = 4 |  |  |
| gnss-ID | For each GNSS supported by the UE. |  |  |
| sbas-ID | Not present |  |  |
| gnss-TimeModels | As defined in 37.571-5 [12] |  |  |
| gnss-DifferentialCorrections | Not present |  |  |
| gnss-NavigationModel | As defined in 37.571-5 [12] |  |  |
| gnss-RealTimeIntegrity | Not present |  |  |
| gnss-DataBitAssistance | Not present |  |  |
| gnss-AcquisitionAssistance | As defined in 37.571-5 [12] |  |  |
| gnss-Almanac | As defined in 37.571-5 [12] |  |  |
| gnss-UTC-Model | As defined in 37.571-5 [12] |  |  |
| gnss-AuxiliaryInformation | As defined in 37.571-5 [12] |  |  |
| } |  |  |  |
| gnss-Error | Not present |  |  |
| } |  |  |  |
| otdoa-ProvideAssistanceData SEQUENCE { |  |  | Sub-test 5 and 7 only |
| otdoa-ReferenceCellInfo | As defined in Table 5.4.1.2-1 |  |  |
| otdoa-NeighbourCellInfo | As defined in Table 5.4.1.2-2 |  |  |
| otdoa-Error | Not present |  |  |
| } |  |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 18 only as defined in clause 5.4.1.5 |
| sensor-AssistanceDataList-r14 | As defined in Table 5.4.1.5-2 |  |  |
| sensor-Error-r14 | Not present |  |  |
| } |  |  |  |
| tbs-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 16 only as defined in clause 5.4.1.3 |
| tbs-AssistanceDataList-r14 SEQUENCE { |  |  |  |
| mbs-AssistanceDataList-r14 SEQUENCE (SIZE(1..n)) OF SEQUENCE{ |  |  |  |
| mbs-AlmanacAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| mbs-AcquisitonAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| tbs-Error-r14 | Not present |  |  |
| } |  |  |  |
| wlan-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 17 only as defined in clause 5.4.1.4 |
| wlan-DataSet-r14 | As defined in Table 5.4.1.4-2 |  |  |
| wlan-Error-r14 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# < Next Modified Section >

### 5.4.1	Default assistance data information elements

#### 5.4.1.1	GNSS Assistance Data Elements

Table 5.4.1.1-1 defines the GNSS assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message. The GNSS assistance data provided depends on the mode being used in the test case, the assistance data supported by the UE and the GNSSs supported by the UE. GNSS assistance data IEs not supported by the UE shall not be sent. GNSS assistance data IEs supported by the UE but not listed in Table 5.4.1.1-1 shall not be sent. The content of the assistance data elements is defined in 37.571-5 [12] clause 7.

Table 5.4.1.1-1: GNSS assistance data to be provided to the UE

| GNSS Assistance Data IE supported by UE | Mode used in test case |  |  |
| --- | --- | --- | --- |
|  | UE-based | UE-assisted, GNSS-AcquisitionAssistance supported by UE | UE-assisted, GNSS-AcquisitionAssistance not supported by UE |
| GNSS-Reference Time | Yes | Yes | Yes |
| GNSS-ReferenceLocation | Yes | No | Yes |
| GNSS-IonosphericModel | Yes | No | No |
| GNSS-TimeModelList | Yes(1) | No | Yes(1) |
| GNSS-NavigationModel | Yes | No | Yes |
| GNSS-AcquisitionAssistance | No | Yes | No |
| GNSS-Almanac | No | No | Yes |
| GNSS-UTC-Model | Yes(3) | Yes(3) | Yes(3) |
| GNSS-AuxiliaryInformation | Yes(2) | Yes(2) | Yes(2) |
| NOTE 1: Only if more than one GNSS supported by the UE. NOTE 2: Only if GLONASS supported by the UE, and/or if the UE supports multiple signals per GNSS, and/or if BDS B1C signal type supported by the UE. NOTE 3: Only if GLONASS and at least one other GNSS supported by the UE. |  |  |  |

#### 5.4.1.2	OTDOA Assistance Data Elements

This subclause defines the OTDOA assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages.

#### -	OTDOA REFERENCE CELL INFO

Table 5.4.1.2-1: OTDOA-ReferenceCellInfo

| Derivation Path: 36.355 clause 6.5.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OTDOA-ReferenceCellInfo ::= SEQUENCE { |  | Cell 1 |  |
| physCellId | 0 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 1 in 36.508 [8] |  |  |
| mnc | As defined for Cell 1 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 1 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcnRef | Not present | Same as the serving cell |  |
| antennaPortConfig | Not present | Same as the serving cell |  |
| cpLength | Normal |  |  |
| prsInfo SEQUENCE { |  |  |  |
| prs-Bandwidth | PRS are transmitted over the used system bandwidth (see subclause 5.2.2) |  |  |
| prs-ConfigurationIndex | FDD: 2 TDD: 4 |  |  |
| numDL-Frames | sf-1 |  |  |
| prs-MutingInfo-r9 | Not present | PRS muting is not used. |  |
| prsID-r14 | Not present | PRS-ID not used | Rel-14 onwards |
| add-numDL-Frames-r14 | Not present | Not required | Rel-14 onwards |
| prsOccGroupLen-r14 | Not present | No PRS occasion group configured | Rel-14 onwards |
| prsHoppingInfo-r14 | Not present | PRS frequency hopping not used | Rel-14 onwards |
| } |  |  |  |
| earfcnRef-v9a0 | Not present | Same as the serving cell |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| cpLengthCRS-r14 | Normal |  | Rel-14 onwards |
| sameMBSFNconfigRef-r14 | TRUE | Same as the serving cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | Same as the serving cell and PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigRef-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| } |  |  |  |

#### -	OTDOA NEIGHBOUR CELL INFO LIST

Table 5.4.1.2-2: OTDOA-NeighbourCellInfoList

| Derivation Path: 36.355 clause 6.5.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE(1)) OF SEQUENCE { |  |  |  |
| SEQUENCE (SIZE(2)) OF SEQUENCE { |  | Cell 2 |  |
| physCellId | 2 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 2 in 36.508 [8] |  |  |
| mnc | As defined for Cell 2 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 2 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcn | Not present | Same as for the reference cell |  |
| cpLength | Not present | Same as for the reference cell |  |
| prsInfo | Not present | Same as for the reference cell |  |
| antennaPortConfig | Not present | Same as for the reference cell |  |
| slotNumberOffset | Not present | Slot timing is the same as for reference cell |  |
| prs-SubframeOffset | Not present |  |  |
| expectedRSTD | 8192 | Value 0 |  |
| expectedRSTD-Uncertainty | 10 | About 1 s |  |
| earfcn-v9a0 | Not present | Same as for the reference cell |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| prs-only-tp-r14 | Not present | Not required | Rel-14 onwards |
| cpLengthCRS-r14 | Not present | Not required | Rel-14 onwards |
| sameMBSFNconfigNeighbour-r14 | TRUE | Same as for the reference cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | Same as for the reference cell and PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigNeighbour-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| } |  |  |  |
| SEQUENCE { |  | Cell 4 |  |
| physCellId | 4 |  |  |
| cellGlobalId SEQUENCE { |  |  |  |
| mcc | As defined for Cell 4 in 36.508 [8] |  |  |
| mnc | As defined for Cell 4 in 36.508 [8] |  |  |
| cellidentity | As defined for E-UTRAN Cell Identifier for Cell 4 in 36.508 [8] |  |  |
| } |  |  |  |
| earfcn | Not present | Same as for the reference cell |  |
| cpLength | Not present | Same as for the reference cell |  |
| prsInfo | Not present | Same as for the reference cell |  |
| antennaPortConfig | Not present | Same as for the reference cell |  |
| slotNumberOffset | Not present | Slot timing is the same as for reference cell |  |
| prs-SubframeOffset | Not present |  |  |
| expectedRSTD | 8192 | Value 0 |  |
| expectedRSTD-Uncertainty | 10 | About 1 s |  |
| earfcn-v9a0 | Not present | Same as for the reference cell |  |
| tpId-r14 | Not present | Transmission Points not used | Rel-14 onwards |
| prs-only-tp-r14 | Not present | Not required | Rel-14 onwards |
| cpLengthCRS-r14 | Not present | Not required | Rel-14 onwards |
| sameMBSFNconfigNeighbour-r14 | TRUE | Same as for the reference cell | Rel-14 onwards |
| dlBandwidth-r14 | Not present | Same as for the reference cell and PRS frequency hopping not used | Rel-14 onwards |
| addPRSconfigNeighbour-r14 | Not present | No additional PRS configuration(s) | Rel-14 onwards |
| } |  |  |  |
| } |  |  |  |

#### 5.4.1.3	MBS Assistance Data Elements

Tables 5.4.1.3-1 and 5.4.1.3-2 define the MBS assistance data elements which shall be provided to the UE in sub-test 16 via LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message.

Table 5.4.1.3-1 defines the MBS assistance data IEs that conditionally depend on the mode being used in the test case and on the assistance data supported by the UE.

Table 5.4.1.3-2 defines the content of the TBS-AssistantDataList.

Table 5.4.1.3-1: MBS assistance data IEs to be conditionally provided to the UE

| MBS Assistance Data IE supported by UE | Mode used in test case |  |
| --- | --- | --- |
|  | UE-based, MBS (Release 14 onwards) | UE-assisted, MBS (Release 14 onwards) |
| mbs-AlmanacAssistance-r14 | Yes | No |
| mbs-AcquisitionAssistance-r14 | Yes | Yes |

Table 5.4.1.3-2: Content of TBS-AssistanceDataList

| Derivation Path: TS 36.355 [4] clause 6.5.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| tbs-AssistanceDataList-r14 SEQUENCE { |  |  |  |  |
| mbs-AssistanceDataList-r14 SEQUENCE { |  |  |  |  |
| mbs-AssistanceDataElement-r14 SEQUENCE { |  |  | Beacon 1 tb1 |  |
| mbs-AlmanacAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| mbs-AcquisitionAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| } |  |  |  |  |
| mbs-AssistanceDataElement-r14 SEQUENCE { |  |  | Beacon 2 tb1 |  |
| mbs-AlmanacAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| mbs-AcquisitionAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| } |  |  |  |  |
| mbs-AssistanceDataElement-r14 SEQUENCE { |  |  | Beacon 3 tb1 |  |
| mbs-AlmanacAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| mbs-AcquisitionAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| } |  |  |  |  |
| mbs-AssistanceDataElement-r14 SEQUENCE { |  |  | Beacon 4 tb1 |  |
| mbs-AlmanacAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| mbs-AcquisitionAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| } |  |  |  |  |
| mbs-AssistanceDataElement-r14 SEQUENCE { |  |  | Beacon 1 tb2 |  |
| mbs-AlmanacAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| mbs-AcquisitionAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| } |  |  |  |  |
| mbs-AssistanceDataElement-r14 SEQUENCE { |  |  | Beacon 2 tb2 |  |
| mbs-AlmanacAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| mbs-AcquisitionAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| } |  |  |  |  |
| mbs-AssistanceDataElement-r14 SEQUENCE { |  |  | Beacon 3 tb2 |  |
| mbs-AlmanacAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| mbs-AcquisitionAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| } |  |  |  |  |
| mbs-AssistanceDataElement-r14 SEQUENCE { |  |  | Beacon 4 tb2 |  |
| mbs-AlmanacAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| mbs-AcquisitionAssistance-r14 |  | According to Table 5.4.1.3-1 and as defined in TS 37.571-5 [12], clause 8 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Next Modified Section >

#### 5.4.1.4	WLAN Assistance Data Elements

Tables 5.4.1.4-1 and 5.4.1.4-2 define the WLAN assistance data elements which shall be provided to the UE in sub-test 17 via LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message.

Table 5.4.1.4-1 defines the WLAN assistance data IE that conditionally depends on the mode being used in the test case and on the assistance data supported by the UE.

Table 5.4.1.4-2 defines the content of the WLAN-DataSet.

Table 5.4.1.4-1: WLAN assistance data IE to be conditionally provided to the UE

| WLAN assistance data IE supported by UE | Mode used in test case |  |
| --- | --- | --- |
|  | UE-based, WLAN (Release 14 onwards) | UE-assisted, WLAN (Release 14 onwards) |
| wlan-AP-Location-r14 (WLAN AP location information) | Yes | No |

Table 5.4.1.4-2: Content of WLAN-DataSet

| Derivation Path: TS 36.355 [4] clause 6.5.6.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| wlan-DataSet-r14::= SEQUENCE (SIZE (1)) OF SEQUENCE { |  |  |  | Rel-14 onwards |
| SEQUENCE (SIZE (4)) OF SEQUENCE.{ |  |  |  |  |
| WLAN-AP-Data-r14 SEQUENCE { |  |  | WLAN AP 1 |  |
| wlan-AP-Identifier-r14 |  | As defined in TS 37.571-5 [12], clause 9 |  |  |
| wlan-AP-Location-r14 |  | According to Table 5.4.1.4-1 and as defined in TS 37.571-5 [12], clause 9 |  |  |
| } |  |  |  |  |
| WLAN-AP-Data-r14 SEQUENCE { |  |  | WLAN AP 2 |  |
| wlan-AP-Identifier-r14 |  | As defined in TS 37.571-5 [12], clause 9 |  |  |
| wlan-AP-Location-r14 |  | According to Table 5.4.1.4-1 and as defined in TS 37.571-5 [12], clause 9 |  |  |
| } |  |  |  |  |
| WLAN-AP-Data-r14 SEQUENCE { |  |  | WLAN AP 3 |  |
| wlan-AP-Identifier-r14 |  | As defined in TS 37.571-5 [12], clause 9 |  |  |
| wlan-AP-Location-r14 |  | According to Table 5.4.1.4-1 and as defined in TS 37.571-5 [12], clause 9 |  |  |
| } |  |  |  |  |
| WLAN-AP-Data-r14 SEQUENCE { |  |  | WLAN AP 4 |  |
| wlan-AP-Identifier-r14 |  | As defined in TS 37.571-5 [12], clause 9 |  |  |
| wlan-AP-Location-r14 |  | According to Table 5.4.1.4-1 and as defined in TS 37.571-5 [12], clause 9 |  |  |
| } |  |  |  |  |
| supportedChannels-11a-r14 |  | As defined in TS 37.571-5 [12], clause 9 |  |  |
| supportedChannels-11bg-r14 |  | As defined in TS 37.571-5 [12], clause 9 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Next Modified Section >

#### 5.4.1.5	Sensor Assistance Data Elements

Tables 5.4.1.5-1 and 5.4.1.5-2 define the Sensor assistance data elements which shall be provided to the UE in sub-test 18 via LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message.

Table 5.4.1.5-1 defines the Sensor assistance data IE that conditionally depends on the mode being used in the test case.

Table 5.4.1.5-2 defines the content of the Sensor-AssistanceDataList.

Table 5.4.1.5-1: Sensor assistance data IE to be conditionally provided to the UE

| Sensor assistance data to be provided to the UE | Mode used in test case |  |
| --- | --- | --- |
|  | UE-based, Sensor (Release 14 onwards) | UE-assisted, Sensor (Release 14 onwards) |
| sensor-AssistanceDataList-r14 | Yes | No |

Table 5.4.1.5-2: Content of Sensor-AssistanceDataList

| Derivation Path: TS 36.355 [4] clause 6.5.5.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Sensor-AssistanceDataList-r14::= SEQUENCE { |  | According to Table 5.4.1.5-1 |  | Rel-14 onwards |
| refPressure-r14 |  | 0 | 101325 Pa |  |
| refPosition-r14 |  | As defined in TS 37.571-5 [12], clause 6.1.3.4, GNSS-ReferenceLocation | As used in GNSS sub-tests |  |
| refTemperature-r14 |  | 20 | 293K |  |
| period-v1520 |  | Not present |  |  |
| area-v1520 |  | Not present |  |  |
| } |  |  |  |  |

# < Next Modified Section >

## 8.4	Default LPP message and information elements contents

The default values of LPP messages and information elements used, unless indicated otherwise in specific clauses of this specification are as defined in clause 5.4 with the following exceptions:

#### -	LPP PROVIDE ASSISTANCE DATA

Table 8.4-1: ProvideAssistanceData

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { | Dependent on test case. |  |  |
| initiator |  |  |  |
| transactionNumber |  |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 7 and 15 only; and as defined in Table 5.4.1.1-1. |
| gnss-CommonAssistData SEQUENCE { |  |  |  |
| gnss-ReferenceTime | As defined in 37.571-5 [12] |  |  |
| gnss-ReferenceLocation | As defined in 37.571-5 [12] |  |  |
| gnss-IonosphericModel | As defined in 37.571-5 [12] |  |  |
| gnss-EarthOrientationParameters | Not present |  |  |
| } |  |  |  |
| gnss-GenericAssistData(SIZE(1..4))OF{ | SIZE is dependent on the number of GNSSs supported by the UE. If one GNSS supported by the UE, SIZE = 1 If two GNSSs supported by the UE, SIZE = 2 If three GNSSs supported by the UE, SIZE = 3 If four GNSSs supported by the UE, SIZE = 4 |  |  |
| gnss-ID | For each GNSS supported by the UE. |  |  |
| sbas-ID | Not present |  |  |
| gnss-TimeModels | As defined in 37.571-5 [12] |  |  |
| gnss-DifferentialCorrections | Not present |  |  |
| gnss-NavigationModel | As defined in 37.571-5 [12] |  |  |
| gnss-RealTimeIntegrity | Not present |  |  |
| gnss-DataBitAssistance | Not present |  |  |
| gnss-AcquisitionAssistance | As defined in 37.571-5 [12] |  |  |
| gnss-Almanac | As defined in 37.571-5 [12] |  |  |
| gnss-UTC-Model | As defined in 37.571-5 [12] |  |  |
| gnss-AuxiliaryInformation | As defined in 37.571-5 [12] |  |  |
| } |  |  |  |
| gnss-Error | Not present |  |  |
| } |  |  |  |
| otdoa-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 5 and 7 only |
| otdoa-ReferenceCellInfo | As defined in Table 8.4.1.2-1 |  |  |
| otdoa-NeighbourCellInfo | As defined in Table 8.4.1.2-2 |  |  |
| otdoa-Error | Not present |  |  |
| } |  |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 18 only as defined in clause 5.4.1.5 |
| sensor-AssistanceDataList-r14 | As defined in Table 5.4.1.5-2 |  |  |
| sensor-Error-r14 | Not present |  |  |
| } |  |  |  |
| tbs-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 16 only as defined in clause 5.4.1.3 |
| tbs-AssistanceDataList-r14 SEQUENCE { |  |  |  |
| mbs-AssistanceDataList-r14 SEQUENCE (SIZE(1..n)) OF SEQUENCE{ |  |  |  |
| mbs-AlmanacAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| mbs-AcquisitonAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| tbs-Error-r14 | Not present |  |  |
| } |  |  |  |
| wlan-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 17 only as defined in clause 5.4.1.4 |
| wlan-DataSet-r14 | As defined in Table 5.4.1.4-2 |  |  |
| wlan-Error-r14 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# < End of Modified Section >

# < END OF CHANGE >