3GPP TSG-RAN5 Meeting #90-e	R5-210296

# Electronic Meeting, 22nd Feb – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0455 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MU and TT definition for REFSENS FR2 CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MU and TT definition for REFSENS CA test cases. For REFSENS single carrier test case MU and TT are defined for PC3 and BW <= 400MHz. Therefore, same values must be valid for REFSENS CA test cases for PC3 and intra-band contiguous case when aggregated BW <= 400MHz, what set a limitation up to 4DL CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined MU and TT for REFSENS CA test cases (up to 4DL CA) same as REFSENS single carrier test case 7.3.2 for PC3 and aggregated BW <= 400MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3. F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 ±5.19 dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b) | MTSU = 1.00 x MU (from Table B.19-2-2 in TR 38.903) |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.2 Reference sensitivity power level for CA (4DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3.4 EIS spherical coverage | [±4.90] dB (Quiet Zone size ≤ 30 cm, FR2a, FR2b) | MTSU = 1.00 x MU (from Table B.19-2-2 in TR 38.903) |
| 7.4 Maximum input level | TBD |  |
| 7.5 Adjacent channel selectivity | TBD |  |
| 7.6.2 In-band blocking | TBD |  |
| 7.9 Spurious emissions | Quiet Zone size ≤ 30 cm Maximum in-band BW ≤ 400MHz For Band n257: ±5.50dB (6GHz ≤ f ≤ 12.75GHz) ±5.46dB (12.75GHz < f ≤ 23.45GHz) ±6.11dB (23.45GHz < f < 40.8GHz) ±7.65dB (40.8GHz ≤ f ≤ 66GHz) ±[7.95] dB (66GHz ≤ f ≤ 80GHz) | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.1.2-1. |  |  |

## <<< Skip unchanged sections >>>

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | PC3 IFF (Quiet Zone size ≤ 30 cm, FR2a, FR2b) 2.34 dB | TT = 0.45 x MTSUIFF |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.2 Reference sensitivity power level for CA (4DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 7.3.4 EIS spherical coverage | IFF (Quiet Zone size ≤ 30 cm, FR2a, FR2b) 2.21 dB | PC3 TT = 0.45 x MTSUIFF |
| 7.4 Maximum input level | TBD |  |
| 7.5 Adjacent channel selectivity | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.6.2 In-band blocking | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.9 Spurious emissions | 0 dB | Minimum requirement + TT T-put limit unchanged |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.3.2-1. |  |  |

## <<< END OF CHANGES >>>