3GPP TSG-RAN5 Meeting #90-e	R5-210316

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| draft CR |  |  |  |  |  |  |  |  |
|  | 38.509 | CR |  | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updae UE Positioning test mode procedures and UE Positioning messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clauses 5.6.2 and 6.6.2 in 38.509 is FFS. And Logged MDT test cases will use test mode procedures and messages defined in the clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added contents in clause 5.6.2 and 6.6.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures and messages for Update UE Location Information will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2, 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.6	UE Positioning test mode procedures

5.6.1	Reset UE Positioning Stored Information

5.6.1.1	General

Same as TS 36.509 [6], subclause 5.5.1.1.

5.6.1.2	Initiation

Same as TS 36.509 [6], subclause 5.5.1.2.

5.6.1.3	Reception of RESET UE POSITIONING STORED INFORMATION message by UE

Same as TS 36.509 [6], subclause 5.5.1.3 with the following exception:

-	where OTDOA is mentioned this applies to OTDOA using LTE cells.

5.6.2	Update UE Location Information

5.6.2.1	General

Same as TS 36.509 [6], subclause 5.5.2.1.

5.6.2.2	Initiation

Same as TS 36.509 [6], subclause 5.5.2.2.

5.6.2.3	Reception of UPDATE UE LOCATION INFORMATION message by UE

Same as TS 36.509 [6], subclause 5.5.2.3.

< Unchanged sections omitted >

6.6	UE Positioning messages

6.6.1	RESET UE POSITIONING STORED INFORMATION

Same as TS 36.509 [6], subclause 6.9 with the following exception:

-	where OTDOA is mentioned this applies to OTDOA using LTE cells.

6.6.2	UPDATE UE LOCATION INFORMATION

Same as TS 36.509 [6], subclause 6.12