3GPP TSG-RAN5 Meeting #90-e	R5-210358

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0855 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 7.3B.4 for EIS Spherical Coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The latest version of the work plan contains test case 7.3B.4 which provides test coverage of EIS spherical coverage for FR2 NSA UEs. TS 38.521-3 is missing such a test. Test case 6.4B.2.4.4 is not included in the contents section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 has been added. Heading of TC 6.4B.2.4.4 has been formated as h5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EIS spherical coverage will not be tested for NSA UEs. The work plan will not progress. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4B.2.4.4, 7.3B.4 (new), F.1.3, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0059 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.4B.2.4.3.5	Test requirement

Same test requirement as in clause 6.4.2.3.5 in TS 38.521-2 [9] for the NR carrier.

##### 6.4B.2.4.4	EVM Equalizer Flatness for inter-band EN-DC including FR2

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	The referred test case 6.4.2.4 in TS 38.521-2 [9] is incomplete.

-	Measurement Uncertainty and Test Tolerance are FFS.

-	38.101-2 [3] clause 6.3.4.3: Relative power tolerances are in square brackets.

< Unchanged sections omitted >

### 7.3B.3	ΔRIB,c ΔRIBNC for EN-DC

#### 7.3B.3.4	Reference sensitivity for ΔRIB,c Inter-band EN-DC including FR2

The ΔRIB,c for NR FR2 band of inter-band CA defined in tables 5.5B.5.1-1 to 5.5B.5.5-1 is set to zero.

### 7.3B.4	EIS Spherical Coverage for Inter-band EN-DC including FR2

7.3B.4.1	Test purpose

Same test purpose as in clause 7.3.4.1 in TS 38.521-2 [9] for the NR carrier.

7.3B.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 2 DL CCs.

7.3B.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3.4.3 in TS 38.521-2 [9] for the NR carrier.

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.3B.2.4.

7.3B.4.4	Test description

7.3B.4.4.1	Initial conditions

Same test description as in clause 7.3.4.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 7.3.4.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.3.4.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 7.3.4.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.3B.4.5	Test requirement

Same test requirement as in clause 7.3.4.5 in TS 38.521-2 [9] for the NR carrier.

## 7.4	Void

< Unchanged sections omitted >

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) | Same as clause 7.3.2 in TS 38.521-1 [8] |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) | Same as clause 7.3.2 in TS 38.521-1 [8] |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) | Same as clause 7.3.2 in TS 38.521-1 [8] |  |
| 7.3B.2.3_1.1 Reference sensitivity for Inter-band EN-DC within FR1 (3 CCs) | Same as clause 7.3A.1 in TS 38.521-1 [8] |  |
| 7.3B.2.4 Reference sensitivity for Inter-band EN-DC including FR2 | Same as clause 7.3.2 in TS 38.521-2 [9] |  |
| 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 | Same as clause 7.3.4 in TS 38.521-2 [9] |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) | MU for NR CC downlink power same as clause 7.4 in TS 38.521-1 [8]. Uplink power measurement same as 6.2B.1.1. |  |
| < Unchanged rows omitted > |  |  |

< Unchanged sections omitted >

## F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.3B.2.1 Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) | Same as 7.3.2 in TS 38.521-1 [8] |  |
| 7.3B.2.2 Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) | Same as 7.3.2 in TS 38.521-1 [8] |  |
| 7.3B.2.3 Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) | Same as 7.3.2 in TS 38.521-1 [8] |  |
| 7.3B.2.3_1.1 Reference sensitivity for Inter-band EN-DC within FR1 (3 CCs) | Same as 7.3A.1 in TS 38.521-1 [8] |  |
| 7.3B.2.4 Reference sensitivity for Inter-band EN-DC including FR2 | Same as 7.3.2 in TS 38.521-2 [9] |  |
| 7.3B.4 EIS Spherical Coverage for Inter-band EN-DC including FR2 | Same as 7.3.4 in TS 38.521-2 [9] |  |
| 7.4B.1 Maximum Input Level for Intra-Band Contiguous EN-DC (2 CCs) | Same as 7.4 in TS 38.521-1 [8] |  |
| 7.4B.2 Maximum Input Level for Intra-Band Non-Contiguous EN-DC (2 CCs) | Same as 7.4 in TS 38.521-1 [8] |  |
| 7.4B.3 Maximum Input Level for Inter-band EN-DC within FR1 (2 CCs) | Same as 7.4 in TS 38.521-1 [8] |  |
| < Unchanged rows omitted > |  |  |

< End of changes >