3GPP TSG-RAN WG5 Meeting #90-e	R5-210365

# Electronic Meeting, 22nd Feb 2021 – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1970 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR-DC RRC TC 8.2.5.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TP incorrectly mentions that UE reports SCGFailureInformationNR. As per TS 38.331 Cl 5.7.3.2 for non (NG)EN-DC cases UE initiates SCGFailureInformation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected TP to say that UE initiates SCGFailureInformation. - Updated Step 4with the correct UE message. - Updated the conformance requirements and Specific message contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.2.5.1.2	Radio link failure / Random access problem / NR-DC

8.2.5.1.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state with NR-DC }

ensure that {

when { UE receives an IP Packet to loopback on SCG DRB and the SS stops transmitting Timing Advance Command and does not respond to the Scheduling Requests from UE }

then { UE encounters random access problem and initiates the SCG failure information procedure to report SCGFailureInformation with failure type randomAccessProblem }

}

8.2.5.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.7.3.2, 5.3.10.3,  5.7.3.3 and 5.7.3.5. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clauses 5.7.3.2]

A UE initiates the procedure to report SCG failures when neither MCG nor SCG transmission is suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with subclause 5.3.10.3;

...

Upon initiating the procedure, the UE shall:

1>	suspend SCG transmission for all SRBs and DRBs;

1>	reset SCG MAC;

1>	stop T304 for the SCG, if running;

1>	stop conditional reconfiguration evaluation for CPC, if configured;

1>	if the UE is in (NG)EN-DC:

2>	initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10], clause 5.6.13a.

1>	else:

2>	initiate transmission of the SCGFailureInformation message in accordance with 5.7.3.5.

[TS 38.331, clause 5.3.10.3]

The UE shall:

...

1>	upon consistent uplink LBT failure indication from SCG MAC:

...

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

[TS 38.331, clause 5.7.3.3]

The UE shall set the SCG failure type as follows:

1…

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide random access problem indication from SCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the failureType as other and set the failureType-v1610 as beamFailureRecoveryFailure;

2>	else:

3>	set the failureType as randomAccessProblem;

[TS 38.331, clause 5.7.3.3]

The UE shall set the contents of the SCGFailureInformation message as follows:

…

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide random access problem indication from SCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the failureType as other and set the failureType-v1610 as beamFailureRecoveryFailure;

2>	else:

3>	set the failureType as randomAccessProblem;

8.2.5.1.2.3	Test description

8.2.5.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 10 is the PSCell

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state NR RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC) according to TS 38.508-1 [4], Table 4.5.1-1, and with UE test loop mode B prepared, as defined in TS 38.508-1 [4], subclause 4.4A.

8.2.5.1.2.3.2	Test procedure sequence

Table 8.2.5.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS shall not respond to the Scheduling Requests for the IP Packet transmitted at next step (Note) | - | - | - | - |
| 2 | The SS transmits one IP Packet on SCG DRB | - | - | - | - |
| 3 | The SS shall not respond to the PRACH Preambles thereby simulating a random access problem. | - | - | - | - |
| 4 | Check: Does the UE transmit in the next 5 sec (arbitrary value) a SCGFailureInformation message with failureType set to ‘randomAccessProblem’? | --> | SCGFailureInformation | 1 | P |
| Note: The UE initiates random access procedure once SR number of attempts reach sr-transMax. |  |  |  |  |  |

8.2.5.1.2.3.3	Specific message contents

Table 8.2.5.1.2.3.3-1: SCGFailureInformation (step 4, Table 8.2.5.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-24A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SCGFailureInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| scgFailureInformation SEQUENCE { |  |  |  |  |
| failureReportSCG |  | FailureReportSCG | Table 8.2.5.1.2.3.3-2 |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.2.5.1.2.3.3-2: FailureReportSCG (Table 8.2.5.1.2.3.3-1)

| Derivation Path: TS 38.331[4], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| FailureReportSCG ::= SEQUENCE { |  |  |  |  |
| failureType |  | randomAccessProblem |  |  |
| measResultFreqList |  | Not checked |  |  |
| measResultSCG-Failure |  | Not checked |  |  |
| } |  |  |  |  |