3GPP TSG-RAN5 Meeting #90-e 	R5-210372

# Electronic Meeting, 22nd Feb 2021 – 8th Mar 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.509 | CR | 0032 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to set UAI function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update of Set UAI Test Function |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Set UAI Test Mode description and message names "SET UAI REQUEST" and SET UAI RESPONSE" In Rel-16 UE Power Saving WI, the UAI can be used to transition UE out of RRC_CONNECTED state when the UE does not expect to send or receive data in the near future. In order to test this part of conformance requirement (TS38.331, cl 5.7.4), there is a need to set and trigger the UAI message securely. A test command can set the UE's UAI message, specifically below IE ReleasePreference-r16 ::= SEQUENCE { preferredRRC-State-r16 ENUMERATED {idle, inactive, connected, outOfConnected} Based the on IE set by this test message, UE will then trigger UAI RRC message in UL, thus providing test coverage for this feature. The rest of the IE's in UAI will not be part of this test message at this time as they are not all testable. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage wont be complete for the WP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 (new), 6.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Spec will have to move to Rel-16 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

## 5.8	Set UAI test function

### 5.8.1	General

The SS use the Set UAI test procedure to set contents of UAI (UE Assistance Information) RRC message and trigger the RRC message by the UE, see figure 5.8.1-1. The different tpe of information that can be sent in a SET UAI REQUEST is preferred RRC state, see clause 6.8.

Figure 5.8.1-1: Set UAI test procedure

### 5.8.2	Initiation

The SS requests the UE to set the preferred RRC state of  UE Assistance Information by transmitting a SET UAI REQUEST message and the UE confirms by responding with a SET UAI RESPONSE message.

### 5.8.3	Reception of SET UAI REQUEST message by UE

When UE receives SET UAI REQUEST message then the UE shall:

- if the UE is operating in RRC_CONNECTED state:

2>	set the preferredRRC-State-r16 within the ReleasePreference-r16 IE in the UE Assistance Information message, to the equivalent value as received in the Preferred RRC State of the SET UAI REQUEST message.

2>	Transmit SET UAI RESPONSE message.

2> Submit the UE Assistance Information message to lower layers for transmission upon which the procedure ends.

1>	else:

2>	the UE behaviour is unspecified.

<Unchanged Sections Omitted>

## 6.8	SET UAI REQUEST

### 6.8.1	SET UAI REQUEST

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| Preferred RRC State |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | octet 1 |

where Preferred RRC State is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  | E1 | E0 | octet 2 |

E1=0 and E0=0; Idle.

E1=0 and E0=1; inactive.

E1=1 and E0=0: connected.

E1=1 and E0=1: outOfConnected.

### 6.8.2	SET UAI RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | octet 1 |

<End of Changes>