3GPP TSG-RAN5 Meeting #90-e	R5-210377

# Electronic Meeting, 22nd Feb - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.544 | CR | 0030 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TRS numeric symbol |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intertek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Postive TRS numeric symbol is incorrect in Table 7.3.1.2-2, Table 7.3.1.5-2 and Table K.3.1-2 Heading styles of section K.2.1 and K.3.1 are incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the TRS number to negtive one in Table 7.3.1.2-2, Table 7.3.1.5-2 and Table K.3.1-2 Correct the Heading styles of K.2.1 and K.3.1 from “heading 2” to “heading 3” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The table information and some styles will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1, Annex K |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of the change >>

### 7.3.1	Total Radiated Sensitivity (TRS) for UTRA FDD LEE

Editor’s notes: This subclause is incomplete. The following items are missing or incomplete:

- Table 7.3.1.2-1 and 7.3.1.5-1 as they are not specified in TS 37.144 yet

- Applicability

#### 7.3.1.1	Definition and applicability

The Total Radiated Sensitivity is defined as:

Where the effective isotropic sensitivity (EIS) is defined as the power available at the antenna output such as the sensitivity threshold is achieved for each polarization.  is the solid angle describing the direction,  is frequency.  and  are the orthogonal polarizations.

In these formulas  and  are the number of sampling intervals for theta and phi.  and  are the measurement angles. The sampling intervals are discussed further in Subclause 4.4.

The TRS can also be calculated from measurements in a Rayleigh fading 3 dimensional isotropic environment with in average uniform elevation and azimuth distribution. The calculation of the TRS is in this case based on searching for the lowest power received by the UE/MS for a discrete number of field combinations in the chamber that gives a BER that is better than the specified target BER level. By calibrating the average power transfer function, an absolute value of the TRS can be obtained. The following expression can be used to find the TRS.

where  is the reference power transfer function for fixed measurement antenna n,  is the reflection coefficient for fixed measurement antenna n and  is the path loss in the cables connecting the measurement receiver to fixed measurement antenna n. These parameters are calculated from the calibration measurement and are further discussed in Annex B.2.  is calculated by using the following equation:

where  is the m:th value of the transfer function for fixed measurement antenna n, which gives the BER threshold.  is the total number of values of the BER threshold power measured for each fixed measurement antenna.

[Applicability is TBD]

#### 7.3.1.2	Minimum Requirements

The average TRS of low, mid and high channel of the LEE in the data transfers position for 1% BER with 12.2kbps DL reference channel as defined in Annex C.3 of TS25.101 [2] shall be lower than minimum performance requirements for roaming bands shown in Table 7.3.1.2-1 for notebook devices and in Table 7.3.1.2-2 for tablet devices. The averaging shall be done in linear scale for the TRS results.

In addition the maximum TRS of each measured channel in the data transfers position shall be better than minimum performance requirements for roaming bands shown in the columns “Max”.

Table 7.3.1.2-1: TRS minimum requirements for UTRA FDD roaming mode for notebook devices in the data transfers position and the primary mechanical mode

| Operating band | Unit | <REFÎor> |  |
| --- | --- | --- | --- |
|  |  | Average | Max |
| I | dBm/3.84 MHz | -103.5 | -102.0 |
| II | dBm/3.84 MHz | TBD | TBD |
| III | dBm/3.84 MHz | TBD | TBD |
| IV | dBm/3.84 MHz | TBD | TBD |
| V | dBm/3.84 MHz | TBD | TBD |
| VI | dBm/3.84 MHz | -101.5 | -99.5 |
| VII | dBm/3.84 MHz | TBD | TBD |
| VIII | dBm/3.84 MHz | -101.5 | -99.5 |
| IX | dBm/3.84 MHz | TBD | TBD |
| XIX | dBm/3.84 MHz | -101.5 | -99.5 |
| NOTE 1: For power class 3, 3bis and 4 this shall be achieved at the maximum output power. NOTE 2: Applicable for dual-mode GSM/UMTS. NOTE 3: Applicable for notebook devices. |  |  |  |

NOTE: TRS minimum performance requirements in Table 7.3.1.2-1 apply to HSPA and LTE UEs supporting only single carrier operation. Their applicability to multi-carrier operation is FFS. This is because it has not been verified whether the UEs measured to derive the requirements supported carrier aggregation or not.

Table 7.3.1.2-2: TRS minimum requirements for UTRA FDD roaming mode for tablet devices in the data transfers position and the primary mechanical mode

| Operating band | Unit | <REFÎor> |  |
| --- | --- | --- | --- |
|  |  | Average | Max |
| I | dBm/3.84 MHz | -105.75 | -104.25 |
| II | dBm/3.84 MHz | TBD | TBD |
| III | dBm/3.84 MHz | TBD | TBD |
| IV | dBm/3.84 MHz | TBD | TBD |
| V | dBm/3.84 MHz | -103.0 | -101.0 |
| VI | dBm/3.84 MHz | TBD | TBD |
| VII | dBm/3.84 MHz | TBD | TBD |
| VIII | dBm/3.84 MHz | TBD | TBD |
| IX | dBm/3.84 MHz | TBD | TBD |
| XIX | dBm/3.84 MHz | -103.0 | -101.0 |
| NOTE 1: For power class 3, 3bis and 4 this shall be achieved at the maximum output power. NOTE 2: Applicable for dual-mode GSM/UMTS. NOTE 3: Applicable for tablet devices with two antennas. |  |  |  |

The normative reference for this clause is TS 37.144 [12] Subclause 7.3.2.

#### 7.3.1.3	Test purpose

The purpose of this test is to ensure that  and  of the UE is above specified limit. The lack of the reception sensitivity decreases the coverage area at the far side from Node B.

#### 7.3.1.4	Method of test

#### 7.3.1.4.1	Initial conditions

A radio communications tester or a corresponding device is used as a NB simulator to setup calls to the DUT. The NB simulator is also used to send test signals to the UE and measure the BER levels of the radio link and the information on the dedicated channel needed to extract the DUT receiver performances.

Chamber environment constraints and coordinate system shall be the same as described in Subclause A.3.

Phantom Specifications shall be the same as described in Subclause A.2.4. Parameters required for Notebooks are specified in Annex A.2.4.2 and those for Tablets are specified in Annex A.2.4.3.

Test environment: normal; see TS36.508 [10] Subclause 4.1.1.

Frequencies to be tested: low range, mid range, high range; see Table 5.1-2.

Positioning Requirements shall be the same as described in Subclause 4.3.

1)	Position the UE according to the DUT positioning for devices with embedded modules specified in Subclause 4.3.5.

2)	Set the SS downlink physical channels according to settings in Table 7.3.1.4.1-1.

3)	Power on the UE.

4)	A call is set up according to the Generic call setup procedure. The power control algorithm shall be set to Power Control Algorithm 2. Compressed mode shall be set to OFF.

5)	Enter the UE into loopback test mode 2 and start the loopback test.

See TS 34.108 [4] and TS 34.109 [5] for details regarding generic call setup procedure and loopback test.

Table 7.3.1.4.1-1: Downlink Physical Channels transmitted during a connection

| Physical Channel | Power |
| --- | --- |
| CPICH | CPICH_Ec / DPCH_Ec = 7 dB |
| P-CCPCH | P-CCPCH_Ec/ DPCH_Ec = 5 dB |
| SCH | SCH_Ec / DPCH_Ec = 5 dB |
| PICH | PICH_Ec / DPCH_Ec = 2 dB |
| DPCH | Test dependent power |

#### 7.3.1.4.2	Procedure

1)	Send continuously Up power control commands to the UE.

2)	As the UE reaches maximum power, start sending PN15 data pattern.

3)	Measure EIS from one measurement point. EIS is the power transmitted from one specific direction to the UE causing BER value of 1%  0.2% using 20000 or more bits; see Annex E.20.

NOTE: To meet BER value target DL power level can be changed using user’s freely selectable algorithm.

4)	Measure the EIS for every direction of selected sampling gird using two orthogonal polarizations to obtain TRS.

5)	Calculate TRS using equations from Subclause 7.3.1.1.

NOTE: To speed up sensitivity measurements non standard setting (i.e. data speed, PCL, BER target) can be used in the measurements. However to obtain TRS result the measured EIS figures shall be normalized by

Where  is sensitivity measurement done with standard setting,  is sensitivity measurement done with non standard settings.  is amount of reference measurement points.

To ensure accuracy of TRS, the amount of reference points measured should be at least 4. It is recommended to spread the reference measurements equally during the measurement time.

The measurement procedure is based on the measurement of the spherical sensitivity pattern of the DUT. The sensitivity values of the DUT at a predefined BER level are sampled in far field in a group of points located on a spherical surface enclosing the DUT. The EIS samples are taken using a constant sample step of 30° both in theta () and phi () directions. All the EIS samples are taken with two orthogonal polarizations, - and -polarisations (See Subclause 4.4).

The Total Radiated Sensitivity is calculated from the measured data by equation in Subclause 7.3.1.2.

#### 7.3.1.4.3	Procedure, reverberation chamber method

1)	Send continuously Up power control commands to the UE.

2)	As the UE reaches maximum power, start sending PN15 data pattern.

3)	Set the base station simulator to a specific output power and perform a BER measurement.

4)	Increase or decrease the base station output power as needed, and repeat step 4 until the lowest output power is found that gives a BER value of 1%  0.2% using 20000 or more bits, see Annex E.20. This corresponds to .

NOTE: To meet BER value target DL power level can be changed using user’s freely selectable algorithm.

5)	Repeat step 3 and 4 until a sufficient number of independent samples (see section 4.5) of  has been measured.

7)	Calculate TRS using equations from chapter 7.3.1.1.

NOTE 1:	The measurement procedure is based on samples of the received signal power at the UE/MS from a full 3 dimensional isotropic environment with uniform elevation and azimuth field distribution. The power received by the DUT is undergoing Rayleigh fading and is transmitted by the fixed measurement antennas. Moreover, it is important that the samples collected are independent, in order to get sufficient accuracy of the estimated TRS value.

The Total Radiated Sensitivity is calculated from the measured data by equation in Subclause 7.3.1.2.

#### 7.3.1.5	Test requirements

The average TRS of low, mid and high channel in the data transfers position for 1% BER with 12.2kbps DL reference channel as defined in Annex C.3 of TS 25.101 [2] shall be lower than test requirements for roaming bands shown in Table 7.3.1.5-1 for notebook devices and in Table 7.3.1.5-2 for tablet devices. The averaging shall be done in linear scale for the TRS results.

In addition the maximum TRS of each measured channel in the data transfers position shall be better than minimum performance requirements for roaming bands shown in the columns “Max”.

Table 7.3.1.5-1: TRS test requirements for UTRA FDD roaming mode for notebook devices in the data transfers position and the primary mechanical mode

| Operating band | Unit | <REFÎor> |  |
| --- | --- | --- | --- |
|  |  | Average | Max |
| I | dBm/3.84 MHz | -103.5+TT | TBD+TT |
| II | dBm/3.84 MHz | TBD+TT | TBD+TT |
| III | dBm/3.84 MHz | TBD+TT | TBD+TT |
| IV | dBm/3.84 MHz | TBD+TT | TBD+TT |
| V | dBm/3.84 MHz | TBD+TT | TBD+TT |
| VI | dBm/3.84 MHz | -101.5+TT | TBD+TT |
| VII | dBm/3.84 MHz | TBD+TT | TBD+TT |
| VIII | dBm/3.84 MHz | -101.5+TT | TBD+TT |
| IX | dBm/3.84 MHz | TBD+TT | TBD+TT |
| XIX | dBm/3.84 MHz | -101.5+TT | TBD+TT |
| NOTE 1: For power class 3, 3bis and 4 this shall be achieved at the maximum output power. NOTE 2: Applicable for dual-mode GSM/UMTS. NOTE 3: Applicable for notebook devices. |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex D.

Table 7.3.1.5-2: TRS test requirements for UTRA FDD roaming mode for tablet devices in the data transfers position and the primary mechanical mode

| Operating band | Unit | <REFÎor> |  |
| --- | --- | --- | --- |
|  |  | Average | Max |
| I | dBm/3.84 MHz | -105.75+TT | -104.25+TT |
| II | dBm/3.84 MHz | TBD+TT | TBD+TT |
| III | dBm/3.84 MHz | TBD+TT | TBD+TT |
| IV | dBm/3.84 MHz | TBD+TT | TBD+TT |
| V | dBm/3.84 MHz | -103.0+TT | -101.0+TT |
| VI | dBm/3.84 MHz | TBD+TT | TBD+TT |
| VII | dBm/3.84 MHz | TBD+TT | TBD+TT |
| VIII | dBm/3.84 MHz | TBD+TT | TBD+TT |
| IX | dBm/3.84 MHz | TBD+TT | TBD+TT |
| XIX | dBm/3.84 MHz | -103.0+TT | -101.0+TT |
| NOTE 1: For power class 3, 3bis and 4 this shall be achieved at the maximum output power. NOTE 2: Applicable for dual-mode GSM/UMTS. NOTE 3: Applicable for tablet devices with two antennas. |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex D.

<< skip unchanged section >>

Annex K (informative): Recommended performance for LEE

Editor’s notes: This annex is incomplete. The following items are missing or incomplete:

- Table K.2.1-1, K.2.1-2, K.2.2-1, K.2.2-2, K.3.1-1, K.3.1-2, K.3.2-1 and K.3.2-2 as they are not complete in TS 37.144 yet

- Table K.2.3-1, K.2.3-2, K.2.4-1, K.2.4-2, K.3.3-1, K.3.3-2, K.3.4-1 and K.3.4-2 as they are not specified in TS 37.144 yet

## K.1	General

This annex introduces the concept of recommended OTA performance for operating bands for UE’s supporting LEE feature. This requirement is not mandatory but is recommended.

The concept of recommended performance is to ensure that UE OTA performance is maximised in order to improve user experience and network performance. It is recognised that the ability to meet the recommended performance depends on the number of frequency bands supported by the UE.

## K.2	Total Radiated Power

The OTA TRP performance for UTRA and E-UTRA should be greater or equal than the recommended values in this clause.

Test method for devices with embedded modules is defined in subclauses 4.3.5.

## K.2.1	UTRA FDD

Table K.2.1-1: Notebook TRP recommended performance for UTRA FDD in data transfer position

| Operating band | Power class 1 | Power class 2 | Power class 3 | Power class 3bis | Power class 4 |
| --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) |
|  | Average | Average | Average | Average | Average |
| I | - | - | 21.5 | TBD | TBD |
| II | - | - | TBD | TBD | TBD |
| III | - | - | TBD | TBD | TBD |
| IV | - | - | TBD | TBD | TBD |
| V | - | - | TBD | TBD | TBD |
| VI | - | - | 21.0 | TBD | TBD |
| VII | - | - | TBD | TBD | TBD |
| VIII | - | - | 21.0 | TBD | TBD |
| IX | - | - | TBD | TBD | TBD |
| XIX | - | - | 21.0 | TBD | TBD |
| NOTE 1: Applicable for multi-mode GSM/UMTS/LTE. NOTE 2: Applicable for notebook devices. |  |  |  |  |  |

NOTE: TRP minimum performance requirements in Table K.2.1-1apply to HSPA and LTE UEs supporting only single carrier operation. Their applicability to multi-carrier operation is FFS. This is because it has not been verified whether the UEs measured to derive the requirements supported carrier aggregation or not.

Table K.2.1-2: Tablet TRP recommended performance for UTRA FDD in data transfer position

| Operating band | Power class 1 | Power class 2 | Power class 3 | Power class 3bis | Power class 4 |
| --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) |
|  | Average | Average | Average | Average | Average |
| I | - | - | 21.5 | TBD | TBD |
| II | - | - | TBD | TBD | TBD |
| III | - | - | TBD | TBD | TBD |
| IV | - | - | TBD | TBD | TBD |
| V | - | - | 19.5 | TBD | TBD |
| VI | - | - | TBD | TBD | TBD |
| VII | - | - | TBD | TBD | TBD |
| VIII | - | - | TBD | TBD | TBD |
| IX | - | - | TBD | TBD | TBD |
| XIX | - | - | 19.5 | TBD | TBD |
| NOTE 1: Applicable for dual-mode GSM/UMTS. NOTE 2: Applicable for tablet devices with two antennas. |  |  |  |  |  |

### K.2.2	UTRA LCR TDD

Table K.2.2-1: Notebook TRP recommended performance for UTRA LCR TDD in data transfer position

| Operating band | Power class 1 | Power class 2 | Power class 3 | Power class 3bis | Power class 4 |
| --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) |
|  | Average | Average | Average | Average | Average |
| a | - | TBD | - | - | - |
| b | - | TBD | - | - | - |
| c | - | TBD | - | - | - |
| d | - | TBD | - | - | - |
| e | - | TBD | - | - | - |
| f | - | TBD | - | - | - |
| NOTE 1: Applicable for dual-mode GSM/UTRA LCR TDD. NOTE 2: Applicable for notebook devices. |  |  |  |  |  |

Table K.2.2-2: Tablet TRP recommended performance for UTRA LCR TDD in data transfer position

| Operating band | Power class 1 | Power class 2 | Power class 3 | Power class 3bis | Power class 4 |
| --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) | Power (dBm) |
|  | Average | Average | Average | Average | Average |
| a | - | TBD | - | - | - |
| b | - | TBD | - | - | - |
| c | - | TBD | - | - | - |
| d | - | TBD | - | - | - |
| e | - | TBD | - | - | - |
| f | - | TBD | - | - | - |
| NOTE 1: Applicable for dual-mode GSM/UTRA LCR TDD. NOTE 2: Applicable for tablet devices. |  |  |  |  |  |

### K.2.3	E-UTRA FDD

[Table K.2.3-1 and K.2.3-2 TBD]

### K.2.4	E-UTRA TDD

[Table K.2.4-1 and K.2.4-2 TBD]

## K.3	Total Radiated Sensitivity

The OTA TRS performance for UTRA and E-UTRA should be lower or equal than the recommended values shown in this clause.

Test method for devices with embedded modules is defined in subclauses 4.3.5.

## K.3.1	UTRA FDD

Table K.3.1-1: Notebook TRS recommended performance for UTRA FDD in the data transfer position

| Operating band | Unit | <REFÎor> Average |
| --- | --- | --- |
| I | dBm/3.84 MHz | -106.5 |
| II | dBm/3.84 MHz | TBD |
| III | dBm/3.84 MHz | TBD |
| IV | dBm/3.84 MHz | TBD |
| V | dBm/3.84 MHz | TBD |
| VI | dBm/3.84 MHz | -104.5 |
| VII | dBm/3.84 MHz | TBD |
| VIII | dBm/3.84 MHz | -104.5 |
| IX | dBm/3.84 MHz | TBD |
| XIX | dBm/3.84 MHz | -104.5 |
| NOTE 1: Applicable for multi-mode GSM/UMTS/LTE. NOTE 2: Applicable for notebook devices. |  |  |

NOTE: TRS minimum performance requirements in Table K.3.1-1 apply to HSPA and LTE UEs supporting only single carrier operation. Their applicability to multi-carrier operation is FFS. This is because it has not been verified whether the UEs measured to derive the requirements supported carrier aggregation or not.

Table K.3.1-2: Tablet TRS recommended performance for UTRA FDD in the data transfer position

| Operating band | Unit | <REFÎor> Average |
| --- | --- | --- |
| I | dBm/3.84 MHz | -108.75 |
| II | dBm/3.84 MHz | TBD |
| III | dBm/3.84 MHz | TBD |
| IV | dBm/3.84 MHz | TBD |
| V | dBm/3.84 MHz | -106.0 |
| VI | dBm/3.84 MHz | TBD |
| VII | dBm/3.84 MHz | TBD |
| VIII | dBm/3.84 MHz | TBD |
| IX | dBm/3.84 MHz | TBD |
| XIX | dBm/3.84 MHz | -106.0 |
| NOTE: Applicable for tablet devices with two antennas. |  |  |

### K.3.2	UTRA LCR TDD

Table K.3.2-1: Notebook TRS recommended performance for UTRA LCR TDD in the data transfer position

| Operating band | Unit | <REFÎor> Average |
| --- | --- | --- |
| a | dBm/1.28 MHz | TBD |
| b | dBm/1.28 MHz | TBD |
| c | dBm/1.28 MHz | TBD |
| d | dBm/1.28 MHz | TBD |
| e | dBm/1.28 MHz | TBD |
| f | dBm/1.28 MHz | TBD |
| NOTE 1: Applicable for dual-mode GSM/UTRA LCR TDD. NOTE 2: Applicable for notebook devices. |  |  |

Table K.3.2-2: Tablet TRS recommended performance for UTRA LCR TDD in the data transfer position

| Operating band | Unit | <REFÎor> Average |
| --- | --- | --- |
| a | dBm/1.28 MHz | TBD |
| b | dBm/1.28 MHz | TBD |
| c | dBm/1.28 MHz | TBD |
| d | dBm/1.28 MHz | TBD |
| e | dBm/1.28 MHz | TBD |
| f | dBm/1.28 MHz | TBD |
| NOTE 1: Applicable for dual-mode GSM/UTRA LCR TDD. NOTE 2: Applicable for tablet devices. |  |  |

### K.3.3	E-UTRA FDD

[Table K.3.3-1 and K.3.3-2 TBD]

### K.3.4	E-UTRA TDD

[Table K.3.4-1 and K.3.4-2 TBD]

<< end of the change >>