3GPP TSG-RAN5 Meeting #90-e	R5-210471

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1050 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SSB Index to use in the PUxCH-PowerControl for RRM tests with more than one SSB - EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The SSB Index is signaled in PUCCH-PowerControl and PUSCH-PowerControl, which are part of PUxCHConfig, signaled inside the BWP-UplinkDedicated. Here are the definitions of TS 38.331 regarding the two lists where SSB Index is required: pathlossReferenceRSs, pathlossReferenceRSs-v1610 A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUCCH pathloss estimation. Up to maxNrofPUCCH-PathlossReference-RSs may be configured. If the field is not configured, the UE uses the SSB as reference signal (see TS 38.213 [13], clause 7.2). The set includes References Signals indicated in pathlossReferenceRSs (without suffix) and in pathlossReferenceRSs-v1610 pathlossReferenceRSToAddModList, pathlossReferenceRSToAddModList2 A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUSCH path loss estimation. The set consists of Reference Signals configured using pathLossReferenceRSToAddModList and Reference Signals configured using pathlossReferenceRSToAddModList2.Up to maxNrofPUSCH-PathlossReferenceRSs may be configured (see TS 38.213 [13], clause 7.1). For test cases with more than one SSB, it should be clarified which one of the SSB Indexes need to be signaled. For PRACH test cases: Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB, Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB. Therefore, it seems reasonable that SSB with index 0 is the one to be signaled |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a reference in the message content exceptions indicating which SSB Index to use in the corresponding PUxCH-PowerControl tables |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no clarity on which SSB Index should be signaled in PUxCH-PowerControl messages |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.2.1.4.3, 4.3.2.2.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

4.3.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 with the following exceptions:

Table 4.3.2.2.1.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

Table 4.3.2.2.1.4.3-1: RACH-ConfigCommon for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n48 |  | FR1 |
| } |  |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| rsrp-ThresholdSSB | RSRP_51 |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | 0 |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 15 |  | 15kHz |
|  | kHz 30 |  | 30kHz |
| } |  |  |  |

Table 4.3.2.2.1.4.3-2: RACH-ConfigGeneric for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  | FR1 |
| msg1-FDM | one |  | FR1 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 4.3.2.2.1.4.3-3: ServingCellConfigCommon for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 7.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | '1100 0000'B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | -5 |  |  |
| } |  |  |  |

# < Unchanged sections omitted >

4.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 with the following exceptions:

Table 4.3.2.2.2.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

Table 4.3.2.2.2.4.3-1: RACH-ConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | Not present |  |  |
| rsrp-ThresholdSSB | RSRP_51 |  | Subtest 1 |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | 0 |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 15 |  | 15kHz |
|  | kHz 30 |  | 30kHz |
| } |  |  |  |

Table 4.3.2.2.2.4.3-2: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 2 entries |  |  |
| ssb[1] | 0 |  |  |
| ssb[2] | 1 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_51 |  | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.3.2.2.2.4.3-3: RACH-ConfigGeneric for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 87 |  | FR1 |
| msg1-FDM | one |  | FR1 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 4.3.2.2.2.4.3-4: ServingCellConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | -5 |  |  |
| } |  |  |  |

Table 4.3.2.2.2.4.3-5: CellGroupConfig for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| newUE-Identity | 1 |  |  |
| t304 | ms1000 |  |  |
| rach-ConfigDedicted |  |  |  |
| uplink | RACH-ConfigDedicated |  |  |
| } |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| srs-Config | SRS-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

< End of changes >