Title:		Discussion on Test Point Analysis for NR FR2 Maximum input level for CA test cases

Source:			CAICT

Introduction

The purpose of this paper is to provide test point selection proposal for NR FR2 Maximum input level for CA test cases.

Discussion

As a reference, the test configuration for FR2 Maximum input level single carrier is shown below:

Table 7.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Downlink Configuration |  | Uplink Configuration |  |
| Modulation | RB allocation | Modulation | RB allocation |
| CP-OFDM QPSK | NOTE1 | DFT-s-OFDM QPSK | NOTE2 |
| NOTE 1: The specific configuration of downlink RB allocation is defined in Table 7.3.2.4.1-2. NOTE 2: The specific configuration of uplink RB allocation is defined in Table 7.3.2.4.1-3. |  |  |  |

  - Test Environment

Test only at Normal test environment for FR2 Maximum input level for CA, which is same as that for single carrier.

Proposal 1: Propose Test Environment at Normal condition only for FR2 Maximum input level for CA.

  - Test Frequencies

The test frequencies for FR2 Maximum input level for CA shall deviate from that for single carrier.

Proposal 2: Mid frequency range shall be tested for FR2 Maximum input level for CA.

Test Channel Bandwidth

The channel bandwidth for FR2 Maximum input level for CA shall test Maximum aggregated BW (contiguous CA) or Maximum cumulative aggregated BW (non-contiguous CA). The low and Mid BW are considered covered by single carrier hence there is no need to test for CA.

Proposal 3: The Highest supported aggregated/cumulative BW shall be tested for FR2 Maximum input level for CA.

Test Subcarrier Spacing (SCS)

The SCS value of 120kHz will be tested for FR2 Maximum input level for CA, which is same as that for single carrier.

Proposal 4: The 120kHz SCS shall be tested for FR2 Maximum input level for CA.

Modulations

The uplink and downlink channel modulation and RB configuration choice to be used for FR2 Maximum input level for CA tests shall follow same as that for single carrier.

Proposal 5: QPSK DFT-s-OFDM modulation with full RB allocation shall be used for uplink channel configuration for FR2 Maximum input level for CA.

Proposal 6: QPSK CP-OFDM modulation with full RB allocation shall be used for downlink channel configuration for FR2 Maximum input level for CA.

Number of Test Points

The total test points for Maximum input level for CA is summarized in Table below.

Maximum Number of Test Steps for NR FR2 Maximum input level for CA

| Environment conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

Conclusion

In conclusion, the following selection of test points are proposed for NR FR2 Maximum input level for CA measurement

Proposal 1: Propose Test Environment at Normal condition only for FR2 Maximum input level for CA.

Proposal 2: Mid frequency range shall be tested for FR2 Maximum input level for CA.

Proposal 3: The Highest supported aggregated/cumulative BW shall be tested for FR2 Maximum input level for CA.

Proposal 4: The 120kHz SCS shall be tested for FR2 Maximum input level for CA.

Proposal 5: QPSK DFT-s-OFDM modulation with full RB allocation shall be used for uplink channel configuration for FR2 Maximum input level for CA.

Proposal 6: QPSK CP-OFDM modulation with full RB allocation shall be used for downlink channel configuration for FR2 Maximum input level for CA.