3GPP TSG-RAN5 Meeting #90-e	R5-210548

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0872 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Default message exceptions for LTE carriers in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When E-UTRA anchor-agnostic approach is applied, several messages exceptions in section 4.6 and 4.7 disable CQI reporting, SRS transmission and Time Aligment Timer in E-UTRA. For E-UTRA RF SA testing defined in 36.521-1, similar message exceptions are defined for those test cases in 36.508, section 5. However, when E-UTRA anchor-agnostic approach is not applied for EN-DC FR1 tests, no such exceptions are defined, what seems inconsistent with configuration for the rest of RF testing involving E-UTRA as explained above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new section 4.8 E-UTRA configuration for EN-DC FR1 tests not applying the E-UTRA anchor-agnostic approach. Defined in that new section the same message exceptions defined in sections 4.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent among RF test cases involving E-UTRA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## 4.6	E-UTRA configuration for EN-DC FR1 tests applying the E-UTRA anchor-agnostic approach

This clause applies to EN-DC test cases where E-UTRA anchor needs to be configured as per the anchor-agnostic approach outlined in clauses 6.1 and 7.1 of TS 38.101-3 [4].The LTE anchor-agnostic approach is defined as measurements on the NR carrier under conditions where the LTE anchor resources do not interfere with NR operation. The configuration defined in this clause ensures establishment of such conditions.

For baseline configuration, the E-UTRA carrier will be configured for each test case in clauses 6 and 7 as defined in the equivalent standalone E-UTRA test in TS 36.521-1 [10]. However, the below exceptions defined in Table 4.6-1, 4.6-2, 4.6-3, 4.6-4 and 4.6-5 are applied to ensure that the E-UTRA anchor resources do not interfere with NR operation.

For EN-DC within FR1 band combinations with multiple E-UTRA component carriers, it is sufficient to configure any one E-UTRA carrier from the carrier group whenever it is determined that anchor agnostic approach can be applied. Unless otherwise stated, the number of component carriers (CCs) listed in the test case titles of Clause 6 and clause 7 shall refer to the number of component carriers configured within the test case.

Table 4.6-1: E-UTRA configuration for EN-DC FR1 tests applying anchor agnostic approach

| Parameter | Value | Comments |
| --- | --- | --- |
| Test Frequency during and after connection setup | Mid (See Table 4.6-2) | As defined in TS 36.508 for the LTE band under test |
| Bandwidth during and after connection setup | 5 MHz (See Table 4.6-2) | Supported by all LTE bands. |
| DL signal levels during connection setup | RS EPRE -85.0 dBm/15kHz | DL physical channels as defined in Annex C0, C.1, C.2 and Annex C.3 of TS 36.521-1 [10]. TS 36.521-1 [10] annex C.0 defines the default DL power level of RS EPRE to be -85dBm/15kHz. |
| UL Signal levels during connection setup | PUSCH Power | Attained by enabling open loop power control and setting up UL signal levels according to Annex H.0, H.2 and H.3 of TS 36.521-1 [10]. |
| DL/UL RMC after connection setup | 0 RB allocation on both DL and UL (see Table 4.6-2) | Once the LTE link is established, then LTE Tx can be restricted by configuring 0 RB allocation on DL and UL. TimeAlignmentTimerDedicated IE to be set to infinity to ensure UE doesn't look for TA adjustments (See Table 4.6-5) |
| CQI Reports and SRS after connection setup | Disabled (See Table 4.6-3 and 4.6-4) | Disable periodic and aperiodic CQI reports to ensure none of these transmissions occur on the LTE uplink. Since LTE transmissions could easily exceed spurious emissions limits, tests that are intended to measure RF parameters on the NR should simply avoid LTE transmit altogether. |

Table 4.6-2: E-UTRA Test Configuration Table

| E-UTRA Test Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency | Downlink |  | Uplink |  |
|  |  | Modulation | RB allocation | Modulation | RB allocation |
| 5 MHz2 | MidRange1 | N/A | 0 | N/A | 0 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [11] clause 4.3.1. NOTE 2: For EN-DC Intra-band tests that need to apply E-UTRA anchor agnostic approach, refer to and pick applicable E-UTRA channel bandwidth from clause 5.3B.1 and indicate within test case if it is different than 5 MHz. |  |  |  |  |  |

Table 4.6-3: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |  |
| cqi-ReportModeAperiodic |  | NOT PRESENT |  |  |
| cqi-ReportPeriodic |  | NOT PRESENT |  |  |
| } |  |  |  |  |

Table 4.6-4: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| } |  |  |  |  |

Table 4.6-5: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated |  | Infinity |  |  |

## 4.7	E-UTRA configuration for EN-DC FR2 tests applying the E-UTRA anchor-agnostic approach

This clause applies to EN-DC test cases where E-UTRA anchor needs to be configured as per the anchor-agnostic approach outlined in clauses 6.1 and 7.1 of TS 38.101-3 [4].The LTE anchor-agnostic approach is defined as measurements on the NR carrier under conditions where the LTE anchor resources do not interfere with NR operation. The configuration defined in this clause ensures establishment of such conditions.

For baseline configuration, the E-UTRA carrier will be configured for each test case in clauses 6 and 7 as defined in the equivalent standalone E-UTRA test in TS 36.521-1 [10]. However, the below exceptions defined in Table 4.7-1 to 4.7-7 are applied to ensure that the E-UTRA anchor resources do not interfere with NR operation.

Since the E-UTRA link is always a functional link when testing EN-DC including FR2 band combinations, it is sufficient to configure any one E-UTRA carrier from the carrier group, irrespective of the number of E-UTRA carriers in the EN-DC combination under test. Unless otherwise stated, the number of component carriers (CCs) listed in the test case titles of Clause 6 and clause 7 shall refer to the number of component carriers configured within the test case.

Table 4.7-1: E-UTRA configuration for EN-DC FR2 tests applying anchor agnostic approach

| Parameter | Value | Comments |
| --- | --- | --- |
| Test Frequency during and after connection setup | Mid (See Table 4.7-2) | As defined in TS 36.508 for the LTE band under test |
| Bandwidth during and after connection setup | 5 MHz (See Table 4.7-2) | Supported by all LTE bands. |
| DL signal levels | See table 4.7-3 | DL physical channels as defined in Annex C0, C.1, C.2 and Annex C.3 of TS 36.521-1 [10]. |
| UL Signal levels for connection setup and UBF transmission | PUSCH Power | Attained by enabling open loop power control and setting up UL signal levels according to Annex H.0, H.2 and H.3 of TS 36.521-1 [10] with the exception for power control message exception defined in Table 4.7-5 |
| DL/UL RMC after connection setup except for UBF transmission | 0 RB allocation on both DL and UL (see Table 4.7-2) | Once the LTE link is established, then LTE Tx can be restricted by configuring 0 RB allocation on DL and UL. TimeAlignmentTimerDedicated IE to be set to infinity to ensure UE doesn't look for TA adjustments (See Table 4.7-7) |
| CQI Reports and SRS after connection setup | Disabled (See Table 4.7-4 and 4.7-6) | Disable periodic and aperiodic CQI reports to ensure none of these transmissions occur on the LTE uplink. Since LTE transmissions could easily exceed spurious emissions limits, tests that are intended to measure RF parametrics on the NR should simply avoid LTE transmit altogether. |

Table 4.7-2: E-UTRA Test Configuration Table

| E-UTRA Test Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency | Downlink |  | Uplink |  |
|  |  | Modulation | RB allocation | Modulation | RB allocation |
| 5 MHz2 | MidRange1 | N/A | 0 | N/A | 0 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [11] clause 4.3.1 NOTE 2: For EN-DC Intra-band tests that need to apply E-UTRA anchor agnostic approach, refer to and pick applicable E-UTRA channel bandwidth from clause 5.3B.1 and indicate within test case if it is different than 5 MHz. |  |  |  |  |  |

Table 4.7-3: Default Downlink power levels for E-UTRA anchor

|  | Unit | Band Group | Channel Bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5MHz | 10MHz | 15 MHz | 20 MHz |
| RS EPRE | dBm/15kHz | FDD_A, TDD_A | N/A | N/A | ≥ -120.0 | N/A | N/A | N/A |
|  |  | FDD_B1, TDD_B1 | N/A | N/A | ≥ -119.5 | N/A | N/A | N/A |
|  |  | FDD_C, TDD_C | N/A | N/A | ≥ -119.0 | N/A | N/A | N/A |
|  |  | FDD_D, TDD_D | N/A | N/A | ≥ -118.5 | N/A | N/A | N/A |
|  |  | FDD_E, TDD_E | N/A | N/A | ≥ -118.0 | N/A | N/A | N/A |
|  |  | FDD_G, TDD_G | N/A | N/A | ≥ -117.0 | N/A | N/A | N/A |
|  |  | FDD_H, TDD_H | N/A | N/A | ≥ -116.5 | N/A | N/A | N/A |
|  |  | FDD_N, TDD_N | N/A | N/A | ≥ -113.5 | N/A | N/A | N/A |
| NOTE 1: The power level is specified at RSRP reference point as defined in TS 36.214 [24] NOTE 2: E-UTRA Band groups are defined in TS 36.133 [12] clause 3.5.1. |  |  |  |  |  |  |  |  |

Table 4.7-4: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |  |
| cqi-ReportModeAperiodic |  | NOT PRESENT |  |  |
| cqi-ReportPeriodic |  | NOT PRESENT |  |  |
| } |  |  |  |  |

Table 4.7-5: UplinkPowerControlCommon-DEFAULT : Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, UplinkPowerControlCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalPUSCH | -60 (-60 dBm) | To attain maximum power from the DUT |  |
| } |  |  |  |

Table 4.7-6: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| } |  |  |  |  |

Table 4.7-7: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated |  | Infinity |  |  |

## 4.8	E-UTRA configuration for EN-DC FR1 tests not applying the E-UTRA anchor-agnostic approach

Unless otherwise stated, the following message exception defined in tables 4.8-1, 4.8-2 and 4.8-3 apply for EN-DC FR1 tests not applying the E-UTRA anchor-agnostic approach.

Table 4.8-1: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |  |
| cqi-ReportModeAperiodic |  | NOT PRESENT |  |  |
| cqi-ReportPeriodic |  | NOT PRESENT |  |  |
| } |  |  |  |  |

Table 4.8-2: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| } |  |  |  |  |

Table 4.8-3: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated |  | Infinity |  |  |

## <<< END OF CHANGES >>>