3GPP TSG-RAN5 Meeting #90-e	R5-210549

# Electronic Meeting, 22 February – 5 March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5253 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding subPRB allocation to test case 6.2.3EC, Maximum Power Reduction (MPR) for UE category M2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the WP, subPRB allocation should be added to test case 6.2.3EC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added subPRB allocation to test configuration table New test requirement tables added Test procedure updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | efeMTC WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3EC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

### 6.2.3EC	Maximum Power Reduction (MPR) for UE category M2

#### 6.2.3EC.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

#### 6.2.3EC.2	Test applicability

The requirements of this test apply in test case 6.6.2.3EC Adjacent Channel Leakage power Ratio for UE category M2 to all types of E-UTRA UE release 14 and forward of UE category M2.

NOTE:	As a result TC 6.2.3EC has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.3EC.3	Minimum conformance requirements

For category M2 UE Power Class 1,2,5 and 6, ,3, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2EC-1 due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3EC.3-1,Table 6.2.3EC.3-2 and Table 6.2.3EC.3-3.

For subPRB allocation of category M2 UE of Power Class 3, the allowed MPR due to higher order modulation and transmit bandwidth configuration (subcarrier) is specified in in Table 6.2.3EC-4.

Table 6.2.3EC.3-1: Maximum Power Reduction (MPR) for category M2 UE for Power Class 2 and 3

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >1 | >4 | > 16 | > 18 | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >1 | >3 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16QAM | >2 | >2 | >3 | >5 | > 16 | > 18 | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1. |  |  |  |  |  |  |  |

Table 6.2.3EC.3-2: Maximum Power Reduction (MPR) for category M2 UE for Power Class 5

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >3 | >5 | > 16 | > 18 | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >3 | >5 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16QAM | >2 | >2 | >5 | - | > 16 | > 18 | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1. |  |  |  |  |  |  |  |

Table 6.2.3EC.3-3: Maximum Power Reduction (MPR) for category M2 UE for Power Class 6

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >3 | >5 | > 16 | > 18 | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >3 | >5 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16QAM | >2 | >2 | >5 | - | > 16 | > 18 | ≤ 2 |

Table 6.2.3EC.3-4: Maximum Power Reduction (MPR) for category M2 UE for Power Class 3 for subPRB allocation

| Channel bandwidth | 5 MHz |  | 10 MHz |  |  |  | 15MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | BPSK | QPSK | BPSK |  | QPSK |  | BPSK |  | QPSK |  |
| WB index for 2 subcarrier transmission | 0 | - | 0 | 1 | - |  | 0 | 2 | - |  |
| MPR | ≤[0.5] | - | ≤[0.5] | ≤[0.5] | - |  | ≤[0.5] | ≤[0.5] | - |  |
| WB index for 3 subcarrier transmission | - | 0 | - |  | 0 | 1 | - |  | 0 | 2 |
| MPR | - | ≤[2] | - |  | ≤[2] | ≤[2] | - |  | ≤[2] | ≤[2] |
| WB index for 6 subcarrier transmission | - | 0 | - |  | 0 | 1 | - |  | 0 | 2 |
| MPR | - | ≤[1] | - |  | ≤[1] | ≤[1] | - |  | ≤[1] | ≤[1] |
| NOTE: WB is specified in TS 36.211 [4] |  |  |  |  |  |  |  |  |  |  |

For PRACH, PUCCH and SRS transmissions, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each subframe, the MPR is evaluated per slot and given by the maximum value taken over the transmission(s) within the slot; the maximum MPR over the two slots is then applied for the entire subframe.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5EC apply.

No other MPR requirement than those specified in Table 6.2.3EC.3-1 and Table 6.2.3EC.3-2 and and Table 6.2.3EC.3-3 applies to category M2 UE.

The normative reference for this requirement is TS 36.101 clause 6.2.3E.

#### 6.2.3EC.4	Test description

##### 6.2.3EC.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.3EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2.3EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, 15 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 0 |
| 8 | 3MHz |  | QPSK | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 13 | 5MHz |  | QPSK | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 15 | 5MHz |  | 16QAM | 3 | 0 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 17 | 5MHz |  | 16QAM | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 20 | 10MHz |  | QPSK | 24 | 0 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 22 | 10MHz |  | 16QAM | 5 | 0 |
| 23 | 10MHz |  | 16QAM | 24 | 0 |
| 24 | 15MHz |  | QPSK | 16 | 0 |
| 25 | 15MHz |  | QPSK | 24 | 0 |
| 26 | 15MHz |  | 16QAM | 16 | 0 |
| 27 | 15MHz |  | 16QAM | 24 | 0 |
| 28 | 20MHz |  | QPSK | 18 | 0 |
| 29 | 20MHz |  | QPSK | 24 | 0 |
| 30 | 20MHz |  | 16QAM | 18 | 0 |
| 31 | 20MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 1 |
| 8 | 3MHz |  | QPSK | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 13 | 5MHz |  | QPSK | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 15 | 5MHz |  | 16QAM | 3 | 3 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 17 | 5MHz |  | 16QAM | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 20 | 10MHz |  | QPSK | 24 | 4 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 22 | 10MHz |  | 16QAM | 5 | 7 |
| 23 | 10MHz |  | 16QAM | 24 | 4 |
| 24 | 15MHz |  | QPSK | 15 | 9 |
| 25 | 15MHz |  | QPSK | 24 | 8 |
| 26 | 15MHz |  | 16QAM | 15 | 9 |
| 27 | 15MHz |  | 16QAM | 24 | 8 |
| 28 | 20MHz |  | QPSK | 18 | 13 |
| 29 | 20MHz |  | QPSK | 24 | 12 |
| 30 | 20MHz |  | 16QAM | 18 | 13 |
| 31 | 20MHz |  | 16QAM | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 UE |  |  |  |  |  |

Table 6.2.3EC.4.1-2: Test Configuration Table, subPRB allocation

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, 15 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 5MHz |  | π/2-BPSK | ¼ | 0 |
| 2 | 5MHz |  | QPSK | ¼ | 0 |
| 3 | 5MHz |  | QPSK | ½ | 0 |
| 4 | 10MHz |  | π/2-BPSK | ¼ | 0 |
| 5 | 10MHz |  | π/2-BPSK | ¼ | 4 |
| 6 | 10MHz |  | QPSK | ¼ | 0 |
| 7 | 10MHz |  | QPSK | ¼ | 4 |
| 8 | 10MHz |  | QPSK | ½ | 0 |
| 9 | 10MHz |  | QPSK | ½ | 4 |
| 10 | 15MHz |  | π/2-BPSK | ¼ | 0 |
| 11 | 15MHz |  | π/2-BPSK | ¼ | 8 |
| 12 | 15MHz |  | QPSK | ¼ | 0 |
| 13 | 15MHz |  | QPSK | ¼ | 8 |
| 14 | 15MHz |  | QPSK | ½ | 0 |
| 15 | 15MHz |  | QPSK | ½ | 8 |
| High range |  |  |  |  |  |
| 16 | 5MHz |  | π/2-BPSK | ¼ | 0 |
| 17 | 5MHz |  | QPSK | ¼ | 0 |
| 18 | 5MHz |  | QPSK | ½ | 0 |
| 19 | 10MHz |  | π/2-BPSK | ¼ | 0 |
| 20 | 10MHz |  | π/2-BPSK | ¼ | 4 |
| 21 | 10MHz |  | QPSK | ¼ | 0 |
| 22 | 10MHz |  | QPSK | ½ | 4 |
| 23 | 10MHz |  | QPSK | ¼ | 0 |
| 24 | 10MHz |  | QPSK | ½ | 4 |
| 25 | 15MHz |  | π/2-BPSK | ¼ | 0 |
| 26 | 15MHz |  | π/2-BPSK | ¼ | 8 |
| 27 | 15MHz |  | QPSK | ¼ | 0 |
| 28 | 15MHz |  | QPSK | ¼ | 8 |
| 29 | 15MHz |  | QPSK | ½ | 0 |
| 30 | 15MHz |  | QPSK | ½ | 8 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.3EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.2.3EC.4.3.

##### 6.2.3EC.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.2.3EC.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	For UE supporting subPRB allocation, repeat step 1-3 with UL RMC according to Table 6.2.3EC.4.1-2.

##### 6.2.3EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

#### 6.2.3EC.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3EC.5-1, 6.2.3EC.5-2 and6.2.3EC.5-3.

Table 6.2.3EC.5-1: UE Power Class 3 and 5 test requirements for bands without TC,c (Low/Mid/High range) and bands with TC,c (Mid range) except band 28 and band 71

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 0 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 3 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 4 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 5 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 6 | 0 | 23 | 2 | 25.7 | 20.3 | 0 | 20 | 2 | 22.7 | 17.3 |
| 7 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 8 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 9 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 10 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 11 | 0 | 23 | 2 | 25.7 | 20.3 | N/A |  |  |  |  |
| 12 | N/A |  |  |  |  | 0 | 20 | 2 | 22,7 | 17,3 |
| 13 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 14 | 0 | 23 | 2 | 25.7 | 20.3 | N/A |  |  |  |  |
| 15 | 1 | 22 | 2 | 25.7 | 19.3 | 0 | 20 | 2 | 22.7 | 17.3 |
| 16 | NA |  |  |  |  | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 17 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20,7 | 13,3 |
| 18 | 0 | 23 | 2 | 25,7 | 20,3 | N/A |  |  |  |  |
| 19 | N/A |  |  |  |  | 0 | 20 | 2 | 22,7 | 17,3 |
| 20 | 1 | 22 | 2 | 25,7 | 19,3 | 1 | 19 | 3,5 | 21,7 | 14,8 |
| 21 | 0 | 23 | 2 | 25,7 | 20,3 | N/A |  |  |  |  |
| 22 | 1 | 22 | 2 | 25,7 | 19,3 | 0 | 20 | 2 | 22,7 | 17,3 |
| 23 | 2 | 21 | 2 | 25,7 | 18,3 | 2 | 18 | 4 | 20,7 | 13,3 |
| 24 | 0 | 23 | 2 | 25,7 | 20,3 | 0 | 20 | 2 | 22,7 | 17,3 |
| 25 | 1 | 22 | 2 | 25,7 | 19,3 | 1 | 19 | 3,5 | 21,7 | 14.8 |
| 26 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 27 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4 | 20.7 | 13.3 |
| 28 | 0 | 23 | 2 | 25.7 | 20.3 | 0 | 20 | 2 | 22.7 | 17.3 |
| 29 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 30 | 1 | 22 | 2 | 25.7 | 19.3 | 1 | 19 | 3.5 | 21.7 | 14.8 |
| 31 | 2 | 21 | 2 | 25.7 | 18.3 | 2 | 18 | 4.0 | 20.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3EC.5-1a: UE Power Class 3 test requirements for bands without TC,c (Low/Mid/High range) and bands with TC,c (Mid range) except band 28 and band 71, subPRB allocation

| Configuration ID | Power class 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) |  | PCMAX_L,c (dBm) |  | T(PCMAX_L,c) (dB) |  |  | Upper limit (dBm) |  | Lower limit (dBm) |
| 1 | 0.5 |  | 22.5 |  | 2 |  |  | 25.7 |  | 19.8 |
| 2 | 2 |  | 21.0 |  | 2 |  |  | 25.7 |  | 18.3 |
| 3 | 1 |  | 22.0 |  | 2 |  |  | 25.7 |  | 19.3 |
| 4 | 0.5 |  | 22.5 |  | 2 |  |  | 25.7 |  | 19.8 |
| 5 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 6 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 7 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 8 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 9 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 10 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 11 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 12 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 13 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 14 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 15 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 16 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 17 | 2 |  | 21.0 |  | 2 |  |  | 25.7 |  | 18.3 |
| 18 | 1 |  | 22.0 |  | 2 |  |  | 25.7 |  | 19.3 |
| 19 | 0.5 |  | 22.5 |  | 2 |  |  | 25.7 |  | 19.8 |
| 20 | 0.5 |  | 22.5 |  | 2 |  |  | 25.7 |  | 19.8 |
| 21 | 2 |  | 21.0 |  | 2 |  |  | 25.7 |  | 18.3 |
| 22 | 2 |  | 21.0 |  | 2 |  |  | 25.7 |  | 18.3 |
| 23 | 1 |  | 22.0 |  | 2 |  |  | 25.7 |  | 19.3 |
| 24 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 25 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 26 | 0.5 | 22.5 | 2 | 25.7 | 19.8 |
| 27 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 28 | 2 | 21.0 | 2 | 25.7 | 18.3 |
| 29 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| 30 | 1 | 22.0 | 2 | 25.7 | 19.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |

Table 6.2.3EC.5-2: UE Power Class 3 and 5 test requirements for bands with TC,c (Low/High range)

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 2 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 3 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 4 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 5 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 6 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 7 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 8 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 9 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 10 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 11 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 12 | N/A |  |  |  |  | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 13 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 14 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 15 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 16 | NA |  |  |  |  | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 17 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 18 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 19 | N/A |  |  |  |  |  |  |  |  | 0 |
| 20 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 21 | 0 | 21.5 | 2 | 25.7 | 18.8 | N/A |  |  |  |  |
| 22 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 5 | 24.2 | 12.8 |
| 23 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 24 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 25 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 26 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 27 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| 28 | 0 | 21.5 | 2 | 25.7 | 18.8 | 0 | 18.5 | 4 | 23.2 | 13.8 |
| 29 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 30 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5 | 23.2 | 11.8 |
| 31 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5 | 22.2 | 10.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3EC.5-2a: UE Power Class 3 test requirements for bands with TC,c (Low/High range), subPRB allocation

| Configuration ID | Power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 2 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 3 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 4 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 5 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 6 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 7 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 8 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 9 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 10 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 11 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 12 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 13 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 14 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 15 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 16 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 17 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 18 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 19 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 20 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 21 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 22 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 23 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 24 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 25 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 26 | 0.5 | 21.0 | 2.0 | 25.7 | 18.3 |
| 27 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 28 | 2 | 19.5 | 3.5 | 27.2 | 15.3 |
| 29 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| 30 | 1 | 20.5 | 2.5 | 26.2 | 17.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-3: UE Power Class 3 and 5 test requirements for band 28 and band 71

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2.5 | 26.2 | 19.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 2 | 1 | 22 | 2.5 | 26.2 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 3 | 2 | 21 | 2.5 | 26.2 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 4 | 1 | 22 | 2.5 | 26.2 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 5 | 2 | 21 | 2.5 | 26.2 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 6 | 0 | 23 | 2.5 | 26.2 | 19.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 7 | 1 | 22 | 2.5 | 26.2 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 8 | 2 | 21 | 2.5 | 26.2 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 9 | 1 | 22 | 2.5 | 26.2 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 10 | 2 | 21 | 2.5 | 26.2 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 11 | 0 | 23 | 2.5 | 26.2 | 19.8 | N/A |  |  |  |  |
| 12 | N/A |  |  |  |  | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 13 | 1 | 1 | 19 | 3.5 | 23.2 | 1 | 19 | 3.5 | 23.2 | 14,8 |
| 14 | 0 | 23 | 2.5 | 26.2 | 19.8 | N/A |  |  |  |  |
| 15 | 1 | 22 | 2.5 | 26.2 | 18.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 16 | N/A |  |  |  |  | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 17 | 2 | 21 | 2.5 | 25.7 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 18 | 0 | 23 | 2.5 | 25.7 | 19.8 | N/A |  |  |  |  |
| 19 | N/A |  |  |  |  | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 20 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 21 | 0 | 23 | 2.5 | 25.7 | 19.8 | N/A |  |  |  |  |
| 22 | 1 | 22 | 2.5 | 25.7 | 18.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 23 | 2 | 21 | 2.5 | 25.7 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 24 | 0 | 23 | 2.5 | 25.7 | 19.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 25 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 26 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 27 | 2 | 21 | 2.5 | 25.7 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| 28 | 0 | 23 | 2.5 | 25.7 | 19.8 | 0 | 20 | 2.5 | 23.2 | 16.8 |
| 29 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 30 | 1 | 22 | 2.5 | 25.7 | 18.8 | 1 | 19 | 3.5 | 23.2 | 14.8 |
| 31 | 2 | 21 | 2.5 | 25.7 | 17.8 | 2 | 18 | 4 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3EC.5-3a: UE Power Class 3 test requirements for band 28 and band 71, subPRB allocation

| Configuration ID | Power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 2 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 3 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 4 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 5 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 6 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 7 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 8 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 9 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 10 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 11 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 12 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 13 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 14 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 15 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 16 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 17 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 18 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 19 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 20 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 21 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 22 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 23 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 24 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 25 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 26 | 0.5 | 22.5 | 2.5 | 26.2 | 19.3 |
| 27 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 28 | 2 | 21.0 | 2.5 | 26.2 | 17.8 |
| 29 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| 30 | 1 | 22.0 | 2.5 | 26.2 | 18.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-4: UE Power Class 2 test requirements for bands without TC,c (Low/Mid/High range) and bands with TC,c (Mid range)

| Configuration ID | Power class 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 2 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 3 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 4 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 5 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 6 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 7 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 8 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 9 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 10 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 11 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 12 | N/A |  |  |  |  |
| 13 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 14 | 2 | 24 | 2.0 | 28.7 | 21.3 |
| 15 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 16 | N/A |  |  |  |  |
| 17 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| 18 | 1 | 25 | 2.0 | 28.7 | 22.3 |
| 19 | N/A |  |  |  |  |
| 20 | 0 | 26 | 2.0 | 28.7 | 23.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5. Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-5: UE Power 2 test requirements for bands with TC,c (Low/High range)

| Configuration ID | Power class 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| 2 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 3 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 4 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 5 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 6 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| 7 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 8 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 9 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 10 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 11 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 12 | N/A |  |  |  |  |
| 13 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 14 | 2 | 22.5 | 2.0 | 28.7 | 19.8 |
| 15 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| 16 | N/A |  |  |  |  |
| 17 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| 18 | 1 | 23.5 | 2.0 | 28.7 | 20.8 |
| 19 | N/A |  |  |  |  |
| 20 | 0 | 24.5 | 2.0 | 28.7 | 21.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5. Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1 |  |  |  |  |  |

Table 6.2.3EC.5-6: UE Power Class 6 test requirements for bands without TC,c (Low/Mid/High range) and bands with TC,c (Mid range)

| Configuration ID | Power class 6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 2 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 3 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 4 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 5 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 6 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 7 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 8 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 9 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 10 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 11 | N/A |  |  |  |  |
| 12 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 13 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 14 | N/A |  |  |  |  |
| 15 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 16 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 17 | 2 | 12 | 3.5 | 16.2 | 7.8 |
| 18 | N/A |  |  |  |  |
| 19 | 0 | 14 | 2.5 | 17.2 | 10.8 |
| 20 | 1 | 13 | 2.5 | 16.2 | 9.8 |
| 21 | N/A |  |  |  |  |
| 22 | 0 | 14 | 2.5 | 22.7 | 17.3 |
| 23 | 2 | 13 | 2.5 | 20.7 | 13.3 |
| 24 | 0 | 14 | 2.5 | 22.7 | 17.3 |
| 25 | 1 | 13 | 2.5 | 21.7 | 14.8 |
| 26 | 1 | 13 | 2.5 | 21.7 | 14.8 |
| 27 | 2 | 12 | 3.5 | 20.7 | 13.3 |
| 28 | 0 | 14 | 2.5 | 22.7 | 17.3 |
| 29 | 1 | 13 | 2.5 | 21.7 | 14.8 |
| 30 | 1 | 13 | 2.5 | 21.7 | 14.8 |
| 31 | 2 | 12 | 3.5 | 20.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5. Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1. |  |  |  |  |  |

Table 6.2.3EC.5-7: UE Power 6 test requirements for bands with TC,c (Low/High range)

| Configuration ID | Power class 6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 13 | 2.5 | 15.7 | 9.3 |
| 2 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 3 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 4 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 5 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 6 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 7 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 8 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 9 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 10 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 11 | N/A |  |  |  |  |
| 12 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 13 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 14 | N/A |  |  |  |  |
| 15 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 16 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 17 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 18 | N/A |  |  |  |  |
| 19 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 20 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 21 | N/A |  |  |  |  |
| 22 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 23 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 24 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 25 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 26 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 27 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| 28 | 0 | 12.5 | 3.5 | 16.7 | 8.3 |
| 29 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 30 | 1 | 11.5 | 4 | 16.2 | 6.8 |
| 31 | 2 | 10.5 | 5 | 16.2 | 4.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5. Note 2: TC,c applicability per band is indicated by note 2 in Table 6.2.2EA-1. |  |  |  |  |  |

<< End of changes>>