3GPP TSG-RAN5 Meeting #90-e	R5-210566

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0150 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on manufacturer declaration required for Receiver Beam Peak Search |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Table A.4.3.9-8 of 38.508-2 shows that the Beam Peak Search is performed for both waveforms, as it refers to 38.521-2 Annex K.1.2. Neverthless, DL signal only uses CP-OFDM waveform in 38.521-2 receiver test cases. Therefore, table A.4.3.9-8 of 38.508-2 should be applicable only to transmitter test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the 38.521-2, Annex K.1.2 reference in Table A.4.3.9-8 of 38.508-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specifications will remain misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table A.4.3.9-8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-2 CR0467 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### A.4.3.9	Additional capabilities for UE declared capability

Table A.4.3.9-1: UE declared capabilities

| Item | UE declared capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Enhanced Type 1 Receiver for NR | 38.101-4, 5 | Rel-15 | pc_nr_enh_type1_receiver | Support for Enhanced Type 1 Receiver (SU-MIMO Interference Mitigation advanced receiver) |
| 2 | Vehicular UE | 38.101-1, 3 | Rel-15 | pc_nr_vehicular_ue |  |

Table A.4.3.9-2: UE declared multi-band peak EIRP relaxation factors for FR2 power class 3

| Item | Supported FR2 bands set | Ref. | Release | peak EIRP relaxation factor per band, MBp (dB) (Note 1) |  |  |  | Maximum sum of MBp, ∑MBP (dB) (Note 2) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | n257 | n258 | n260 | n261 |  |  |
| 1 | n257, n258 | 38.101-2, 6.2.1.3 | Rel-15 |  |  | N/A | N/A | 1.3 |  |
| 2 | n257, n260 |  |  |  | N/A |  | N/A | 1.0 |  |
| 3 | n258, n260 |  |  | N/A |  |  | N/A | 1.0 |  |
| 4 | n258, n261 |  |  | N/A |  | N/A |  | 1.0 |  |
| 5 | n260, n261 |  |  | N/A | N/A | N/A | N/A | 0.0 | No relaxation factor allowed |
| 6 | n257, n258, n260 |  |  |  |  |  | N/A | 1.7 |  |
| 7 | n257, n258, n261 |  |  |  |  | N/A |  | 1.7 |  |
| 8 | n257, n260, n261 |  |  |  | N/A |  |  | 0.5 |  |
| 9 | n258, n260, n261 |  |  | N/A |  |  |  | 1.5 |  |
| 10 | n257, n258, n260, n261 |  |  |  |  |  |  | 1.7 |  |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3. Note 2: Max allowed sum of MBp over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>

Table A.4.3.9-3: UE declared multi-band peak EIRP Spherical coverage relaxation factors for FR2 power class 3

| Item | Supported FR2 bands set | Ref. | Release | EIRP Spherical coverage relaxation factor per band, MBs (dB) (Note 1) |  |  |  | Maximum sum of MBs, ∑MBs (dB) (Note 2) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | n257 | n258 | n260 | n261 |  |  |
| 1 | n257, n258 | 38.101-2, 6.2.1.3 | Rel-15 |  |  | N/A | N/A | 1.25 |  |
| 2 | n257, n260 |  |  |  | N/A |  | N/A | 0.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 3 | n258, n260 |  |  | N/A |  |  | N/A | 0.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 4 | n258, n261 |  |  | N/A |  | N/A |  | 1.25 |  |
| 5 | n260, n261 |  |  | N/A | N/A |  |  | 0.75 | No relaxation allowed for n260 |
| 6 | n257, n258, n260 |  |  |  |  |  | N/A | 1.75 | Maximum 0.4 dB relaxation allowed for n260 |
| 7 | n257, n258, n261 |  |  |  |  | N/A |  | 1.75 |  |
| 8 | n257, n260, n261 |  |  |  | N/A |  |  | 1.25 | Maximum 0.4 dB relaxation allowed for n260 |
| 9 | n258, n260, n261 |  |  | N/A |  |  |  | 1.25 | Maximum 0.4 dB relaxation allowed for n260 |
| 10 | n257, n258, n260, n261 |  |  |  |  |  |  | 1.75 | Maximum 0.4 dB relaxation allowed for n260 |
| Note 1: UE vendor to fill in the needed relaxation factor per band that is ≥0. One row to be filled in, the one matching the supported FR2 bands of the UE as declared in Table A.4.3.1-3 Note 2: Max allowed sum of MBs over all supported FR2 bands as defined in TS 38.521-2 clause 6.2.1.1.3.3 |  |  |  |  |  |  |  |  |  |

Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.2 | Rel-15 |  |
| 2 | FDD Band n2 | 38.101-1, 7.2 | Rel-15 |  |
| 3 | FDD Band n3 | 38.101-1, 7.2 | Rel-15 |  |
| … |  |  |  |  |
| 7 | FDD Band n7 | 38.101-1, 7.2 | Rel-15 | Mandatory for non-vehicular UE if band support is indicated in Table A.4.3.1-1 |
| ... |  |  |  |  |
| 28 | FDD Band n28 | 38.101-1, 7.2 | Rel-16 | 4 Rx operation is targeted for FWA form factor |
| ... |  |  |  |  |
| 30 | FDD Band n30 | 38.101-1, 7.2 | Rel-16 |  |
| ... |  |  |  |  |
| 66 | FDD Band n66 | 38.101-1, 7.2 | Rel-15 |  |
| … |  |  |  |  |
| 70 | FDD Band n70 | 38.101-1, 7.2 | Rel-15 |  |
| 71 | FDD Band n71 | 38.101-1, 7.2 | Rel-16 | 4 Rx operation is targeted for FWA form factor |

Table A.4.3.9-4b: TDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 34 | TDD Band n34 | 38.101-1, 7.2 | Rel-15 |  |
| ... |  |  |  |  |
| 38 | TDD Band n38 | 38.101-1, 7.2 | Rel-15 | Mandatory for non-vehicular UE if band support is indicated in Table A.4.3.1-2 |
| 39 | TDD Band n39 | 38.101-1, 7.2 | Rel-15 |  |
| ... |  |  |  |  |
| 40 | TDD Band n40 | 38.101-1, 7.2 | Rel-15 |  |
| 41 | TDD Band n41 | 38.101-1, 7.2 | Rel-15 | Mandatory for non-vehicular UE if band support is indicated in Table A.4.3.1-2 |
| ... |  |  |  |  |
| 48 | TDD Band n48 | 38.101-1, 7.2 | Rel-16 | Mandatory for non-vehicular UE if band support is indicated in Table A.4.3.1-2 |
| … |  |  |  |  |
| 77 | TDD Band n77 | 38.101-1, 7.2 | Rel-15 | Mandatory for non-vehicular UE if band support is indicated in Table A.4.3.1-2 |
| 78 | TDD Band n78 | 38.101-1, 7.2 | Rel-15 | Mandatory for non-vehicular UE if band support is indicated in Table A.4.3.1-2 |
| 79 | TDD Band n79 | 38.101-1, 7.2 | Rel-15 | Mandatory for non-vehicular UE if band support is indicated in Table A.4.3.1-2 |

Table A.4.3.9-4c: 2 Rx antenna ports Capabilities

| Item | Band | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3.2 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3.2 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3.2 |  |
| 4 | FDD Band n5 | 38.101-1, 7.3.2 |  |
| 5 | FDD Band n7 | 38.101-1, 7.3.2 |  |
| 6 | FDD Band n8 | 38.101-1, 7.3.2 |  |
| 7 | FDD Band n12 | 38.101-1, 7.3.2 |  |
| 7a | Reserved |  |  |
| 7b | FDD Band n14 | 38.101-1, 7.3.2 |  |
| 8 | FDD Band n20 | 38.101-1, 7.3.2 |  |
| 9 | FDD Band n25 | 38.101-1, 7.3.2 |  |
| 10 | FDD Band n28 | 38.101-1, 7.3.2 |  |
| 10a | FDD Band n29 | 38.101-1, 7.3A.2.4 |  |
| 10b | FDD Band n30 | 38.101-1, 7.3.2 |  |
| 11 | TDD Band n34 | 38.101-1, 7.3.2 |  |
| 12 | TDD Band n38 | 38.101-1, 7.3.2 |  |
| 13 | TDD Band n39 | 38.101-1, 7.3.2 |  |
| 14 | TDD Band n40 | 38.101-1, 7.3.2 |  |
| 15 | TDD Band n41 | 38.101-1, 7.3.2 |  |
| 16 | TDD Band n48 | 38.101-1, 7.3.2 |  |
| 17 | TDD Band n50 | 38.101-1, 7.3.2 |  |
| 18 | TDD Band n51 | 38.101-1, 7.3.2 |  |
| 19 | FDD Band n65 | 38.101-1, 7.3.2 |  |
| 20 | FDD Band n66 | 38.101-1, 7.3.2 |  |
| 21 | FDD Band n70 | 38.101-1, 7.3.2 |  |
| 22 | FDD Band n71 | 38.101-1, 7.3.2 |  |
| 23 | FDD Band n74 | 38.101-1, 7.3.2 |  |
| 24 | TDD Band n77 | 38.101-1, 7.3.2 |  |
| 25 | TDD Band n78 | 38.101-1, 7.3.2 |  |
| 26 | TDD Band n79 | 38.101-1, 7.3.2 |  |

Table A.4.3.9-5: Beam Peak Search Vendor Declarations with respect to test frequency range for single CC

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1. |
| 3 | n260 | n260 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from mid to low and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 5 | n261 | n261 single CC beam peak is leveraged from n257 single CC mid channel to n261 low, mid and high channels | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 2 |
| NOTE 1: The beam peak searches shall be performed for every test frequency range by default unless the device manufacturer explicitly declares that the beam peak at the mid test frequency range is applicable for the remaining (low, high) test frequency ranges. NOTE 2: Beam peak search results can be re-used from bands that completely contain the target bands if explicitly declared by the manufacturer. |  |  |  |  |  |

Table A.4.3.9-6: Beam Peak Search Vendor Declarations with respect to test frequency range for different CA BW classes

| Item | Bands | NR CA bandwidth class | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258, n260, n261 | A, B, C, D, E, F, G, H, I, J, K, L, M, N, O, P, Q | The beam peak is leveraged from a reference (frequency band, CBW) or (frequency band combination, CA BW class) to a group of other intra-band contiguous combinations and CA BW classes | 38.521-2, K.1.1 & K.1.2 | Rel-15 | A beam peak search shall be performed for every intra-band contiguous combination and CA BW class by default unless the device manufacturer explicitly declares that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes. |

Table A.4.3.9-7: Beam Peak Search Vendor Declarations with respect to modulation for single CC

| Item | Band | Intent | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from QPSK modulation to 16QAM and 64QAM | 38.521-2, K.1.1 & K.1.2 | Rel-15 | NOTE 1 |
| NOTE 1: The beam peak searches shall be performed for every modulation by default unless the device manufacturer explicitly declares that the beam peak at the QPSK modulation is applicable for the remaining 16QAM and 64QAM modulations. |  |  |  |  |  |

Table A.4.3.9-8: Beam Peak Search Vendor Declarations with respect to waveform for single CC

| Item | Band | Intent | Reference Waveform | Ref. | Release | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | n257 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 2 | n258 | n258 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 3 | n260 | n260 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| 4 | n261 | n261 single CC beam peak is leveraged from the reference waveform to the other waveform | CP-OFDM or DFT-s-OFDM | 38.521-2, K.1.1 | Rel-15 | NOTE 1 |
| NOTE 1: The beam peak searches shall be performed for every waveform by default unless the device manufacturer explicitly declares that the beam peak from one waveform is applicable for the other waveform. |  |  |  |  |  |  |

<< End of change>>