3GPP TSG RAN Meeting #90-e		R5-210617

Electronic Meeting, 22nd February – 5th March 2021

Agenda item:	5.3.2.17

Source:	Apple Portugal

Title:	AP#87e.24 Input for Large Device Size

Document for:	Discussion and Endorsement

# 1	Introduction

The purpose of this discussion paper is to provide inputs to the following action point [1], specifically with respect to size of large devices (laptops, routers):

| Action ID | sWG | Action | Responsible | Relevant Tdoc |
| --- | --- | --- | --- | --- |
| AP#87e.24 | RF | OEM vendors and operators to provide data on max antenna separation distances and device size for NR FR2 devices that can currently not be tested with the largest QZ size of 30cm in diameter. | OEMs and Operators | R5-202082 R5-202083 R5-202084 R5-202085 R5-198262 R5-204191 R5-204200 |

# 2	Discussion

This contribution further addresses AP#87e.24 to provide our view on how to handle test coverage for large devices. As stated in [4], having many QZ sizes would not only significantly increase the complexity of validation and certification, but also have a cost of investing in many CATRs to accommodate many QZ sizes. Having two QZ sizes would help reduce complexity of FR2 testing and certification for OEMs. First QZ size is for handsets and tablets, and second QZ size is for other large devices such as laptop and router.

[4] shared observations on the first category of devices listed above (handsets and tablets) and created an observation on extending current QZ size to accommodate such devices.

Observation 1: Extending QZ size beyond 30 cm, to say 35 cm, in current test systems with grey-box approach would cover most of handsets and tablets and limit the device size at 40 cm.

With reference to large devices such as laptops and routers, [2] at RAN5#85 provided a reference survey of all commercially available laptops with WWAN (LTE) connectivity in terms of dimensions and [3] at RAN5#87e discussed the subject of larger QZ sizes which led to the aforementioned AP#87e.24.

Further the discussion around [4] at RAN5#89 spawned an Action Point for TE vendors to provide QoQZ and XPD MU impact for extended QZ sizes.

Observation 2: Manufacturer input requested vis-à-vis size of larger devices (laptops, routers).

Proposal 1: Consider 55 cm as a maximum size of large devices (in the category of laptops, routers) when evaluating FR2 test coverage, computing QZ and the associated QoQZ and XPD MU impact.

# 3	Conclusions

In this contribution, the following observations and proposal were made

Observation 1: Extending QZ size beyond 30 cm, to say 35 cm, in current test systems with grey-box approach would cover most of handsets and tablets and limit the device size at 40 cm.

Observation 2: Manufacturer input requested vis-à-vis size of larger devices (laptops, routers).

Proposal 1: Consider 55 cm as a maximum size of large devices (in the category of laptops, routers) when evaluating FR2 test coverage, computing QZ and the associated QoQZ and XPD MU impact.

# 4	References

R5-203204_Action_Points_RAN5#87-e_atRFsession

R5-198262, On Larger Quiet Zone Sizes, Keysight Technologies UK Ltd, 3GPP TSG RAN WG5 Meeting #85-bis, November 2019

R5-202082, Discussion on size of Quiet Zone above 30 cm, Rohde & Schwarz, 3gPP TSG RAN WG5 Meeting #87-e, May 2020

R5-205713, AP#87e.23 Test coverage for large devices, Apple Inc, 3GPP TSG RAN WG5 Meeting #89-e, November 2020