3GPP TSG-RAN5 Meeting #90-e	R5-210665

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0110 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR IMS TC 8.31-Creating and leaving conference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently there is no test coverage to verfiy that an UE could correctly handle creating and leaving conference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TC 8.31 for creating and leaving conference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No testing of UE compliance for such use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.31 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2... CR 0278 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.31	Creating and leaving a conference / 5GS

8.31.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to start a conference call }

then { UE sends INVITE to the conference factory and completes the conference call initiation and subscribes to conference event }

}

(2)

with { Conference call going on }

ensure that {

when { UE is made to leave the call }

then { UE sends BYE and processes notification for conf event if any }

}

8.31.2	Conformance Requirements

[TS 24.147, clause 5.3.1.3.1]:

A conference can be created by means of SIP, as described in subclause 5.3.1.3.2 or subclause 5.3.1.3.3.

NOTE:	Additionally, creation of a conference can be provided by other means.

The conference participant shall make use of the procedures for session establishment as described in subclauses 5.1.2A and 5.1.3 of 3GPP TS 24.229 [5] when creating conferences by means of SIP.

[TS 24.147, clause 5.3.1.3.2]:

Upon a request to create a conference with a conference factory URI, the conference participant shall:

1)	generate an initial INVITE request in accordance with subclause 5.1.3.1 of 3GPP TS 24.229 [5]; and

2)	set the request URI of the INVITE request to the conference factory URI.

On receiving a 200 (OK) response to the INVITE request with the "isfocus" feature parameter indicated in Contact header, the conference participant shall store the content of the received Contact header as the conference URI. In addition to this, the conference participant may subscribe to the conference event package as described in RFC 4575 [11] by using the stored conference URI.

NOTE 1:	A conference participant can decide not to subscribe to the conference event package for conferences with a large number of attendees, due to, e.g. the signalling traffic caused by the notifications about users joining or leaving the conference.

NOTE 2:	A conference can also be created with a conference URI. The procedures for this case at the conference participant are identical to those for joining a conference, as described in subclause 5.3.1.4.1. It is not assumed that the conference participant is aware that the conference gets created in this case.

NOTE 3:	The UE can discover the conference factory URI from the Management Object as defined in 3GPP TS 24.166 [38]. Further discovery mechanisms for the conference factory URI are outside the scope of the present document.

3GPP TS 24.147 [35] clause 5.3.1.3.1 and 5.3.1.3.2.

8.31.3	Test description

8.31.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1) and registered to IMS.

8.31.3.2	Test procedure sequence

Table 8.31.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt an IMS Conference call |  |  |  |  |
| 2 | Check: Does the UE send INVITE with the first SDP offer? | --> | INVITE | 1 | P |
| 3-8 | Steps 2-7 of annex A4.1 happen | - | - | - | - |
| 9 | The SS responds INVITE with 200 OK and gives the final conference URI within the response | <-- | 200 OK | - | - |
| 10 | The UE acknowledges the receipt of 200 OK for INVITE | --> | ACK | - | - |
|  | EXCEPTION: steps 11 – 14 describe optional behaviour depending on UE configuration. The SS shall wait up to 3s for the SUBSCRIBE of step 11 | - | - | - | - |
| 11 | UE subscribes the conference event | --> | SUBSCRIBE | - | - |
| 12 | SS responds to the subscription | <-- | 200 OK | - | - |
| 13 | SS sends the initial state of the conference event to the UE | <-- | NOTIFY | - | - |
| 14 | UE responds to the NOTIFY | --> | 200 OK | - | - |
| 15 | The UE leaves the conference with BYE | --> | BYE | 2 | P |
| 16 | The SS sends 200 OK for BYE | <-- | 200 OK | - | - |
| 17 | If the UE had subscribed to the conference event package, the SS notifies the UE that its subscription to conference event package is terminated | --> | NOTIFY | - | - |
| 18 | The UE sends 200 OK for NOTIFY (if sent by SS) | --> | 200 OK | - | - |

8.31.3.3	Specific message contents

Table 8.31.3.3-1: INVITE (step 1, table 8.31.3.2-1)

| Derivation Path: TS 34.229-1 [2], Step 2 of C.21, Condition A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |  |  |
| To addr-spec |  | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |  |  |

Table 8.31.3.3-2: 183 Session in Progress for INVITE (step 4, table 8.31.3.2-1)

| Derivation Path: TS 34.229-1 [2], Step 4 of C.21 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact addr-spec feature-param |  | sip:temporary@conf-factory appended with px_IMS_HomeDomainName isfocus |  |  |
| Record-Route rec-route |  | <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |

Table 8.31.3.3-3: 200 OK for INVITE (step 9, table 8.31.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A3.1, Condition A22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Record-Route rec-route |  | Same value as in the 183 response |  |  |
| Contact addr-spec feature-param |  | sip:final@conf-factory appended with px_IMS_HomeDomainName isfocus |  |  |

Table 8.31.3.3-4: ACK (step 10, table 8.31.3.2-1)

| Derivation Path: TS 34.229-1 [2], Step 2 of C.21, Condition A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | sip:final@conf-factory appended with px_IMS_HomeDomainName |  |  |

Table 8.31.3.3-5: SUBSCRIBE for conference event package (step 11, table 8.31.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A2.7, A1

Table 8.31.3.3-6: 200 OK for SUBSCRIBE (step 12, table 8.31.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A5.2

Table 8.31.3.3-7: NOTIFY for conference event package (step 13, table 8.31.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A5.3, Condition A3

Table 8.31.3.3-8: 200 OK for other requests than REGISTER or SUBSCRIBE (step 14/16/18, table 8.31.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A3.1, Condition A22

Table 8.31.3.3-9: BYE (step 15, table 8.31.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A2.8, Condition A8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Request-Line Request-URI |  | sip:final@conf-factory appended with px_IMS_HomeDomainName |  |  |

Table 8.31.3.3-10: NOTIFY for conference event package (step 17, table 8.31.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A5.3, Condition A4

<End of modified section 1>