3GPP TSG-RAN5 Meeting #90-e	R5-210672

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2031 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MDT TC 8.1.6.1.4.4-CEF-RRC re-establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In R16, the NR UE also needs to support MDT features of CEF (Connection Establishment Failure) logging and reporting. Thus, new test cases of MDT-CEF logging and reporting need to be introduced in RAN5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new sub clause 8.1.6.1.4.4 to describe the test case of NR MDT-CEF reporting at RRC re-establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of relevent test case for MDT-CEF features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0125 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

##### 8.1.6.1.4.4	Connection Establishment Failure / Logging and reporting / Reporting at RRC connection re-establishment

8.1.6.1.4.4.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available }

ensure that {

when { UE performs an an RRC Connection re-establishment procedure }

then { UE sends an RRCReestablishmentComplete message with connEstFailInfoAvailable }

}

8.1.6.1.4.4	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.7.5. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCReestablishmentComplete message;

…

8.1.6.1.4.4.3	Test description

8.1.6.1.4.4.3.1	Pre-test conditions

System Simulator:

- NR Cell 1 and Cell 2.

- System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

- None.

Preamble:

- The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.1.4.4.3.2	Test procedure sequence

Table 8.1.6.1.4.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.4.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -80 | ”Off” | Only Cell 1 is available. (NOTE 1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | ”Off” | -80 | Only Cell 2 is available. (NOTE 1). |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3. |  |  |  |  |  |

Table 8.1.6.1.4.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 2 | The SS is configured to not transmit contention resolution in step 2 RACH procedure. |  |  |  |  |
| 3 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 4 | The SS waits for 2sec (T300 expiry) and ignores further received Random Access Preamble. | - | - | - | - |
| 5 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 6 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 7 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 8 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 9 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2 and DRB(s). The RRCReconfiguration message is configured using RRCReconfiguration-SRB2-DRB(n, m) where n and m are the number of DRB(s) configured with RLC-AM and RLC-UM respectively. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 11 | The SS changes cell 1 parameters according to the row "T1"in table 8.1.6.1.4.4.3.2-1 | - |  | - | - |
| 12 | The UE transmits an RRCReestablishmentRequest message on Cell 2. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 13 | The SS transmits an RRCReestablishment message on Cell 2. | <-- | NR RRC: RRCReestablishment | - | - |
| 14 | Check: Does the UE send an RRCReestablishmentComplete message with connEstFailInfoAvailable on Cell 2? | --> | NR RRC: RRCReestablishmentComplete | 1 | P |

8.1.6.1.4.4.3.3	Specific message contents

Table 8.1.6.1.4.4.3.3-1: Paging (step 1, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.6.1-9

Table 8.1.6.1.4.4.3.3-2: RRCSetupRequest (step 2, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.6.1-23

Table 8.1.6.1.4.4.3.3-3: RRCSetup (step 5, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.6.1-21

Table 8.1.6.1.4.4.3.3-4: RRCSetupComplete (step 6, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.6.1-22

Table 8.1.6.1.4.4.3.3-5: Service Request (step 6, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.7.1-16

Table 8.1.6.1.4.4.3.3-6: SecurityModeCommand (step 7, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.7.1-25

Table 8.1.6.1.4.4.3.3-6: SecurityModeComplete (step 8, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.7.1-26

Table 8.1.6.1.4.4.3.3-7: RRCReconfiguration (step 9, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.6.1-13, condition NR

Table 8.1.6.1.4.4.3.3-8: SERVICE ACCEPT (step 9, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.7.1-17

Table 8.1.6.1.4.4.3.3-9: RRCReconfigurationComplete (step 10, Table 8.1.6.1.4.4.3.2-2)

Derivation Path: 38.508-1 [4] Table 4.6.1-14

Table 8.1.6.1.4.4.3.3-10: RRCReestablishmentRequest (step 12, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.6.1-12

Table 8.1.6.1.4.4.3.3-11: RRCReestablishment (step 13, Table 8.1.6.1.4.4.3.2-2)

Derivation path: 38.508-1 [4], Table 4.6.1-10

Table 8.1.6.1.4.4.3.3-12: RRCReestablishmentComplete (step 14, Table 8.1.6.1.4.4.3.2-2)

| Derivation Path: 38.508-1 [4] Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FFS | FFS | FFS | FFS |

<End of modified section 1>