3GPP TSG-RAN5 Meeting #90-e	R5-210681

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5006 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE TC 6.1.1.2a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, TC 6.1.1.2a is designed for those band which has only one test frequency. In TC 6.1.1.2a, Cell 1,2,4 and 11 are used. However, according to 36.523-3 table 7.4.3.1-1, Cell 1,2 and 4 all have even SFN offset and Cell 11 has odd SFN offset. As a result, SIB1 of Cell 1,2 and 4 overlap in time and cause mutual interference to each other. Then for those UE which equips only one Rx antenna (i.e. Cat.1bis UE) Es/Iot of these cells may be too low for SIB1 reception. If UE fails to receive SIB1, UE will consider these cells are barred (36.331 cl. 5.2.2.5) and may exclude this cell during cell reselection for up to 300s (36.304 cl. 5.3.1). Then it will fail the test if UE is required to reselect to this cell in test procedure. To avoid this issue, One can observe that TP 1, 2 and 3 of 6.1.1.2(which is reused by 6.1.1.2a) are partially overlapped: (1) with { UE in Automatic network selection mode and EHPLMN, UPLMN and OPLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN } (2) with { UE in Automatic network selection mode and UPLMN, OPLMN and other PLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN } (3) with { UE in Automatic network selection mode and OPLMN and other PLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN } Considering that TP3 already verifies UE prioritizing OPLMN over other PLMN by TP3. Then it is unnecessary to verify UE prioritizing UPLMN over OPLMN and other PLMN in TP2. We only need to verify UE prioritizes UPLMN over OPLMN. For the same reason it is also unnecessary to verify UE prioritizing EHPLMN over UPLMN and OPLMN in TP1. Verifying EHPLMN over UPLMN is already enough. By reducing one cell in each TP we only need to power on two cells at the same time. Then we can choose one cell with odd SFN offset and one cell with even SFN offset to avoid SIB overlapping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, TPs are updated. To be more specific, "OPLMN" is removed from TP1, "other PLMN" is removed from TP2, 2. cell 2 is removed. 3. preamble and power levels are updated. 4. test procedure is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.1.2a	PLMN selection of "Other PLMN/access technology combinations" / Automatic mode / Single Frequency operation

6.1.1.2a.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and EHPLMN and UPLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the EHPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(2)

with { UE in Automatic network selection mode and UPLMN and OPLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the UPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(3)

with { UE in Automatic network selection mode and OPLMN and other PLMN/access technology combinations cells available and UE is fitted with a USIM with Access Technology data files for each PLMN }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of the OPLMN and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

(4)

with { UE in Automatic network selection mode and other PLMN/access technology combinations cells not belonging to any of EHPLMN, UPLMN or OPLMN available }

ensure that {

when { UE is switched on or return to coverage }

then { UE selects a cell of other PLMN/access technology combinations and UE attempts a location registration on the selected cell and when successfully registered indicates the selected PLMN to the user. }

}

6.1.1.2a.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.2.2

6.1.1.2a.3	Test description

6.1.1.2a.3.1	Pre-test conditions

System Simulator:

-	Three intra-frequency multi-PLMN cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.2a.3.1-1.

Table 6.1.1.2a.3.1–1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 11 | PLMN2 |
| 4 | PLMN3 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.2a.3.1-2.

Table 6.1.1.2a.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEHPLMN | 1 | PLMN1 Remaining mandatory entries use default values |  |
| EFPLMNwAcT | 1 | PLMN2 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 | PLMN3 Remaining defined entries use default values | E-UTRAN |
| EFUST |  | Services 20, 42 and 71 are supported. |  |

Preamble:

-	The UE is in state Switched OFF (State 1).

6.1.1.2a.3.2	Test procedure sequence

Table 6.1.1.2a.3.2 – 1 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” “T3” “T4”etc are applied at the points indicated in the Main behaviour description in Table 6.1.1.2a.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 Table 6.2.2.1-1.

Table 6.1.1.2a.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 |  | Cell 4 | Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -79 |  | ”Off“ | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | “Off” |  | -85 | -79 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T3 | RS EPRE | dBm/15kHz | “Off” |  | -79 | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T4 | RS EPRE | dBm/15kHz | “Off” |  | ”Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T5 | RS EPRE | dBm/15kHz | -79 |  | "Off" | "Off" | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.2a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 1 | P |
| 4-18 | Steps 3 to 17 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 19 | Check: Is PLMN1 indicated by the UE? | - | - | 1 | P |
| 20 | SS adjusts cell levels according to row T2 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 21 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 2 | P |
| 22-26 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 27 | Void | - | - | - | - |
| - | EXCEPTION: Step 27Aa1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 27Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 27 | Check: Is PLMN2 indicated by the UE? | - | - | 2 | P |
| 27A | SS adjusts cell levels according to row T5 of table 6.1.1.2a.3.2-1 and change PLMN identifiers of Cell 11 to PLMN4 | - | - | - | - |
| 27B | The generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. |  |  |  |  |
| 28 | SS adjusts cell levels according to row T3 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 29 | Check: Does the UE send an RRCConnectionRequest on Cell 4? | --> | RRCConnectionRequest | 3 | P |
| 30-34 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 4. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 35 | Check: Is PLMN3 indicated by the UE? | - | - | 3 | P |
| 36 | SS adjusts cell levels according to row T4 of table 6.1.1.2a.3.2-1 | - | - | - | - |
| 37 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 4 | P |
| 38-42 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 11. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 43 | Check: Is PLMN4 indicated by the UE? | - | - | 4 | P |

6.1.1.2a.3.3	Specific message contents

None

<End of modified section 1>