3GPP TSG-RAN5 Meeting #90-e	R5-210682

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5007 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE TC 6.1.1.6a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, TC 6.1.1.6a is designed for those band which has only one test frequency. In TC 6.1.1.6a, Cell 1,2,4 and 11 are used. However, according to 36.523-3 table 7.4.3.1-1, Cell 1,2 and 4 all have even SFN offset and Cell 11 has odd SFN offset. As a result, SIB1 of Cell 1,2 and 4 overlap in time and cause mutual interference to each other. Then for those UE which equips only one Rx antenna (i.e. Cat.1bis UE) Es/Iot of these cells may be too low for SIB1 reception. If UE fails to receive SIB1, UE will consider these cells are barred (36.331 cl. 5.2.2.5) and may exclude this cell during cell reselection for up to 300s (36.304 cl. 5.3.1). Then it will fail the test if UE is required to reselect to this cell in test procedure. Cosidering this issue only exists for TDD, and all TDD band has at least two test frequencies. So we suggest add a new two carrier version TC for TDD then 6.1.1.6a is dedicated for FDD. Addition of 6.1.1.6b is addressed in CR R5-210683 2. According to 38.508-1 cell 2 and cell 11 can't be powered on at the same time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Cell 4 and Cell 11 are swapped. 2. PLMN3 and PLMN4 are swapped. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-1 CR 5008 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.1.6a	PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Single Frequency operation

6.1.1.6a.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.1.6.1

6.1.1.6a.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.6.2

6.1.1.6a.3	Test description

6.1.1.6a.3.1	Pre-test conditions

System Simulator:

-	Four intra-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2.

System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.1.6a.3.1-1.

Table 6.1.1.6a.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 2 | PLMN2 |
| 4 | PLMN3 |
| 11 | PLMN4 |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 6.1.1.6a.3.1-2.

Table 6.1.1.6a.3.1–2: USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEPSLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN3. |  |
| EFPLMNwAcT | 1 | PLMN1 Remaining mandatory entries use default values | E-UTRAN |
| EFOPLMNwACT | 1 2 | PLMN2 PLMN3 Remaining defined entries use default values | E-UTRAN E-UTRAN |
| EFHPLMNwAcT | 1 | PLMN4 | E-UTRAN |
| EFUST |  | Service 71 is not supported |  |

Preamble:

-	The UE is in state Registered, Idle Mode (State 2) on Cell 1 according to [18].

6.1.1.6a.3.2	Test procedure sequence

Table 6.1.1.6a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 4 | Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | -85 | “Off” | (NOTE 1) |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | -85 | "Off" | "Off" | Only Cell 2 is available. (NOTE 1) |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -79 | -85 | -85 | "Off" | (NOTE 1). |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -85 | "Off" | -85 | -79 | (NOTE 1) |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |  |

Table 6.1.1.6a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level setting according to the row “T1” in table 6.1.1.6a.3.2-1 | - | - | - | - |
| 2 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 3 | Check: Does the UE send an RRCConnectionRequest on Cell 2? | --> | RRCConnectionRequest | 1 | P |
| 4-8 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 2. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 9 | Check: Is PLMN 2 indicated by the UE? | - | - | 1 | P |
| 10 | The SS changes the power level setting according to the row “T2” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 11 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 12 | Check: Does the UE send an RRCConnectionRequest on Cell 2 within 90 s? | --> | RRCConnectionRequest | 2 | F |
| 13 | Check: Is PLMN 2 indicated by the UE? | - | - | 2 | P |
| 14 | The SS changes the power level setting according to the row “T3” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 15 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 16 | Check: Does the UE send an RRCConnectionRequest on Cell 1? | --> | RRCConnectionRequest | 3 | P |
| 17-21 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 1. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 22 | Check: Is PLMN 1 indicated by the UE? | - | - | 3 | P |
| 23 | The SS changes the power level setting according to the row “T4” in table 6.1.1.6a.3.2-1. | - | - | - | - |
| 24 | Cause the UE in Automatic network selection mode to initiate user reselection and registration onto an available PLMN. (see Note) | - | - | - | - |
| 25 | Check: Does the UE send an RRCConnectionRequest on Cell 11? | --> | RRCConnectionRequest | 4 | P |
| 26-30 | Steps 2 to 6 of the generic test procedure in TS 36.508 subclause 6.4.2.7 are performed on Cell 4. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |
| 31 | Void | - | - | - | - |
| - | EXCEPTION: Step 32a1 describes optional behaviour that depends on the UE capability | - | - | - | - |
| 32a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 33 | Check: Is PLMN 4 indicated by the UE? | - | - | 4 | P |
| Note: MMI command “TRIGGER_USER_RESELECTION” to be used. AT command AT+COPS is not suitable to achieve the test purpose.. |  |  |  |  |  |

6.1.1.6a.3.3	Specific message contents

Table 6.1.1.6a.3.3-1: SystemInformationBlockType3 for cells 1 and 2 (preamble and all steps, table 6.1.1.6a.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst |  | dB24 |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>