3GPP TSG-RAN5 Meeting #90-e	R5-210683

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5008 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE TC 6.1.1.6b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, TC 6.1.1.6a is designed for those band which has only one test frequency. In TC 6.1.1.6a, Cell 1,2,4 and 11 are used. However, according to 36.523-3 table 7.4.3.1-1, Cell 1,2 and 4 all have even SFN offset and Cell 11 has odd SFN offset. As a result, SIB1 of Cell 1,2 and 4 overlap in time and cause mutual interference to each other. Then for those UE which equips only one Rx antenna (i.e. Cat.1bis UE) Es/Iot of these cells may be too low for SIB1 reception. If UE fails to receive SIB1, UE will consider these cells are barred (36.331 cl. 5.2.2.5) and may exclude this cell during cell reselection for up to 300s (36.304 cl. 5.3.1). Then it will fail the test if UE is required to reselect to this cell in test procedure. Cosidering this issue only exists for TDD, and all TDD band has at least two test frequencies. So we suggest add a new two carriers version TC for TDD |  |  |  |  |  |  |  |  |
| Summary of change: |  | LTE TC 6.1.1.6b PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Two Frequencies operation is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normal UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.6b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1336 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1.1.6b	PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Two Frequencies operation

6.1.1.6b.1	Test Purpose (TP)

Same Test Purpose as in clause 6.1.1.6a.1.

6.1.1.6b.2	Conformance requirements

Same Conformance requirements as in clause 6.1.1.6a.2.

6.1.1.6b.3	Test description

6.1.1.6b.3.1	Pre-test conditions

System Simulator:

Same with Pre-test conditions for system simulator in 6.1.1.6a.2 except:

-	Four inter-frequency multi-PLMN cells as specified in TS 36.508 clause 4.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 4.4.2-2.

-	Cell 2 is replaced by Cell 3,

-	Cell 4 is replaced by Cell 11,

-	Cell 11 is replaced by Cell 12.

UE:

Same with Pre-test conditions for UE in 6.1.1.6a.2.

Preamble:

Same with Pre-test conditions for Preamble in 6.1.1.6a.2.

6.1.1.6b.3.2	Test procedure sequence

Same with Test procedure in 6.1.1.6a.3.2 except:

-	Cell 2 is replaced by Cell 3,

-	Cell 4 is replaced by Cell 11,

-	Cell 11 is replaced by Cell 12.

6.1.1.6b.3.3	Specific message contents

Same with Specific message contents in 6.1.1.6a.3.3 except:

-	Cell 2 is replaced by Cell 3.

<End of modified section 1>