3GPP TSG-RAN5 Meeting #90-e	R5-210768

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1743 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PDSCH-TimeDomainResourceAllocationList to consider coreset0 for Demod FR2 test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Start symbol and length of PDSCH-TimeDomainResourceAllocationList defined in section 5.4.2.0 for FR2 Demod test cases are not compatible with CORESET#0 index defined on test frequency tables because it uses 2 symbols of PDCCH (symbol 0 and 1). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new PDSCH-TimeDomainResourceAllocation entry compatible with CORESET#0 to be used for FR2 Demod during NR attach |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR attach will not be achieved because symbol 1 will be used for both PDCCH and PDSCH. Rate matching is not allowed for RA-RNTI according to 38.214, because the PDSCH scheduled with RA-RNTI or TC-RNTI will rate matching only against broadcast but not against its PDCCH |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 5.4.2	Radio resource control information elements for Demodulation Performance and CSI reporting tests

## <<< Skip unchanged sections >>>

#### 5.4.2.0	Parameters common to all Demod and CSI tests

## <<< Skip unchanged sections >>>

###### PDSCH-TimeDomainResourceAllocationList

Table 5.4.2.0-27: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 3 entries |  | DEMOD_FR2 |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start Symbol(S)=2. Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 44 | Start symbol(S)=2, Length(L)=4 | For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |
|  | 96 | Start symbol(S)=1, Length(L)=9 | DEMOD_FR2 |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 | DEMOD_FR2 |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength |  |  |  |
|  | 40 | Start symbol(S)=1, Length(L)=13 | DEMOD_FR2 |
| } |  |  |  |
| } |  |  |  |

## <<< END OF CHANGES >>>