3GPP TSG-RAN5 Meeting #90-e	R5-210772

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1745 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment xOverhead setting with PDSCH RMCs for Demod FR2 testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PDSCH Reference Channel for FR2 testing defined in TS 38.521-4, i.e. table A.3.2.2.5-1, consider Overhead for TBS determination with value 6 however, xOverhead IE in PDSCH-ServingCellConfig is Not Present. Hence, there is a misalignment around xOverhead IE between TS 38.521-4 and TS 38.508-1 for Demod FR2 test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated xOverhead IE accordin with TS 38.521-4 table A.3.2.2.5-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDSCH-ServingCellConfig will not have correct configuration for Demod FR2 test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### PDSCH Configuration

###### PDSCH-ServingCellConfig

Table 5.4.2.0-25: PDSCH-ServingCellConfig

| Derivation Path: Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| codeBlockGroupTransmission | Not present |  |  |
| xOverhead | Not present |  |  |
| xOverhead | xOh6 |  | DEMOD_FR2 |
| nrofHARQ-ProcessesForPDSCH | Set according to the test id | Typically n4 for FDD, n8 for TDD |  |
| pucch-Cell | Not present |  |  |
| } |  |  |  |

## <<< END OF CHANGES >>>