3GPP TSG-RAN5 Meeting #90-e	R5-210775
Electronic Meeting, 22 February – 5 March 2021

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On Testability Aspects for FR2 Demodulation Testing

Document for:	Discussion and Endorsement

# Introduction

This contribution provides an analysis of different factors affecting on maximum testable SNR for Demod FR2 test cases.

# Discussion

In [1] and [2], it was discussed a new factor to be considered in SNR dynamic range analysis to conclude with maximum testable SNR consider FR2 testing.

In RAN5#89e  following statements were agreed:

- Use 10dB for fading crest factor as proposed in [1], as worst case value till more optimized value would be agreed

- Define an action point for defining an acceptable fading crest factor margin for FR2 Demod performance and CSI test cases allowing clipping due to fading or not:

| - Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#89e.23 | RF | RF Provide input on acceptable clipping frequency due to fading and/or acceptable fading crest factor margin for FR2 Demod performance and CSI test cases | Qualcomm | R5-206168, R5-205702 RAN5#90e | RAN5#90e | Open |

In regards to a more optimize fading crest factor margin, a calculation has been done considering different instances of fading, depending on fading profile and DL signal.

The following table shows the interval of crest factor values calculated for several realizations of each profile and its average value:

Table 1 Crest factor gain (in dB) for Demod FR2 fading profiles

| Profile | Min crest factor (dB) | Max crest factor (dB) | Average crest factor (dB) |
| --- | --- | --- | --- |
| TDLA30 | 7.8 | 9.4 | 8.7 |
| TDLC60 | 6.6 | 8.3 | 7.3 |

Hence, based on above calculation, fading crest factor can be optimized to 8.7dB.

Observation 1: Based on empirical calculations of different fading instances, an average crest factor can be set as 8.7dB instead of 10dB

Proposal 1: Use 8.7dB as fading crest factor to be considered in when determining maximum testable SNR

Proposal 2: Consider a backoff of 21.7 dB (13 dB PAPR + 8.7 dB average crest factor) when determining maximum testable SNR

# Conclusion

The following observations and proposals were made in this contribution:

Observation 1: Based on empirical calculations of different fading instances, an average crest factor can be set as 8.7dB instead of 10dB

Proposal 1: Use 8.7dB as fading crest factor to be considered in when determining maximum testable SNR

Proposal 2: Consider a backoff of 21.7 dB (13 dB PAPR + 8.7 dB average crest factor) when determining maximum testable SNR

# References

- R5-205702, On Fading Impact on CREST and Demod Testability Analysis, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #89-e, November 2020

- R5-206614, On the achievable SNR for FR2 demod and perf test cases, Anritsu, 3GPP TSG RAN WG 5 Meeting #89-e, November 2020