3GPP TSG-RAN5 Meeting #90-e	R5-210812

# Electronic Meeting, 22nd Feb 2021 – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1747 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE ServingCellConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To align the IE ServingCellConfig with core specification updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IE ServingCellConfig has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be fully applicable and aligned with the core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	ServingCellConfig

Table 4.6.3-167: ServingCellConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
|  | Not present |  | MEAS, RESUME |
| downlinkBWP-ToReleaseList | Not present |  |  |
| downlinkBWP-ToAddModList | Not present |  |  |
| firstActiveDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS |
| bwp-InactivityTimer | Not present |  |  |
| defaultDownlinkBWP-Id | BWP-Id |  |  |
|  | Not present |  | MEAS, RESUME |
| uplinkConfig | Not present |  | MEAS, No_UL |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition SUL_NUL |  | PUSCH_PUCCH_ON_SUL |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig | Not present |  | RESUME |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| powerBoostPi2BPSK | Not present |  |  |
| uplinkChannelBW-PerSCS-List | Not present |  |  |
| enablePL-RS-UpdateForPUSCH-SRS-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUSCH0-0-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUCCH-r16 | Not present |  |  |
| enableDefaultBeamPL-ForSRS-r16 | Not present |  |  |
| uplinkTxSwitching-r16 | Not present |  |  |
| mpr-PowerBoost-FR2-r16 | Not present |  |  |
| } |  |  |  |
| supplementaryUplink | Not present |  |  |
| supplementaryUplink SEQUENCE { |  |  | PUSCH_PUCCH_ON_SUL |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
|  | BWP-UplinkDedicated with condition RESUME |  | RESUME |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| carrierSwitching | Not present |  |  |
| powerBoostPi2BPSK | Not present |  |  |
| uplinkChannelBW-PerSCS-List | Not present |  |  |
| enablePL-RS-UpdateForPUSCH-SRS-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUSCH0-0-r16 | Not present |  |  |
| enableDefaultBeamPL-ForPUCCH-r16 | Not present |  |  |
| enableDefaultBeamPL-ForSRS-r16 | Not present |  |  |
| uplinkTxSwitching-r16 | Not present |  |  |
| mpr-PowerBoost-FR2-r16 | Not present |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDCCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  | MEAS, RESUME |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig | Not present |  | MEAS, RESUME |
| csi-MeasConfig | Not present |  |  |
| sCellDeactivationTimer | Not present |  |  |
| crossCarrierSchedulingConfig | Not present |  |  |
| tag-Id | 0 |  |  |
| dummy1 | Not present |  |  |
| pathlossReferenceLinking | Not present |  |  |
| servingCellMO | Not present |  |  |
|  | MeasObjectId |  | MEAS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PUSCH_PUCCH_ON_SUL | For the purpose of SUL test under condition that supplementary uplink is configured with both PUSCH and PUCCH on SUL carrier. |
| MEAS | A NR or EN-DC measurement is configured. |
| No_UL | No uplink CA |
| RESUME | Used in RRCResume Message |

<End of modified section>