3GPP TSG-RAN5 Meeting #90-e	R5-210838

# Online, , 22th Feb 2020 - 5th Mar 2020

Title:						On MU for FR2 Blocker Test using offset antenna

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

In RAN5#89-e, it was endorsed in [2] to introduce offset antenna for blocker signal for FR2 blocking test cases, and in that case apply additional MU as analyzed in [2]. After talking among come companies for double check, it is realized that the blocker frequency assumption used in [2] was beyond the actual blocker frequency used in the test case. +/- 800Mhz of the n258 band edge was assumed as the blocker frequency in [2] but in the actual test case blocker signal is put withing the band (Table 7.6.2.5-1 of TS 38.521-2). In this paper, MU analysis for the FR2 blocking test case using offset antenna with the correct blocker frequency assumption is provided. Note that the MU is now smaller than in [2] using the correct assumption of blocker frequencies due to less gain difference of desired and blocker signal.

Table 1 Frequency assumption used in simulation

|  | n258 mid (Wanted) | n258 low (Blocker low) | n258 high (Blocker high) |
| --- | --- | --- | --- |
| In [2] | 25875 | 23450 | 28300 |
| This paper | 25875 | 24250 | 27500 |

# 2.	MU evaluation result

We carried out a simulation analysis for residual error after compensation( DDEIS in Fig1. in [2] ). The same simulation assumption as in [2] other than the blocker frequency assumption in Table 1.

Figure 1 2x8 beam pattern

Table 1 shows the mean error and std.dev for the 2x8 and 1x4 assumption and for various offset angles(0..7). Here we take max(abs(low), abs(high)) in mean error and std.dev(i.e. worst case).

Table 1 Residual error (n258 in-band blocking assumption)

| offset angle[deg] | Mean error[dB] | Std.dev[dB] | Mean error[dB] | Std.dev[dB] |
| --- | --- | --- | --- | --- |
|  | 2x8 Assumption |  | 1x4 Assumption |  |
| 0 | 0.000 | 0.000 | 0.000 | 0.000 |
| 0.5 | 0.001 | 0.001 | 0.000 | 0.000 |
| 1 | 0.004 | 0.003 | 0.001 | 0.001 |
| 1.5 | 0.010 | 0.006 | 0.002 | 0.002 |
| 2 | 0.018 | 0.011 | 0.004 | 0.003 |
| 2.5 | 0.028 | 0.018 | 0.006 | 0.004 |
| 3 | 0.041 | 0.026 | 0.009 | 0.006 |
| 3.5 | 0.057 | 0.036 | 0.013 | 0.009 |
| 4 | 0.075 | 0.048 | 0.016 | 0.011 |
| 4.5 | 0.096 | 0.062 | 0.021 | 0.015 |
| 5 | 0.120 | 0.078 | 0.026 | 0.018 |
| 5.5 | 0.147 | 0.096 | 0.031 | 0.022 |
| 6 | 0.178 | 0.117 | 0.038 | 0.026 |
| 6.5 | 0.213 | 0.142 | 0.044 | 0.031 |
| 7 | 0.252 | 0.170 | 0.051 | 0.036 |

Proposal 1 : Introduce new MU terms regarding impact from offset antenna for FR2 blocker test. Put the Table 1(2x8) into TR 38.903 as a look up table for the MU values due to offset antenna. For the MTSU derivation purpose it is set to 0.0dB.

- Conclusion

Proposal 1 : Introduce new MU terms regarding impact from offset antenna for FR2 blocker test. Put the Table 1(2x8) into TR 38.903 as a look up table for the MU values due to offset antenna. For the MTSU derivation purpose it is set to 0.0dB.

# 4.	References

[1] R5-204084, “On FR2 ACS and IBB Test Implementation and MU”, Anritsu, RAN5#88e

[2] R5-216615, “On FR2 Blocker Test with Offset Antenna”, Anritsu, RAN5#89e