3GPP TSG-RAN5 Meeting #90-e	R5-210873

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1752 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of aperiodic CSI-RS reference configuration for RRM tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - To align with the changes made in CR R4-2101612 and R4-2103492 agreed in RAN4 #98e. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - default configuration for aperiodic CSI-RS configuration and reporting are updated to align with CR R4-2101612 and R4-2103492. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specs are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table 7.3.1-11B: NZP-CSI-RS-ResourceSet for BM

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId-BM |  |  |
|  | NZP-CSI-RS-ResourceSetId-BM with condition APERIODIC |  | APERIODIC |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for BM (0) | entry 1 |  |
|  | NZP-CSI-RS-ResourceId for BM (0) with condition APERIODIC |  | APERIODIC |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for BM (1) | entry 2 |  |
|  | NZP-CSI-RS-ResourceId for BM (1) with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| aperiodicTriggeringOffset | 4 |  | APERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For apeiodic CSI-RS resources |

<End of modified section 1>

<Start of modified section 2>

Table 7.3.1-12G: CSI-ReportConfig for BM

| Derivation Path: Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigId | CSI-ReportConfigId-BM |  |  |
|  | CSI-ReportConfigId-BM with condition APERIODIC |  | APERIODIC |
| resourcesForChannelMeasurement | CSI-ResourceConfigId-BM |  |  |
|  | CSI-ResourceConfigId-BM with condition APERIODIC |  | APERIODIC |
| reportConfigType CHOICE { |  |  |  |
| periodic | Set according to parameters given in test cases |  |  |
| aperiodic SEQUENCE { |  |  | APERIODIC |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF { | 1 entry |  |  |
| INTEGER[1] | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-RSRP | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| APERIODIC | For aperiodic CSI-RS resources |

<End of modified section 2>

<Start of modified section 3>

–	PUSCH-TimeDomainResourceAllocationList

Table 7.3.1-22: PUSCH-TimeDomainResourceAllocationList-BM

| Derivation Path: Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF SEQUENCE { | 1 entriy | same number of entries as reportSlotOffsetList in Table 7.3.1-12G |  |
| k2[1] | 4 | Same with k2 configrued in reportSlotOffsetList in Table 7.3.1-12G |  |
| mappingType[1] | typeA |  |  |
| startSymbolAndLength[1] | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |

<End of modified section 3>