3GPP TSG-RAN5 Meeting #90-e	R5-210874

# Electronic Meeting, 22th Feb – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5268 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 6.2.3A.1_3 Maximum Power Reduction for CA and HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_TDD_HPUE_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are still brackets in the test requirement which can be removed. The minimum requirement for MPR of power class 2 UE is applicable for CA bandwidth Class C. However, the combined bandwidth of some test points do not meet the condition of Class C . The test points for 64 QAM are still missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the brackets and updated Test Tolerance Added the test points for 64 QAM and removed test points that do not meet CA Class C requirement Removed the editor’s note |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test requirement is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3A.1_3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

6.2.3A.1_3	Maximum Power Reduction (MPR) for CA and HPUE (intra-band contiguous DL CA and UL CA)

6.2.3A.1_3.1	Test purpose

The number of RB identified in Table 6.2.3A.1_3.3-1 is based on meeting the requirements for Adjacent Channel Leakage power Ratio (ACLR) for CA and the maximum power reduction (MPR) for intra-band contiguous CA Bandwidth Class C due to Cubic Metric (CM).

6.2.3A.1_3.2	Test applicability

The requirements of this test apply in test case 6.6.2.3A.1_3 Adjacent Channel Leakage power Ratio (ACLR) for CA and HPUE to all types of E-UTRA Power Class 2 UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

NOTE:	As a result TC 6.2.3A.1_3 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.3A.1_3.3	Minimum conformance requirements

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A.0-2 due to higher order modulation and contiguously allocated transmissions (resource blocks) is specified in Table 6.2.3A.1_3.3-1. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2.3A.1_3.3-1: Maximum Power Reduction (MPR) for Power Class 2

| Modulation | CA bandwidth Class C / Smallest Component Carrier Transmission Bandwidth Configuration |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 25 RB | 50 RB | 75 RB | 100 RB |  |
| QPSK | > 6 and ≤ 25 | > 6 and ≤ 50 | > 6 and ≤ 75 | > 6 and ≤ 100 | ≤ 1 |
| QPSK | > 25 | > 50 | > 75 | > 100 | ≤ 2 |
| 16 QAM | ≤ 6 | ≤ 8 | ≤ 16 | ≤ 18 | ≤ 1.5 |
| 16 QAM | > 6 and ≤ 25 | > 8 and ≤ 50 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 25 | > 50 | > 75 | > 100 | ≤ 3 |
| 64 QAM | ≤ 8 and allocation wholly contained within a single CC | ≤ 12 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 18 and allocation wholly contained within a single CC | ≤ 2 |
| 64 QAM | > 8 or allocation extends across two CC's | > 12 or allocation extends across two CC's | > 16 or allocation extends across two CC's | > 18 or allocation extends across two CC's | ≤ 3 |
| 256 QAM | FFS |  |  |  | FFS |

For PUCCH and SRS transmission, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For UE power class 2 intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A.0-2 is specified as follows

MPR = CEIL { min(MA, MIM5), 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

MA = 	8.2				; 0 ≤ A < 0.04

9.2 - 40A			; 0.04 ≤ A < 0.075

8 – 16A			; 0.075 ≤ A < 0.25

4.83 – 3.33A			; 0.25 ≤ A ≤ 0.4,

3.83 – 0.83A			; 0.4 ≤ A ≤ 1,

Where MA is defined FFS for 256 QAM,

and MIM5 is defined as follows

MIM5 =	5.0		; IM5 < 1.5 * BWChannel_CA

6.0		; 1.5 * BWChannel_CA ≤ IM5 <  BWChannel_CA/2 + FOOB

MA		; IM5 ≥ BWChannel_CA/2 + FOOB

For intra-band carrier aggregation, the MPR is evaluated per slot and given by the maximum value taken over the transmission(s) on all component carriers within the slot, the maximum MPR over the two slots is then applied for the entire subframe.

For the UE maximum output power modified by MPR, the power limits specified in sub-clause 6.2.5A.1_3 apply.

The normative reference for this requirement is in TS 36.101 [2] clause 6.2.3A.

6.2.3A.1_3.4	Test description

Use 6.2.3A.1.4.1 except replace Table 6.2.3A.1.4.1-1 with Table 6.2.3A.1_3.4.1-1.

Table 6.2.3A.1_3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |
| 2 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |
| 3 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 |  |  |
| 4 | 100 | 25 |  | 16QAM | 6 | P_6@0 | S_0@0 |  |  |
| 5 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |
| 6 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 |  |  |
| 7 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |
| 8 | 75 | 75 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |
| 9 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 |  |  |
| 10 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 |  |  |
| 11 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 |  |  |
| 12 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 |  |  |
| 13 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 |  |  |
| 14 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |
| 15 | 100 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |
| 16 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 |  |  |
| 17 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 |  |  |
| 18 | 100 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 |  |  |
| 19 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 |  |  |
| 20 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 |  |  |
| 21 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| 22 | 100 | 75 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |
| 23 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 |  |  |
| 24 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 |  |  |
| 25 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 |  |  |
| 26 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 |  |  |
| 27 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 |  |  |
| 28 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |
| 29 | 100 | 100 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |
| 30 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 |  |  |
| 31 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |  |  |
| 32 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 |  |  |
| 33 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 |  |  |
| 34 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 |  |  |
| 35 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 |  |  |
| 36 | 75 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 |  |  |
| 37 | 75 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |
| 38 | 75 | 50 |  | QPSK | 125 | P_75@0 | S_50@0 |  |  |
| 39 | 75 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 |  |  |
| 40 | 75 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |
| 41 | 75 | 50 |  | 16QAM | 125 | P_75@0 | S_50@0 |  |  |
| 42 | 75 | 50 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |
| 43 | 100 | 25 |  | 64QAM | 8 | P_8@0 | S_0@0 |  |  |
| 44 | 100 | 25 |  | 64QAM | 125 | P_100@0 | S_25@0 |  |  |
| 45 | 75 | 75 |  | 64QAM | 16 | P_16@0 | S_0@0 |  |  |
| 46 | 75 | 75 |  | 64QAM | 150 | P_75@0 | S_75@0 |  |  |
| 47 | 100 | 50 |  | 64QAM | 12 | P_12@0 | S_0@0 |  |  |
| 48 | 100 | 50 |  | 64QAM | 150 | P_100@0 | S_50@0 |  |  |
| 49 | 100 | 75 |  | 64QAM | 16 | P_16@0 | S_0@0 |  |  |
| 50 | 100 | 75 |  | 64QAM | 175 | P_100@0 | S_75@0 |  |  |
| 51 | 100 | 100 |  | 64QAM | 18 | P_18@0 | S_0@0 |  |  |
| 52 | 100 | 100 |  | 64QAM | 200 | P_100@0 | S_100@0 |  |  |
| 53 | 75 | 50 |  | 64QAM | 12 | P_12@0 | S_0@0 |  |  |
| 54 | 75 | 50 |  | 64QAM | 125 | P_75@0 | S_50@0 |  |  |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

6.2.3A.1_3.5	Test Requirements

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.3A.1_3.5-1.

Table 6.2.3A.1_3.5-1: CA UE Power Class test requirements, E UTRA bands

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 26 | +2.7/-2.7 |  |  |
| 2 |  |  | 26 | +2.7/-3.7 |  |  |
| 3 |  |  | 26 | +2.7/-4.7 |  |  |
| 4 |  |  | 26 | +2.7/-4.2 |  |  |
| 5 |  |  | 26 | +2.7/-4.7 |  |  |
| 6 |  |  | 26 | +2.7/-5.7 |  |  |
| 7 |  |  | 26 | +2.7/-14.2 |  |  |
| 8 |  |  | 26 | +2.7/-2.7 |  |  |
| 9 |  |  | 26 | +2.7/-3.7 |  |  |
| 10 |  |  | 26 | +2.7/-4.7 |  |  |
| 11 |  |  | 26 | +2.7/-4.2 |  |  |
| 12 |  |  | 26 | +2.7/-4.7 |  |  |
| 13 |  |  | 26 | +2.7/-5.7 |  |  |
| 14 |  |  | 26 | +2.7/-14.2 |  |  |
| 15 |  |  | 26 | +2.7/-2.7 |  |  |
| 16 |  |  | 26 | +2.7/-3.7 |  |  |
| 17 |  |  | 26 | +2.7/-4.7 |  |  |
| 18 |  |  | 26 | +2.7/-4.2 |  |  |
| 19 |  |  | 26 | +2.7/-4.7 |  |  |
| 20 |  |  | 26 | +2.7/-5.7 |  |  |
| 21 |  |  | 26 | +2.7/-14.2 |  |  |
| 22 |  |  | 26 | +2.7/-2.7 |  |  |
| 23 |  |  | 26 | +2.7/-3.7 |  |  |
| 24 |  |  | 26 | +2.7/-4.7 |  |  |
| 25 |  |  | 26 | +2.7/-4.2 |  |  |
| 26 |  |  | 26 | +2.7/-4.7 |  |  |
| 27 |  |  | 26 | +2.7/-5.7 |  |  |
| 28 |  |  | 26 | +2.7/-14.2 |  |  |
| 29 |  |  | 26 | +2.7/-2.7 |  |  |
| 30 |  |  | 26 | +2.7/-3.7 |  |  |
| 31 |  |  | 26 | +2.7/-4.7 |  |  |
| 32 |  |  | 26 | +2.7/-4.2 |  |  |
| 33 |  |  | 26 | +2.7/-4.7 |  |  |
| 34 |  |  | 26 | +2.7/-5.7 |  |  |
| 35 |  |  | 26 | +2.7/-14.2 |  |  |
| 36 |  |  | 26 | +2.7/-2.7 |  |  |
| 37 |  |  | 26 | +2.7/-3.7 |  |  |
| 38 |  |  | 26 | +2.7/-4.7 |  |  |
| 39 |  |  | 26 | +2.7/-4.2 |  |  |
| 40 |  |  | 26 | +2.7/-4.7 |  |  |
| 41 |  |  | 26 | +2.7/-5.7 |  |  |
| 42 |  |  | 26 | +2.7/-14.2 |  |  |
| 43 |  |  | 26 | +2.7/-4.7 |  |  |
| 44 |  |  | 26 | +2.7/-5.7 |  |  |
| 45 |  |  | 26 | +2.7/-4.7 |  |  |
| 46 |  |  | 26 | +2.7/-5.7 |  |  |
| 47 |  |  | 26 | +2.7/-4.7 |  |  |
| 48 |  |  | 26 | +2.7/-5.7 |  |  |
| 49 |  |  | 26 | +2.7/-4.7 |  |  |
| 50 |  |  | 26 | +2.7/-5.7 |  |  |
| 51 |  |  | 26 | +2.7/-4.7 |  |  |
| 52 |  |  | 26 | +2.7/-5.7 |  |  |
| 53 |  |  | 26 | +2.7/-4.7 |  |  |
| 54 |  |  | 26 | +2.7/-5.7 |  |  |
| NOTE 1: Lower limit is assuming ΔTC,c and ΔTIB,c are zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1.3-1. |  |  |  |  |  |  |

<End of modified section>