3GPP TSG-RAN5 Meeting #90-e	R5-210897

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1753 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test frequency parameters for band n83 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The parameter “OffsetToCarrier” for band n83 should be updated as per table C.3.2-1. The frequency of PointA is updated as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating test frequency parameters for band n83. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies for n83 is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.83 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.1.1	Test frequencies for NR operating bands in FR1

4.3.1.1.1	NR operating bands in FR1

<Unchanged Skipped>

4.3.1.1.1.83	Reference test frequencies for NR operating band n83 (SUL)

Table 4.3.1.1.1.83-1: Test frequencies for NR operating band n83 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |
| 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |
| 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |
| 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 625.24 | 125048 | 504 |
|  |  |  | High | 738 | 147600 | 727.38 | 145476 | 6 |

Table 4.3.1.1.1.83-2: Test frequencies for NR operating band n83 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 539.74 | 107948 | 504 |
|  |  |  | High | 743 | 148600 | 736.52 | 147304 | 6 |
| 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 537.22 | 107444 | 504 |
|  |  |  | High | 740.5 | 148100 | 731.5 | 146300 | 6 |
| 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 534.88 | 106976 | 504 |
|  |  |  | High | 738 | 147600 | 726.66 | 145332 | 6 |

<End of modified section 1>