3GPP TSG-RAN5 Meeting #90-e

# Electronic Meeting, 22th Feb– 5th Mar 2021

Source:        Samsung, Huawei

Title:           TP analysis on FR1 6.5A.3.1.1 Spurious emission for CA (2UL CA)

Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for NR Spurious emission of 2UL inter-band CA case.

Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA is defined as table below.

Table 6.6.3.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for spurious emission for single carrier mode is defined as table below.

Table 6.5.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | OuterFull |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Test Environment

In LTE, Spurious emission for CA is tested under NC only. According to baseline endorsed in [1], it’s proposed to reuse NC for Spurious emission of inter-band UL CA.

Proposal 1: Define test environment as NC for spurious emission in FR1 for UL inter-band CA.

Test Subcarrier Spacing

In [5] the smallest SCS are selected for Single Carrier Spurious emission verification. It is proposed to reuse the same way for inter-band UL CA.

Proposal 2: Select smallest SCS for Spurious emission in FR1 for inter-band UL CA.

Test Frequencies

In LTE, Spurious emission for inter-band CA is tested for both Low range and High range. The same test coverage as for LTE would be enough to ensure that the ACLR obeys the test requirement at low range and high range.

Proposal 3: Test Spurious emission for Low range and High range frequencies in FR1 for inter-band UL CA.

Test Channel Bandwidth

In LTE, Spurious emission for inter-band CA is tested for Lowest and Highest NRB_agg. Hence it proposed to reuse the LTE method for ACLR test for inter-band UL CA.

Proposal 4: Test ACLR for Lowest and Highest NRB_agg in FR1 for UL inter-band CA.

Test Modulations and waveforms

For single carrier Spurious emission TC peresented in [5]  only CP-OFDM with OPSK is selected for testing. For CA case it’s proposed to reuse this configuration.

Proposal 5: Test Spurious emission for CA with CP-OFDM QPSK.

RB allocation

Considering RB allocation defined in LTE CA and NR SC for Spurious emission, the OuterFull, Edge_1RB_Left and Edge_1RB_Right would be enough to ensure NR CA spurious test coverage.

Proposal 6: Test Spurious emission for CA with OuterFull, Edge_1RB_Left and Edge_1RB_Right

Number of test points for CA spurious emission

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 1 | 3 | 12 |

Conclusion

For inter-band UL CA NR ACLR requirement, the test condition is proposed as below:

Proposal 1: Define test environment as NC for spurious emission in FR1 for UL inter-band CA.

Proposal 2: Select smallest SCS for Spurious emission in FR1 for inter-band UL CA.

Proposal 3: Test Spurious emission for Low range and High range frequencies in FR1 for inter-band UL CA.

Proposal 4: Test ACLR for Lowest and Highest NRB_agg in FR1 for UL inter-band CA.

Proposal 5: Test Spurious emission for CA with CP-OFDM QPSK.

Proposal 6: Test Spurious emission for CA with OuterFull, Edge_1RB_Left and Edge_1RB_Right

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-182019, "Discussion of NR FR1 Test Point for TX Spurious Emission test cases "

[4] 3GPP TS 36.521-1 v16.7.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v16.6.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 16).

[6] 3GPP TS 38.101-1 v16.6.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16).