3GPP TSG-RAN5 Meeting #90-e	R5-210918

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1160 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating MOP and MPR for MIMO testing for several NR bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test requirements in clause 6.2D.1 and 6.2D.2 is absent for bands n38, n39 and n40. They need to be updated to include these bands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating tables 6.2D.1.3-1, 6.2D.1.5-1 and 6.2D.2.5-1 for bands n38, n39 and n40. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MOP and MPR for MIMO testing can’t be tested for n38, n39 and n40. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.1, 6.2D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.2D.1	UE maximum output power for UL MIMO

Editor’s Note:

- No test points are defined for 2-layer UL MIMO since there is no configuration satisfying MPR=0dB requirements in RAN4. Testing with 1.5dB MPR as recommended by RAN4 has been covered in 6.2D.2.

- Implementation to verify ‘If UE is configured for transmission on single-antenna port, the requirements in clause 6.2.1 apply’ is still under discussion.

6.2D.1.1	Test purpose

To verify that the error of the UE maximum output power for UL MIMO does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2D.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support UL MIMO.

6.2D.1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1.3-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | +2/-3 |  |  |
| n2 |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  | 23 | +2/-31 |  |  |
| n25 |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  | 23 | +2/-3 |  |  |
| n38 |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  | 23 | +2/-3 |  |  |
| n40 |  |  |  |  | 23 | +2/-3 |  |  |
| n41 |  |  | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  | 23 | +2/-3 |  |  |
| n70 |  |  |  |  | 23 | +2/-3 |  |  |
| n71 |  |  |  |  | 23 | +2/-3 |  |  |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: 1 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Note 2: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

Table 6.2D.1.3-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. |  |  |  |

For UEs supporting uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power requirements specified in Table 6.2D.1.3-1 shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

Table 6.2D.1.3-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [9]. NOTE 3: For PUSCH configured with ULFPTx Mode set to Mode-1, all the transmitter requirement for CP-OFDM based modulation is not needed to be verified if the requirement for UL MIMO has been validated. |  |  |  |  |  |  |

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2.1 apply for the power class as indicated by the ue-PowerClass field in capability signaling.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.1.

6.2D.1.4	Test description

6.2D.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2D.1.4.1-1 and Table 6.2D.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1.4.1-1: Test Configuration Table for 2-layer UL MIMO

NOTE: 	 No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

Table 6.2D.1.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.1.4.1-1 and Table 6.2D.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.1.4.3.

6.2D.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

4.	If UE supports ULFPTx, repeat test steps 1~3 with UL RMC according to Table 6.2D.1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability.

6.2D.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO.

6.2D.1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1.5-1.

Table 6.2D.1.5-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n2 |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n3 |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n7 |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n25 |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n30 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n34 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n38 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n39 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n40 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n66 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n70 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n71 |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: 1 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.1.5-2 |  |  |  |  |  |  |  |  |

Table 6.2D.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.0.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4] for the applicable operating bands.

<End of modified section 1>

<Start of modified section 2>

6.2D.2	UE maximum output power reduction for UL MIMO

6.2D.2.1	Test purpose

To verify that the power reduction of UE due to higher order modulations and transmit bandwidth configuration does not exceed the specified maximum power reduction.

6.2D.2.2	Test applicability

The requirements of this test apply in test cases 6.5.2.4 Adjacent Channel Leakage power Ratio for UL MIMO to all types of NR UE release 15 and forward that support UL MIMO.

6.2D.2.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.3-1 is specified in Table 6.2.2.3-1. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.2.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.2.

6.2D.2.4	Test description

6.2D.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2D.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.2.4.1-1: Test Configuration Table for Power Class 3

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

Table 6.2D.2.4.1-2: Test Configuration Table for Power Class 2

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest and Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 4 | Default |  | CP-OFDM QPSK | Outer Full |
| 5 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 6 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 7 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 8 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 9 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 10 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 11 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 12 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 13 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 14 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: CP-OFDM 256 QAM test applies only for UEs which supports 256QAM in FR1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2D.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.2.4.3.

6.2D.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbol with transient periods are not under test.

6.2D.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

6.2D.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.2.5-1 and Table 6.2D.2.5-2.

Table 6.2D.2.5-1: UE Power Class test requirements (for Band n1, n2, n3, n7, n25, n30, n38, n39, n40, n41, n48, n66, n70, n71, n77, n78, n79) for Power Class 3

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 2 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 3 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 4 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 5 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 5.0 | （5.02） | 3 | 25.0 + TT | 16.0 - TT | （14.5 - TT2） |
| 6 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 7 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 8 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 6.0 | （5.02） | 3 | 25.0 + TT | 14.0 - TT | （13.5 - TT2） |
| 9 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 10 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 11 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 25.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 12 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 13 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| 14 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 | （6.02） | 3 | 25.0 + TT | 11.5 - TT | （9.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n25, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2: UE Power Class test requirements (for Bands n41, n77, n78, n79) for Power Class 2

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 2.0 | （2.52） | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |  |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 4 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |  |
| 5 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |  |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 8 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |  |
| 9 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |  |
| 12 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |  |
| 13 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |  |
| 14 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-3: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

<End of modified section 2>