3GPP TSG-RAN5 Meeting #90-e	R5-210928

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1756 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Rel-17 CBW 30MHz test frequencies for n83 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 30MHz channel bandwidth are introduced for band n83 in 38.101-1-h00. Test frequencies for 30MHz for n83 need to be added into TS 38.508-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test frequencies for 30MHz in table 4.3.1.1.1.83-1 and 4.3.1.1.1.83-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 30MHz can’t be tested for band n83. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.83 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.1.1	Test frequencies for NR operating bands in FR1

4.3.1.1.1	NR operating bands in FR1

<Unchanged Skipped>

4.3.1.1.1.83	Reference test frequencies for NR operating band n83 (SUL)

Table 4.3.1.1.1.83-1: Test frequencies for NR operating band n83 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 327.61 | 65522 | 2198 |
|  |  |  | High | 745.5 | 149100 | 743.07 | 148614 | 1 |
| 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 325.18 | 65036 | 2198 |
|  |  |  | High | 743 | 148600 | 738.14 | 147628 | 1 |
| 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 322.75 | 64550 | 2198 |
|  |  |  | High | 740.5 | 148100 | 733.21 | 146642 | 1 |
| 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 320.32 | 64064 | 2198 |
|  |  |  | High | 738 | 147600 | 728.28 | 145656 | 1 |
| 30 | 160 | Uplink | Low | 718 | 143600 | 703.6 | 140720 | 0 |
| (Note 1) |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |
| Note 1: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1 |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.83-2: Test frequencies for NR operating band n83 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 642.34 | 128468 | 219 |
|  |  |  | High | 743 | 148600 | 738.32 | 147664 | 1 |
| 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 639.82 | 127964 | 219 |
|  |  |  | High | 740.5 | 148100 | 733.3 | 146660 | 1 |
| 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 637.48 | 127496 | 219 |
|  |  |  | High | 738 | 147600 | 728.46 | 145692 | 1 |
| 30 | 78 | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 |
| (Note 2) |  |  | High | 733 | 146600 | 716.8 | 143360 | 6 |
| Note 2: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1 |  |  |  |  |  |  |  |  |

<End of modified section 1>