3GPP TSG-RAN5 Meeting #90-e	R5-210945

# Electronic Meeting, 22nd February – 5th March 2021

Title:	Discussion on the default TDD configuration for EN-DC RF test cases

Source:	Huawei, Hisilicon

Agenda item:	5.3.2.17

Document for:	Endorsement

- Introduction

In TS 38.521-3 EN-DC Tx test cases, E-UTRA configuration is reconfigured to TDD Config 2 for the case of E-UTRA and NR UL simultaneous transmission when both bands are TDD (overlapping test points), while E-UTRA carrier is configured with the default TDD Config 1 as per TS 38.508 for other non-overlapping test points.

The purpose of this document is to analyse why TDD Config 2 also applies to the non-overlapping EN-DC Tx test cases so that TDD config 2 can replace TDD Config to be the default E-UTRA TDD configuration in TS 38.508.

- TDD configuration

In TS 38.521-3 EN-DC Tx test cases, E-UTRA configuration is reconfigured to TDD Config 2. As per 36.211, the supported uplink-downlink configurations for E-UTRA TDD are listed in Table 4.2-2.

Table 4.2-2: Uplink-downlink configurations

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

According to the default TDD-UL-DL-ConfigCommon in 38.508-1, take SCS 15kHz as an example, the TDD uplink-downlink configurations for NR TDD is described as the following figure, where “S” denotes the subframe mixed with 10 downlink slots, 2 GP slots and 2 uplink slots.

As per 38.521-3 clause 6.2B.1.3,

7.	For the case of testing overlapping E-UTRA and NR UL transmission scenario when both bands are TDD, ensure E-UTRA UL transmission overlaps with NR UL transmission in time by giving SCG a delay of 3 E-UTRA subframes, or by giving MCG a delay of 2 subframes.

If E-UTRA carrier in MCG is delayed by 2 subframes to enable simultaneous LTE and NR transmissions in EN-DC, then

It’s seen that for E-UTRA carrier, only TDD Config 2 and special subframe config 7 can achieve the accurate time domain alignment with the NR TDD carrier. Consequently, for overlapping test points when both E-UTRA band and NR band are TDD, E-UTRA carrier must be configured with TDD Config 2 and special subframe config 7.

Observation 1: For EN-DC test cases with overlapping test points including spurious emission, E-UTRA carrier must be configured with TDD Config 2 and special subframe Config 7 for time domain alignment with NR TDD configuration.

For other non-overlapping test points, E-UTRA TDD was configured to TDD config 1 as per the default configuration in 36.508. According to Table 4.2-2, TDD Config 2 only have one uplink subframe in sbf 2, which has one less UL subframe in sbf 3 than TDD Config 1. However, in 38.521-3 for EN-DC Tx test cases, the required measurement period is at least one uplink subframe as follows:

3.	For test ID 1~6 measure the sum of mean transmitted power over all EN-DC component carriers in the EN-DC, which shall meet the requirements described in table 6.2B.1.3.5-1 and the period of the measurement shall be at least the continuous duration of one active sub-frame.

It means one uplink subframe in E-UTRA carrier is sufficient to support all EN-DC Tx testing using non-overlapping test points. Hence, for EN-DC test cases with non-overlapping TPs, TDD Config 2 can take place of TDD Config 1 with guarantee of the testability and the same testing time.

Observation 2: For EN-DC test cases with non-overlapping test points, TDD Config 2 can take place of TDD Config 1 with guarantee of the testability and the same testing time.

Proposal: For EN-DC Tx test cases, TDD Config 2 could be used as the default configuration for E-UTRA TDD since it is not only necessary for overlapping testing but also applicable to non-overlapping testing.

- Proposals

For EN-DC Tx test cases, it is proposed RAN5 consider below proposal.

Proposal: For EN-DC Tx test cases, TDD Config 2 could be used as the default configuration for E-UTRA TDD since it is not only necessary for overlapping testing but also applicable to non-overlapping testing.

- References

[1] 3GPP TS 36.211 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation (Release 16)”.

[2] 3GPP TS 38.521-3 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 16)”.