3GPP TSG-RAN WG5 Meeting #90-e	R5-210992

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1459 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to XCAP test case 15.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per agreed R5-202502, In simservs document at step 5b, test case 15.5 shall have only <rule-deactivated/> element inside <conditions> element, but it currently has <no-answer> element included as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed <no-answer> from <conditions> element at step 5b. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case as unconditional communication forwarding can not be activated after step5b. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## 15.5	Communication Forwarding unconditional

### 15.5.1	Definition

Test to verify that the UE activates and deactivates IMS Multimedia Telephony Communication Forwarding unconditional. This process is described in 3GPP TS 24.604 [106].

### 15.5.2	Conformance requirement

Generic requirements for Communication Forwarding can be found from Annexes F.1 and F.4.

[TS 24.604, clause 4.2.1.2]:

The CFU service enables a served user to have the network redirect to another user communications which are addressed to the served user's address. The CFU service may operate on all communication, or just those associated with specified services. The served user's ability to originate communications is unaffected by the CFU supplementary service. After the CFU service has been activated, communications are forwarded independent of the status of the served user.

As a service provider option, a subscription option can be provided to enable the served user to receive an indication that the CFU service has been activated. This indication shall be provided when the served user originates a communication if the CFU service has been activated for the served user's address and for the service requested for the communication.

The maximum number of diversions permitted for each communication is a service provider option. The service provider shall define the upper limit of diversions. When counting the number of diversions, all types of diversion are included.

Reference(s)

3GPP TS 24.604 [106].

### 15.5.3	Test purpose

1)	To verify that the UE can request activation of Communication Forwarding unconditional with a correctly composed HTTP PUT request; and

2)	To verify that the UE can request deactivation of Communication Forwarding unconditional; and

3)	To verify that the UE can authenticate its HTTP requests by including a correctly composed Authorization header with credentials of the user to the request. The UE may either include the Authorization header to its initial request or when sending the request again after receiving 401 response from SS.

### 15.5.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is configured with the name of the XCAP root directory on the XCAP server and the user's directory name. If needed the UE is also configured with the HTTP Digest password to be used for XCAP. UE has activated an IPCAN bearer (e.g. PDP context or EPS bearer) with SS.

SS is configured with the HTTP Digest password for XCAP or shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

If the UE uses GAA as XCAP authentication scheme, GAA bootstrapping exchange has been performed according to annex C.29.2.

Test procedure

The generic test procedure according to annex C.29.1 is applied:

At step 1 activation of Communication Forwarding unconditional;

At step 5b, when sending 200 OK for GET, SS delivers a simservs document as specified in TS 24.604 [106] cl 4.9, including a non-empty rule set. Specifically, the SS includes the following:

<?xml version="1.0" encoding="UTF-8"?>

<simservs

xmlns="http://uri.etsi.org/ngn/params/xml/simservs/xcap"

xmlns:cp="urn:ietf:params:xml:ns:common-policy"

xmlns:ocp="urn:oma:xml:xdm:common-policy">

<communication-diversion active="true">

<cp:ruleset>

<cp:rule id="rule1">

<cp:conditions>

<rule-deactivated/>

</cp:conditions>

<cp:actions>

<forward-to>

<target>

px_XCAP_TargetUri

</target>

<notify-caller>true</notify-caller>

</forward-to>

</cp:actions>

</cp:rule>

</cp:ruleset>

</communication-diversion>

</simservs>

At step 7 deactivation of Communication Forwarding unconditional is respectively triggered at the UE.

### 15.5.5	Test requirements

1. SS shall check that the UE can authenticate itself correctly with the authentication scheme it supports:

-	HTTP Digest authentication (see Annex C.29.1 step 2 NOTE 1) or

-	GAA based authentication as specified in TS 33.222 [121] and TS 24.109 [119] (see Annex C.29.2).

2. SS shall check that after Annex C.29.1 step 6 the simservs document stored in the SS contains the following pieces of information supplied by the UE:

-	<communication-diversion> element with "active" attribute set as "true" or with “active” attribute not present.

-	within <cp:ruleset> one <cp:rule> element for communication forwarding as follows:

-	<cp:conditions> element missing or empty as forwarding is supposed to be unconditional and not containing a <rule-deactivated> element

-	<cp:actions> element containing <forward-to> element containing <target> element

-	value of target address to be px_XCAP_TargetUri

3. SS shall check that after step 9 the simservs document stored in the SS contains the following pieces of information supplied by the UE:

-	<communication-diversion> element with "active" attribute being set "false"

or

-	<communication-diversion> element with "active" attribute set as "true" or with “active” attribute not present.

-	within <cp:ruleset> one <cp:rule> element found at step 2 for Communication Forwarding unconditional as follows:

-	<cp:conditions> element containing a <rule-deactivated> element

<End of modifed section>