3GPP TSG-RAN WG5 Meeting #90-e	R5-211142

# Electronic Meeting, 22nd February – 5th March, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0383 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TP analysis for DC_11A_n79A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test points are not analyzed for DC_11A_n79A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added DC_11A_n79A in Table 4.3.3.1-1 to indicate it is completed Added test frequencies for DC_11A_n79A in Table D.2.8-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to test DC_11A_n79A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3, D.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ..38.521-3. CR .0928.. |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR implementation guideline: Table rows to be inserted in Table 4.3.3.1-1 after implementation of CR #0359. Table rows to be inserted in Table D.2.8-1 after implementation of CR #0360 TR rapporteur to guide MCC in order to implement the CRs impacting the new Table format |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.3.3.1	Reference sensitivity test cases for EN-DC

This clause contains information on test point analysis and status for FR1 NR CA test cases in TS 38.521-3 [4] clause 7. The analyses are performed per CA configuration in Table 4.3.3.1-1, Table 4.3.3.1-2 and Table 4.3.3.1-3.

Table 4.3.3.1-1: Reference Sensitivity test cases per EN-DC configuration (2CC)

| Band or band configuration | Single UL | Refsens exception, victim band (Note 2) | Requirement coverage (Note 2) | Comments |
| --- | --- | --- | --- | --- |
| DC_(n)XAA |  |  |  |  |
| 5 | Only single UL requirements defined | 5 |  |  |
| 12 | Only single UL requirements defined | 12 |  |  |
| 38 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 41 | Yes | N/A | NE | Added at RAN5#88-e |
| 48 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 71 | Yes | 71 | Exception | Added at RAN5#88-e |
| DC_XA_nXA |  |  |  |  |
| DC_2A_n2A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_3A_n3A | Yes | 3 | Exception | Added at RAN5#88-e |
| DC_5A_n5A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_7A_n7A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_41A_n41A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_48(n)n48AA | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_66A_n66A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_XA-nYA (Note 1) |  |  |  |  |
| DC_1A_n3A | Yes | 1 (IMD3) CBI | NE (Note 1) IMD3 CBI | Added at RAN5#89-e |
| DC_1A_n78A | No | 1 (IMD4) | IMD4 | Added at RAN5#88-e |
| DC_3A_n1A | Yes | 1 (IMD3) CBI | NE (Note 1) IMD3 CBI | Added at RAN5#89-e |
| DC_3A_n78A | Yes | 3 (IMD2) 3 (IMD4) 3 (HD) n78 (HM) | NE (Note 1) IMD2 PC2&PC3 IMD4 PC2&PC3 HD HM | Added at RAN5#89-e |
| DC_7A_n78A | No | N/A | NE | Added at RAN5#89-e |
| DC_11A_n79A | No | 11 (HM ¦2*fB11DL-1*fB79¦) | NE, HM | Added at RAN5#90-e |
| DC_20A_n3A | No | n3 (IMD4), 20 (IMD4) | IMD4, IMD4 | Added at RAN5#89-e |
| DC_20A_n78A | No | 20 (IMD4), n78A (HD) | IMD4, HD | Added at RAN5#88-e |
| DC_40A_n1A | No | N/A | NE | Added at RAN5#89-e |
| DC_40A_n78A | No | 40 (HM), | HM | Added at RAN5#89-e |
| NOTE 1: If single UL is allowed in a DC_XA-nYA configuration the IMD requirements does not apply for a single UL UE and non-exception requirements are tested. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |

Table 4.3.3.1-2: Reference Sensitivity test cases per EN-DC configuration (3CC)

| Band or band configuration | Single UL | UL config | Refsens exception, victim band | Requirement coverage (Note 2) | Fallback CA configurations | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XCA |  |  |  |  |  |  |
| 41 | Yes | DC_(n)41AA | N/A | None | DC_(n)41AA DC_41A_n41A | Added at RAN5#89-e |
| 41 | Yes | DC_41A_n41A | N/A | Non-exception | DC_(n)41AA DC_41A_n41A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined | DC_(n)48AA | N/A | None | DC_(n)48AA DC_48A_n48A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined | DC_48A_n48A | N/A | Non-exception | DC_(n)48AA DC_48A_n48A | Added at RAN5#89-e |
| DC_XA-nYC |  |  |  |  |  |  |
| DC_XA-nY(2A) |  |  |  |  |  |  |
| DC_XA-nYA-nZA |  |  |  |  |  |  |
| DC_XA-YA-nZA |  |  |  |  |  |  |
| DC_1A-7A_n78A | No | DC_1A_n78A | 7 (3 band IMD4) | IMD4 |  |  |
|  | No | DC_7A_n78A | 1 (3 band IMD4) | IMD4 |  |  |
| DC_1A-20A_n3A | Yes | DC_1A_n3A | No 3CC exception | No test required |  |  |
|  | No | DC_20A_n3A |  |  |  |  |
| DC_1A-20A_n78A | No | DC_1A_n78A | 20 (IMD5 ¦ | IMD5 |  |  |
|  | No | DC_20A_n78A | 1 (IMD3 | IMD3 |  |  |
| DC_3A-20A_n78 | Yes | DC_3A_n78A | No 3CC exception |  |  |  |
|  | No | DC_20A_n78A | 3 (3 band IMD3) | IMD3 |  |  |
| DC_3A-40A_n1A | Yes | DC_3A_n1A | 40 (3 band IMD5) | IMD5 if UE supporting dual UL |  |  |
|  | No | DC_40A_n1A | No 3CC exception |  |  |  |
| DC_7A-20A_n78A | No | DC_7A_n78A | 20 (3 band IMD2) 20 (3 band IMD5) | IMD2, IMD5 |  |  |
|  | No | 2 DC_0A_n78A | 7 (3 band IMD2) | IMD2 |  |  |
| DC_(n)XAA-nYA |  |  |  |  |  |  |
| DC_XA_nXA_nYA |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA-nYA configuration the IMD requirements does not apply for a single UL UE NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

Table 4.3.3.1-3: Reference Sensitivity test cases per EN-DC configuration (4CC)

| Band or band configuration | Single UL | UL config | Refsens exception, victim band | Requirement coverage (Note 2) | Fallback CA configurations | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)XCA |  |  |  |  |  |  |
| 41 | Yes | DC_(n)41AA | N/A | None | DC_(n)41CA DC_41C_n41A | Added at RAN5#89-e |
| 41 | Yes | DC_41A_n41A | N/A | Non-exception | N/A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined | DC_(n)48AA | N/A | None | DC_(n)48CA DC_48C_n48A | Added at RAN5#89-e |
| 48 | Only single UL requirements defined | DC_48A_n48A | N/A | Non-exception | N/A | Added at RAN5#89-e |
| DC_XA-nY(2A)-nZA |  |  |  |  |  |  |
| DC_XA-nYA-nZC |  |  |  |  |  |  |
| NOTE 1: If single UL is allowed in a DC_XA-nYA configuration the IMD requirements does not apply for a single UL UE. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 |  |  |  |  |  |  |

{Unchanged sections skipped}

# Annex A: Derivation documents

The documents and spreadsheets used to give the background for the selected test points for each test case are included in the present document as zip files.

The name of the zip shall:

-	Include a prefix allowing easier grouping of fi“38.521-1_TPanalysis”, “38.521-2_TPanalysis” or “38.521-3_TPanalysis”.les in the same area, e.g. .

-	Include Test Case Number(s) and an abbreviation Test Case Name, e.g. “6.2.1_MOP”, “7.6.2.InB_Block” or “6.2.1_MOP+6.2.2_MPR”.

-	In cases where multiple analysis is needed per test cases, e.g. for different CA configurations, include the CA band combination applicable in the parentheses, e.g. add “(1A-3A)” for CA_1A-3A.

Concatenated example file name: “38.521-1_TPanalysis_6.2.1_MOP.zip”.

If there is an update of test points for a test case the old corresponding zip file shall be replaced with a new zip file with a version stepping in the file name. e.g. “nnn_v2.zip”. The aim is to provide a reference to completed test cases, so that test points for similar test cases can be selected on a common basis.

For cases when no spreadsheet is used then the principles for selecting reference sensitivity test points are described in Annex B, C or D.

# Annex B: Principles for test point selection for NR CA reference sensitivity test cases

# B.1	General

From TS 38.521-1 [2] (Table 7.3.2.4.1-1), the initial conditions used for NR RX reference sensitivity is given below.

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range, Mid range, High range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest (NOTE 4) Lowest UL / Lowest DL, Lowest UL / Highest DL (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFERENCE SENSITIVITY (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFERENCE SENSITIVITY refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: According to asymmetric channel bandwidths specified in clause 5.3.6. NOTE 4: For n70, in addition to default test configurations, additional configurations shall be used to verify reference sensitivity requirements with the UE TX-RX frequency separation of 295MHz (table 5.4.4-1): 5 MHz CH BW with DL @ low range, UL @ mid range 5 MHz CH BW with DL @ mid range, UL @ high range 10 MHz CH BW with DL @ low range, UL @ high range |  |  |  |  |

For reference, the initial test condition for E-UTRA-CA (inter-band DL CA and UL CA) is listed below: (Table 7.3B.4.1-1 in TS 38.521-1 [2])

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 7.1-2 |  |  |  | B: Low range, High range C: Low range, High range (Note 4) |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg, Highest NRB_agg (Notes 3, 4) |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  | UL Allocation |  |  |  |  |  |
| PCC NRB | SCCs NRB | CC MOD | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | QPSK | 6+25 | QPSK | 6 | P_6@0 | - | - | - |
| 15 | 50 | QPSK | 15+50 | QPSK | 15 | P_15@0 | - | - | - |
| 25 | 25 | QPSK | 25+25 | QPSK | 20 | P_20@0 | - | - | - |

# B.2	Test case structure

To improve the very time-consuming TP analysis made in E-UTRA for each CA configuration, a new approach is taken for NR. This approach which is described in this clause is relying on certain properties of the RAN4 CA requirements.

- For E-UTRA, fallbacks are skipped unless there is a documented exception in TS36.521-2. This may cause that a fallback is skipped even if no technical analysis has been made leading to insufficient testing of demanding CA configurations. For NR fallback can only be skipped if there has been a technical analysis performed in TR 38.905.

- The RAN4 decision to only specify the affected (aggressor and victim) bands for an exception enables a different approach than in E-UTRA that in most cases makes fallback analysis redundant. This is not entirely true for requirements under clause 7.3A.5 and 7.3A.6 of TS38.101-1 [5] but it is assumed that the same approach can be used also in those cases as it greatly simplifies test cases.

- Test cases can be split into default and additional test cases where default test cases only test where there is no exception (single carrier requirements apply), and additional only test the exceptions in TS 38.101-1 [5] clause 7.3A.

- Default test cases need to be defined from 2 up to 5 CCs. Since they cover no exception, no fallback analysis is required, and the applicability rule should say that only highest CC number is required (similar to other Rx tests).

- As the default test cases are not covering any CA exceptions, there is no need to configure UL CA in these test cases. Single UL configuration is sufficient to fulfil the test purpose.

- Additional test cases only need to be defined for 2CC (and for intermodulation also 3CC) since this is sufficient for testing the exception. Adding more CCs is not considered to affect the test result and does not add any value.

- Additional test cases only need to consider pure inter-band configurations since requirements are the same for intra-band contiguous or non-contiguous, and the essential aspect to test is the interference between bands.

- It is important for sufficient test coverage to ensure that all band combinations with the highest number of CCs supported in the UE can always be tested.

- The highest supported CC number may be with a band including an exception that can be avoided by setting a different test frequency (e.g. harmonic interference). In such cases, there is a need to include test points in the default test case so that this CA configuration can be tested without an exception (see example 1 below).

- The highest supported CC number may be with a band including an exception that cannot be avoided by setting a different test frequency (e.g. cross band isolation). In this case the test requirements of the default test that are normally general for any CA config need to include this specific exception requirement. In addition, one fallback configuration needs to be tested to ensure test coverage of the single carrier requirement for the victim band that applies in this case. In order to simplify test applicability this fallback configuration will be specified inside the higher order test case (see example 2 below).

- The highest supported CC number may be with a band including a limitation that is not present in the lower order fallback (e.g. UL is not possible in one of the bands). In this case, one fallback configuration needs to be tested to ensure test coverage of the single carrier requirement with UL active in the band. In order to simplify test applicability this fallback configuration will be specified inside the higher order test case.

- To ensure that it has been checked that the CA configuration has been fully analysed with regard to test configuration and fallback testing, the CA configuration is listed in TR 38.905.

# B.3	Test Environment

Reference sensitivity is one of the critical test cases for NR. Considering NR CA testing scenario is very similar to E-UTRA_CA test case, similar Test Environment shall be used for NR CA testing, i.e. NC, TL/VL, TL/VH, TH/VL, TH/VH.

# B.4	Test Frequencies selection

In E-UTRA DL CA and UL CA reference sensitivity testing, Low range and High range are selected for intra-band CA testing. Mid range is selected for inter band E-UTRA CA testing.

Considering NR CA testing scenario is very similar to E-UTRA_CA test case, similar Test Frequencies should be used for NR CA default case.

It is proposed that Low and High Range are tested as default for intra-band and mid range for inter-band. For CA combinations containing intra-band configuration in an inter-band CA configuration it is proposed that low and high range are tested for intra-band CA and mid channel for inter-band without intra-band component. It is also proposed that the fallback configurations from intra-band configuration to single carrier component do not need to be tested even if the test frequency would differ (e.g. XA-YC -> XA-YA fallback does not need to be tested even if YC is tested with low/high frequency and YA would be tested with mid frequency)

For CA configurations affected by exceptions the test frequency cannot be freely chosen. One test frequency per exception requirement is sufficient to test the requirement, in addition to testing the case where the exception is avoided. This is indicated in table 2.3-2. Exceptions in TS 38.101-1 [5] clause 7.3A.5 (2UL intermodulation) are not specified since the test frequency and channel bandwidth is already specified in TS 38.101-1 [5].

Table B.4-1 Test frequency selection per band pair for UL harmonics exception.

| Band pair (Note 2) | Frequency | Channel BW [MHz] | Comment |  |
| --- | --- | --- | --- | --- |
| n1A / n77A | Mid / Low | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | Mid / 3900 | 20/ Highest | Exception Note 2 of TS 38.101 table 7.3A.4-1 |  |
|  | Mid / 3870 | 20/20 | Exception Note 3 of TS 38.101 table 7.3A.4-1 |  |
| n3A / n77A | TBD/TBD | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | TBD/TBD | Highest/Highest | Exception Note 2 of TS38.101 table 7.3A.4-1 |  |
|  | TBD/TBD | 20/20 | Exception Note 3 of TS38.101 table 7.3A.4-1 |  |
| n3A / n78A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | Mid/3495 MHz | Highest/Highest | Exception Note 2 of TS38.101 table 7.3A.4-1 |  |
|  | Mid/3465 MHz | 20/20 | Exception Note 3 of TS38.101 table 7.3A.4-1 |  |
| n8A / n78A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | Mid/3590 MHz | Highest/Highest | Exception Note 5 of TS38.101 table 7.3A.4-1 |  |
| n70A / n71A | Mid/High | Highest/Highest | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |
|  | Low/Low | Highest / 10 | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 20@10 |  |
|  | Low/Low | 5 / 5 | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 8@10 |  |
| NOTE 1: This selection is used in test case 7.3A.1_1 unless otherwise stated. NOTE 2: Aggressor band in bold. |  |  |  |

Proposal 2a: Low range and high range for intra band CA and mid range for inter-band CA shall be selected for NR in default test cases 7.3A.1 to 7.3A.4 in general, but final selection is band dependent. For CA combinations containing intra-band configuration in an inter-band CA configuration low and high range shall be selected for intra-band CA and mid channel for inter-band without intra-band component. The fallback configurations from intra-band configuration to single carrier component do not need to be tested even if the test frequency would differ.

Proposal 2b: In the additional test case, one test frequency per exception is selected.

# B.5	Test Channel Bandwidth selection

The objective is how to verify the NR CA reference sensitivity. In intra-band E-UTRA CA reference sensitivity testing, two extreme bandwidth combinations corresponding to Lowest NRB_agg Highest NRB_agg are selected. Highest NRB_agg is selected for inter band E-UTRA CA testing.

As a simplification the highest aggregated channel bandwidth is proposed to test NR CA reference sensitivity.

Proposal 3: Highest aggregated channel bandwidth combinations shall be selected for NR CA reference sensitivity measurement. (Highest NRB_agg).

# B.6	Modulation selections

QPSK is used for both uplink and downlink modulations for E-UTRA reference sensitivity measurement which is the same as for E-UTRA standalone reference sensitivity testing. There is no particular reason to deviate from current E-UTRA configurations of modulation scheme selection. NR modulations shall also follow what is selected in standalone NR testing.

# B.7	Examples

Example 1 (highest CC number has an exception that can be avoided):

- UE supports CA_XA-YA-ZA-RA (4DL CA)

- CA_X-Y has an exception if testing Low+Low freq

- CA_X-Y has no exception if testing Mid+Mid freq

- Other bands pairs have no exception

Applicable Test cases for the example:

7.3A.1 2CC non-exception/“default”

Skip test (no fallback analysis required)

7.3B 3CC non-exception/“default”

Skip test (no fallback analysis required)

7.3A.3 4CC non-exception/“default”

Test CA_XA-YA-ZA-RA in Mid+Mid freq in bands X and Y respectively avoiding the exception

7.3A.1_1 2CC exception/“additional”

Test CA_X-Y in Low+Low freq

Example 2 (highest CC number has an exception that cannot be avoided):

- UE supports CA_XA-YA-ZA-RA (4DL CA)

- CA_X-Y has an exception always (e.g. cross band isolation) with band Y being the victim

- Other bands pairs have no exception

Applicable Test cases for the example:

7.3A.1 2CC non-exception/“default”

Skip test (no fallback analysis required)

7.3A.2 3CC non-exception/“default”

Skip test (no fallback analysis required)

7.3A.3 4CC non-exception/“default”

Test CA_XA-YA-ZA-RA and add exception in test requirements

Add test points for 3CC fallback avoiding the exception CA_YA-ZA-RA

7.3A.1_1 2CC exception/“additional”

Test CA_X-Y

# B.8 Current test completion status per CA configuration

The completion status per EN-DC configuration is documented in clause 4.3.3.1.

# Annex C: Principles for test point selection for FR2 NR CA reference sensitivity test cases

FFS

# Annex D: Principles for test point selection for EN-DC reference sensitivity test cases

# D.1	General

The purpose of this Annex is to describe the test point selection of NR and E-UTRA carriers for RX sensitivity testing within FR1. Considering the high number of EN-DC band combinations the procedure has been developed to carefully reduce the number of test points. Since the objective of TS 38.521-3 [4] specification is providing conformance testing requirement for NR in the case of EN-DC scenarios, the number of E-UTRA test points can be reduced.

TS 38.521-3 [4] include exception test cases and non-exception test cases for inter-band EN-DC with FR1. The exceptional test cases must be tested as per configurations defined in TS 38.521-3 [4]. The procedure covers selection of both non-exceptional case and exception cases.

When no NR or E-UTRA exception or additional requirements exist, the E-UTRA anchor is configured such that it does not interfere with NR operation (based on TS 38.101-3 [7] clause 6.1 and 7.1). The EN-DC testing is performed in E-UTRA anchor-agnostic mode. E-UTRA is tested but in SA as per TS 38.521-1 [2].

General initial conditions to be used for NR RX sensitivity testing are:

Table D.1-1 Initial test condition for standalone NR RX sensitivity

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest supported SCS per test channel BW |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | RB allocation | Modulation |  | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (Note 1) | DFT-s-OFDM QPSK |  | REFERENCE SENSITIVITY (Note 2) |

# D.2	Requirements

## D.2.1	Defined EN-DC configurations

Downlink EN-DC configurations in 38.101-3 V16.5.0 clause 5.5B are:
Currently defined downlink EN-DC configurations in 38.101-3 [7] V16.6.0 clause 5.5B are:

- Intra-band contiguous EN-DC configurations within a single band includes max 2 NR CCs and max 3 E-UTRA CC, but in total max 4 CCs.

- Intra-band non-contiguous EN-DC configurations within a single band does not include NR CA (max 1NR CC), only E-UTRA CA (max 3 E-UTRA CC)

- Inter-band EN-DC configurations include up to 6 bands and 8 CCs but for NR the maximum is:

- 2 NR CCs in 1 band (nXC or nX(2A))

- 4 NR CCs in 2 bands (nXA-nYA, nXA-nYC, nXA-nY(2A), nX(2A)-nY(2A))

- Combinations of intra-band contiguous EN-DC + inter-band E-UTRA/NR CA.

- Combinations of intra-band non-contiguous EN-DC + inter-band E-UTRA/NR CA.

Currently defined uplink EN-DC configurations in TS 38.101-3 [7] V16.6.0 clause 5.5B are:

- The UL configuration contains at least 1 E-UTRA CC and 1 NR CC, meaning 2 UL CC.

- 3 UL CC configurations are also defined only with intra-band contiguous CA on either E-UTRA or NR (DC_XA-nYC or DC_XC-nYA).

## D.2.2	Definition of exception requirements

Before going into the test case details, the term “exception” needs to be clarified since it is different for EN-DC compared to SA. We can have the following situations:

- EN-DC config without exception

  - Anchor agnostic, only need to be tested for UE not supporting SA

- EN-DC config with exception when the exception applies

  - Need to be tested for all EN-DC UEs

- EN-DC config with exception when the exception does not apply (single carrier requirement applies)

  - Need to be tested for some exception types. Which exception types that need to this test coverage is under study.

  - In some cases, it is not possible to avoid the exception.

  - Anchor agnostic approach shall not be followed. It is important that E-UTRA aggressor is active to test performance when the interferer falls outside of the victim carrier

## D.2.3	Reference sensitivity

Intra-band

Unlike for standalone NR, there are exceptions for intra-band operation, currently limited to band 3 for non-contiguous operation and band 71 for contiguous operation.

The exceptions in band 3 is due to 2UL intermodulation interference making it similar to the corresponding inter-band intermodulation case. This scenario did not happen in standalone NR mode due to that there is no UL CA defined for CA_n3(2A).

It can therefore be treated in the same way as inter-band intermodulation and added in the “additional” test case.

The intra-band contiguous exceptions are not expected to be very common. Currently only band 71 is affected, which can be added in the “additional” test case.

Inter-band

For both SA and NSA there are 4 different types of interference related to inter-band operation:

- UL harmonic interference (HD)

- Receiver Harmonic Mixing (HM)

- Intermodulation due to Dual uplink (IMD)

- Cross band isolation (CBI)

The exception requirements for dual uplink intermodulation apply only for a RAN4 specified test frequency setting per UL configuration, meaning the only way to avoid the exception is to test with single UL active. However, there may be cases when the exception can be avoided by testing a different UL configuration. For example, in DC_1A-3A_n28A the UL configuration can be either DC_1A_n28A or DC_3A_n28A. If DC_1A_n28A is configured there is an IMD product causing exception in band 3 that cannot be avoided. But if DC_2A_n28A is configured in UL there is no exception and single carrier requirements can be tested.

More details on how the exception requirements were derived by RAN4 can be found in TR 37.863 (Rel-15), TR 37.716 (Rel-16) and TR 37.817 (Rel-17). These RAN4 TRs can be used to identify all the applicable requirements for a certain EN-DC configuration which are otherwise spread out over multiple sections in TS 38.101-3 [7]. Additionally, the TRs specify exactly which aggressors that contribute to the exception for IMD which is not defined in the TS.

RAN5 have agreed for SA tests that all the interference types except intermodulation can be tested with 1UL. Doing the same for NSA would simplify the test cases and would also mean that the main interferer (the aggressor) can be tested with higher power.

Most other Rx requirements than reference sensitivity in intra-band contiguous EN-DC configuration require a configuration with 2UL however, meaning the “default” test cases need to be tested with 2UL active in this scenario

There are combinations of intra-band contiguous/non-contiguous EN-DC + inter-band E-UTRA/NR CA which are listed in the inter-band EN-DC configuration tables of T S38.101-3 [7]. These configurations do not add any exception requirements and are therefore proposed not to be tested unless they contain maximum number of NR CCs supported by the UE.

## D.2.4	Rx requirements other than reference sensitivity

The requirements are in some cases referring back to standalone requirements in TS 36.101/ TS 38.101-1. In other cases, specific exceptions for EN-DC are defined in TS 38.101-3 [7]. This is summarized in Table 2.1.3-1.

Table D.2.4-1: Rx requirements

| Clause | Title | Requirement |  |  |
| --- | --- | --- | --- | --- |
|  |  | Intra-band contiguous | Intra-band non-contiguous | Inter-band |
| 7.4B | Max input level | Exception with 2UL | Standalone | Standalone |
| 7.5B | ACS | Exception with 2UL | Standalone | Standalone |
| 7.6B.2 | IBB | Exception with 2UL | Standalone | Standalone |
| 7.6B.3 | OOBB | Exception with 2UL | Standalone | Standalone (only 2 band needed) |
| 7.6B.4 | NBB | Exception with 2UL | Standalone | Standalone |
| 7.7B | Spurious response | Exception with 2UL | Standalone | Standalone (only 2 band needed) |
| 7.8B | Intermodulation | Exception with 2UL | Standalone | Standalone |
| 7.9B | Rx spurious | Standalone | Standalone | Standalone |

## D.2.5	Test case structure and test coverage

As seen above, the EN-DC requirements are similar to the SA requirements from a structure point of view. Therefore, the same test case structure as for SA where test cases are separated into “default” and “additional” test cases can be considered for EN-DC which can greatly reduce complexity in the RAN5 spec as well as reducing the need to complicated fallback test analysis. However, the additional complexity of EN-DC requirements means that a clear separation between exception (“additional”) test cases and “default” test cases is not as straightforward.

The following principles are suggested for EN-DC Rx test cases:

- Let the 2CC test cases cover all reference sensitivity exceptions. Include test coverage of reference sensitivity when exception is avoided (exactly which cases need this additional test coverage is under study to be included based on worst case assessment).

- The 2CC test cases always need to be tested even for UE supporting more CCs since some of the exceptions are not covered in >2CC test cases.

- Let the 3CC-5CC test cases cover mainly non-exceptions, for EN-DC configurations where no exception apply (anchor agnostic testing), and when the exception is avoided (not anchor agnostic). 2UL 3 band intermodulation exception exceptions are tested in the 3CC test.

- Highest number of NR CCs per band combination supported in the UE need to be tested. Anchor agnostic testing unless the EN-DC configuration has an exception requirement. Only 1 E-UTRA CC need to be tested for intra-band non-contiguous EN-DC and inter-band EN-DC. Maximum number of E-UTRA CCs need to be tested for intra-band contiguous EN-DC (1 band). This is further explained in clause 3.2.6.

- Since requirements for 3UL and 2UL for the Rel-16 EN-DC configurations are the same there is no technical reason to test receiver requirements with 3UL configured. This may change in Rel-17 but will not have an impact on structure if we can add 3UL as new test points in existing tests.

- The test configuration shall be with 1UL or 2UL active depending on the exception type as indicated in Table D.2.5-1.

Table D.2.5-1: UL configuration to test

| Exception type | Intra-band contiguous EN-DC | Intra-band non-contiguous EN-DC | Inter-band EN-DC |
| --- | --- | --- | --- |
| Intra-band contiguous (band 71) | 2UL1 | - | - |
| Intra-band non-contiguous (band 3) | - | 2UL1 | - |
| UL harmonics, Rx mixing, cross band isolation | - | - | 2UL |
| 2UL Intermodulation | - | - | 2UL1 |
| EN-DC config w/o exception | 2UL2 | 1UL (anchor agnostic)3 | 1UL (anchor agnostic) |
| NOTE 1: Exception requirements in this apply only with 2UL and apply only for one test frequency/BW setting, no refsens requirement apply otherwise so the exception can only be avoided by having only 1UL active. NOTE 2: Other than refsens requirements in this configuration mandate 2UL NOTE 3: Requirements in this configuration mandate 2UL (TS 38.101-3, 7.1) unless UE supports only single UL. However, there is no Rel-16 configuration w/o exception where 2UL is supported meaning only 1UL is configured in the test case |  |  |  |

- Void

- Other than reference sensitivity test cases should use the MSD=0 test points defined in reference sensitivity test case, or minimum achievable MSD. This is currently ensured by using anchor agnostic configuration for inter-band EN-DC, but other solutions are not precluded.

- Test cases need to be defined from 2 up to 5 CCs for Rel-16. The number of CCs may increase in Rel-17. The reason for not needing more than 5CC is that it is sufficient to test max number of NR CCs as well as all defined exceptions.

## D.2.6	EN-DC configurations to test

### D.2.6.1	Lower order fallbacks

In E-UTRA RAN5 specifications, the lower order CA fallback cases are important in the test point analysis. If requirements are the same in the lower order fallback, then the fallback can be skipped to save test time. The same principle can be applied for EN-DC, but there is a need to keep in mind that this is only for CA fallback and not EN-DC fallback. Additionally, lower order CA fallbacks can be split into E-UTRA CA fallbacks and NR CA fallbacks.

EN-DC fallback

EN-DC fallback does not need to be handled since this would mean falling back to pure E-UTRA operation, which is covered by TS 36.521-1 test cases.

E-UTRA CA fallback

Since it has been agreed to use anchor agnostic approach for EN-DC test cases (TS 38.521-3 [4], clause 4.6), it is enough to test with 1 E-UTRA CC unless more CCs are needed to test an exception requirement in TS 38.101-3 [7].

There are exception requirements in TS 38.101-3 [7]for intra-band-contiguous EN-DC (not reference sensitivity, but other Rx requirements as shown in clause D.2.3) meaning the maximum number of E-UTRA CCs need to be tested in this scenario.

E-UTRA CA fallback in intra-band contiguous EN-DC can result in change of EN-DC config from contiguous to non-contiguous (e.g. DC_(n)41DA -> DC_41C_n41A). This change means requirements are different, but since the non-contiguous requirement is same as the standalone requirement it does not need to be tested.

E-UTRA CA fallback in inter-band contiguous EN-DC can result in change of EN-DC config to intra-band contiguous (e.g. DC_1A-(n)41AA->DC_(n)41AA). This change means requirements are different, implying that both EN-DC config types may need to be tested. See clause D.2.6.2.

NR CA fallback

As shown in clause D.2.1, the maximum number of NR CCs for Rel-16 is 4, which only occurs in inter-band EN-DC (2 NR bands). The fallback to 2-3 NR CCs (2 bands) don’t need to be tested since requirements are still the same. The fallback to single NR band with 1CC may need to be tested in the cases where the 2CC configuration had a reference sensitivity exception that is avoided in the higher order case.

Table D.2.6.1-1: EN-DC configurations requiring testing and max number of CCs

| Clause number | Title | Requirement coverage | EN-DC configuration type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Intra-band cont EN-DC | Intra-band non-cont EN-DC | Inter-band EN-DC |
| 7.3B.2.1 | Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs) | - Band n71 exceptions - standalone NR requirements | Yes | - | - |
| 7.3B.2.2 | Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs) | - Band n3 exceptions - standalone NR requirements | - | Yes | - |
| 7.3B.2.3 | Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs) | - UL harmonics, harmonic mixing, cross band isolation, 2UL intermodulation (2 band) - standalone NR requirements | - | - | Yes |
| 7.3B.2.3_1.1 | Reference sensitivity for EN-DC within FR1 (3 CCs) | - standalone NR requirements, - 2UL intermodulation (3 band) | Yes (2E-UTRA+1NR, Note 1) | - | Yes (1E-UTRA+2NR, 2E-UTRA+1NR, Note 2) |
| 7.3B.2.3_1.2 | Reference sensitivity for EN-DC within FR1 (4 CCs) | - standalone NR requirements | Yes (3E-UTRA+1NR, Note 1) | - | Yes ( 1E-UTRA+3NR, Note 4) |
| 7.3B.2.3_1.2 | Reference sensitivity for EN-DC within FR1 (5 CCs) | - standalone NR requirements | - | - | Yes (1E-UTRA+4NR, Note 4) |
| NOTE 1: This is needed for other than refsens Rx test case that refers back to refsens test config table NOTE 2: For EN-DC configs affected by 2UL intermodulation, 2E-UTRA+1NR may need to be tested when E-UTRA CC1 is aggressor and E-UTRA CC2 is victim or when E-UTRA CC1 and CC2 are aggressors and NR CC is victim) NOTE 3: Void NOTE 4: Test of max number of NR CCs. Anchor agnostic testing unless the EN-DC configuration has an exception requirement |  |  |  |  |  |

### D.2.6.2	EN-DC configurations requiring testing and max number of CCs

There is an issue in only requiring highest CC number to be tested, since this rule can only be applied within one EN-DC configuration type. For example, if the UE supports 3E-UTRA+1NR intra-band contiguous EN-DC and 1E-UTRA+2NR inter-band EN-DC, both configurations need to be tested for sufficient test coverage of core requirements. Another problem is that highest CC number to test within Inter-band EN-DC is not easy to determine it shall include 1 or 2 E-UTRA CCs, not more.

### D.2.6.3	Test coverage

As explained in clause D.2.1 only certain configurations are specified in TS 38.101-3 [7].

Additionally, some configurations may not need to be tested for reasons like:

- To test an exception, it is sufficient to test with fewer CCs (“No test needed” in tables below)

- No exception requirement exists (“N/A” in tables below)

- No such configuration is defined in TS 38.101-3[7] clause 5.5B (“N/A” in tables below)

For configurations that need to be tested, anchor agnostic approach can be used for E-UTRA when testing a non-exception requirement. In other cases, the E-UTRA carrier need to be fully configured.

The different cases that may happen for 3CC and 4CC configurations, and the proposed test coverage are listed in the tables below where the following general rules have been applied:

- Harmonic exceptions are only tested in the 2CC test and need not be considered in >2CC test cases. 2CC test cases are always run

- Any inter-band EN-DC configuration with more than 2 E-UTRA bands will not be tested

- Any inter-band EN-DC configuration with more than 1 E-UTRA CC per band will not be tested

Table D.2.6.3-1: 3CC test coverage

| EN-DC type | E-UTRA CA | NR CA | Notation | Test coverage |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Exception type |  |  |  |
|  |  |  |  | No exception | UL harmonics, harmonic mixing, cross band isolation | 2UL intermodulation |  |
| Intra-band contiguous EN-DC (1 band) | No | Yes | DC_(n)XAC Note 1 | N/A | N/A | N/A |
|  | Yes | No | DC_(n)XCA | Test needed due to other than refsens exception in intra-band EN-DC | N/A | N/A |
| Intra-band non-contiguous EN-DC (1 band) | Yes (cont) | No | DC_XC_nXA | No test needed. Only 2CC need to be tested | N/A | N/A |
|  | Yes (non-cont) | No | DC_XA-XA_nXA | No test needed. Only 2CC need to be tested | N/A | N/A |
|  | No | Yes (cont) | Note 1 | N/A | N/A | N/A |
|  | No | Yes (non-cont) | Note 1 | N/A | N/A | N/A |
| Inter-band EN-DC | No | Yes (intra-cont) | DC_XA-nYC | Test needed (if max NR CC) – anchor agnostic | Test needed (if max NR CC) | Test needed (if max NR CC) |
|  | No | Yes (intra-non-cont) | DC_XA-nY(2A) | Test needed (if max NR CC) – anchor agnostic | Test needed (if max NR CC) | Test needed (if max NR CC) |
|  | No | Yes (inter) | DC_XA-nYA-nZA | Test needed (if max NR CC) – anchor agnostic | Test needed (if max NR CC) | Test needed (if max NR CC or IMD 3 band) |
|  | Yes (intra-cont) | No | DC_XC-nYA | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested |
|  | Yes (intra-non-cont) | No | DC_XA_XA-nYA | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested |
|  | Yes (inter) | No | DC_XA-YA-nZA | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested | Test needed if IMD(3band) |
| Intra-band contiguous EN-DC (2 band) | No | Yes | DC_(n)XAA-nYA | Test needed (if max NR CC) – anchor agnostic | Test needed (if max NR CC) | Test needed (if max NR CC) |
|  | Yes | No | DC_XA-(n)YAA | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested |
| Intra-band non-contiguous EN-DC (2 band) | No | Yes | DC_XA_nXA_nYA | Test needed (if max NR CC) – anchor agnostic | Test needed (if max NR CC) | Test needed (if max NR CC) |
|  | Yes | No | DC_XA_YA_nYA | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested |
| NOTE 1: No such config in TS 38.101-3 [7] V16.5.0 NOTE 2: CC BW class C is indicated in the table. The same rules apply to BW class B NOTE 3: Different test coverage is indicated by color coding in this table. |  |  |  |  |  |  |

Table A.4.2.6.3-2: 4CC test coverage

| EN-DC type | E-UTRA CA | NR CA | Notation | Test coverage |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Exception type |  |  |
|  |  |  |  | No exception | UL harmonics, harmonic mixing, cross band isolation | 2UL intermodulation |
| Intra-band contiguous EN-DC (1 band) | No | Yes | Note 1 | N/A | N/A | N/A |
|  | Yes | No | DC_(n)XDA | Test needed due to other than refsens exception in intra-band EN-DC | N/A | N/A |
| Intra-band non-contiguous EN-DC (1 band) | Yes (cont) | No | DC_XD_nXA | No test needed. Only 2CC need to be tested | N/A | N/A |
|  | Yes (non-cont) | No | DC_XA-XC_nXA | No test needed. Only 2CC need to be tested | N/A | N/A |
|  | No | Yes (cont) | Note 1 | N/A | N/A | N/A |
|  | No | Yes (non-cont) | Note 1 | N/A | N/A | N/A |
| Inter-band EN-DC | No | Yes (cont) | Note 1 | N/A | N/A | N/A |
|  | No | Yes (non-cont) | DC_XA-nY(2A)-nZA | Test needed (if max NR CC) – anchor agnostic | Test needed (if max NR CC) | Test needed (if max NR CC) |
|  | No | Yes (inter) | DC_XA-nYA-nZC | Test needed (if max NR CC) – anchor agnostic | Test needed (if max NR CC) | Test needed (if max NR CC) |
|  | Yes (all types) | No | DC_XD-nYA, DC_XA-YC_nZA, DC_XA-XA-YA-nZA, DC_XA-YA-ZA_nRA | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested | No test needed. 3CC need to be tested if IMD(3band) |
|  | Yes (all types) | Yes (cont) | DC_XC_nYC, DC_XA-XA_nYC, DC_XA-YA_nYC | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC |
|  |  | Yes (non-cont) | DC_XC_nY(2A), DC_XC_nY(2A), DC_XA-YA_nY(2A) | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC |
|  |  | Yes (inter) | DC_XC-nYA-nZA, DC_XA-XA_nYA-nZA, DC_XA-YA_nZA-nRA | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC or IMD(3band) |
| Intra-band contiguous EN-DC (2-3 band) | No | Yes | Note 1 | N/A |  |  |
|  | Yes | No | DC_XA-YA_(n)ZAA, DC_XC_(n)YAA | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested |
|  | Yes | Yes | DC_(n)XCA-nYA | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC |
| Intra-band non contiguous EN-DC (2-3 band) | No | Yes | Note 1 | N/A |  |  |
|  | Yes | No | DC_XA-YA-ZA_nZA | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested | No test needed. Only 2CC need to be tested |
|  | Yes | Yes | DC_XA-YA_nYA-nZA | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC | No test needed. 3CC need to be tested if max NR CC or IMD(3band) |
| NOTE 1: No such config in TS 38.101-3 [7] V16.5.0 NOTE 2: 2CC BW class C is indicated in the table. The same rules apply to BW class B NOTE 3: Different test coverage is indicated by color coding in this table. |  |  |  |  |  |  |

## D.2.7	Test Environment

The test environment for FR1 EN-DC RX sensitivity measurement is Normal, TL/VL, TL/VH, TH/VL, TH/VH (same as for NR CA).

## D.2.8	Test Frequencies selections for EN-DC

In E-UTRA DL CA and UL CA reference sensitivity testing, Low range and High range are selected for intra-band CA testing. Mid range is selected for inter band E-UTRA CA testing.

In standalone NR CA reference sensitivity testing, Low and High Range are tested as default for intra-band and Mid range for inter-band. For CA combinations containing intra-band configuration in an inter-band CA configuration, low and high range are tested for intra-band CA and mid channel for inter-band without intra-band component.

The same principle shall apply for EN-DC testing.

For EN-DC configurations affected by exceptions the test frequency cannot be freely chosen. One test frequency per exception requirement is sufficient to test the requirement. This is indicated in Table D.2.8-1. Exceptions in TS 38.101-3 clause 7.3A.5 (2UL intermodulation) are not specified since the test frequency and channel bandwidth is already specified in TS 38.101-1.

Table D.2.8-1 Test frequency selection per band pair for exceptions avoidable by test frequency setting (UL harmonics, Rx mixing, cross band isolation).

| Band set (Note 2) | Frequency | Channel BW [MHz] | Exception type | Comments |
| --- | --- | --- | --- | --- |
| 1A / n3A | Mid/Mid | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | Low/High | 20 / 40 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 3A / n1A | Mid/Mid | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | High/Low | 20 / 20 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 3A / n41A |  |  |  | CMCC CR R5-205929 |
| 3A / n78A | Mid/High | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | Mid/3495 | Highest/Highest | UL Harmonic Interference | Exception Note 2 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | Mid/3525 | 20/20 | UL Harmonic Interference | Exception Note 3 of TS 38.101-3 table 7.3B.2.3.1-1 |
| 3A / n78A | High /Mid | 20/20 | Receiver Harmonic Mixing Exception | Receiver Harmonic Mixing Exception in TS 38.101-3 Table 7.3B.2.3.2-1 |
| 11A / n79A | Mid (DL 1485.9 MHz) / 4457.7 | Highest (10 MHz) / Highest (100 MHz) | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | Mid (DL 1485.9 MHz) / 4567.7 MHz | Highest (10 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=20+100 MHz, FINT = (120+100)/2 = 110 |
| 20A / n78A | Mid/Mid | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | Mid/3388 | Highest/Highest | UL Harmonic Interference | Exception Note 7 of TS 38.101-3 table 7.3B.2.3.1-1 |
| 40A / n78A | Low/Mid | Highest/Highest | - | Non-Exception. To be used in test cases 7.3B.2.3_1.0 to 7.3B.2.3_1.2 |
|  | 2366.6/Mid | 20/20 | Receiver Harmonic Mixing Exception | Receiver Harmonic Mixing Exception Note 8 in TS38.101-3 Table 7.3B.2.3.2-1 |
| 66A / n78A |  |  |  | Qualcomm CR R5-206142 |
| NOTE 1: This selection is used in test case 7.3B.2.3 unless otherwise stated NOTE 2: Aggressor band in bold |  |  |  |  |  |

For EN-DC configurations affected by exceptions where the exception cannot be avoided and hence always apply, the test frequency and bandwidth are defined by RAN4, and not repeated here. There are no reference sensitivity requirements for other frequency/bandwidth.

## D.2.8	Test Channel Bandwidth selections for EN-DC

### D.2.8.1	Test point selection EN-DC configuration without exception

Follow similar arguments for frequency points selection, for regular inter-band EN-DC testing, the NR channel bandwidths shall follow what is specified in SA scenario.  Since the objective is to test NR performance, it is sufficient to select 5 MHz channel bandwidth for E-UTRA carrier, which is common for all E-UTRA bands.

### D.2.8.2	Test point selection EN-DC configuration without exception

For inter-band EN-DC configuration with exception requirement due to UL harmonics, Rx mixing and cross band isolation, maximum aggregated BW shall be tested.

For inter-band EN-DC configuration with exception requirement due to 2UL intermodulation, the test channel bandwidth selection for both NR and E-UTRA are EN-DC combo dependent. There are fixed channel bandwidth pairs required for these exceptional test scenarios.

For intra-band EN-DC configuration with exception requirement, the test channel bandwidth selection for both NR and E-UTRA are EN-DC combo dependent. There are fixed channel bandwidth pairs required for these exceptional test scenarios.

Only 5 MHz channel bandwidth shall be employed by E-UTRA band, Lowest, Mid, Highest channel bandwidth shall be selected for NR carrier for EN-DC non- exceptional testing.

For testing of exceptional test scenarios for EN-DC RX sensitivity due to UL harmonics, Rx mixing and cross band isolation, channel bandwidth selection for both NR and E-UTRA carriers shall be maximum aggregated BW.

For testing of exceptional test scenarios for EN-DC RX sensitivity due to 2UL intermodulation, channel bandwidth selection for both NR and E-UTRA carriers shall follow the value given in the core spec.

## D.2.9	RB allocation and RB location selections

### D.2.9.1	Test point selection non-exceptional case

Following the E-UTRA anchor-agnostic proposed in R5-185916 [6], E-UTREA operation does not interfere with NR connection, 0 RB were proposed for both UL and DL channels for non-exceptional test scenarios.

There is no reason to deviate from current NR configurations for RB allocation selection employed in standalone testing.

### D.2.9.2	Test point selection exceptional case

For EN-DC inter-band and intra-band EN-DC exceptional test scenarios, the RB allocation for both NR and E-UTRA are selected as specified in TS 38.101-3 [7] for the EN-DC band combination.

Based on the E-UTRA anchor-agnostic approach, 0 RB shall be used for both UL and DL channels for non-exceptional test scenarios. RB allocation for NR carrier in inter-band EN-DC non- exceptional testing shall follow what is selected for standalone testing.

For testing of exceptional test scenarios for inter-band EN-DC RX sensitivity, the RB allocation selection for both NR and E-UTRA carriers shall follow the values provided in the spec.

### D.2.10	Modulation scheme selections

The modulation scheme for both non-exceptional and exceptional EN-DC intra-band and inter-band test scenarios are selected as:

-	For E-UTRA: QPSK (same as used for E-UTRA reference sensitivity testing in TS 36-521-1 [9])

-	For NR: Use same modulation scheme as used for NR standalone testing.

### D.2.11	Current test completion status per EN-DC configuration

The completion status per EN-DC configuration is documented in clause 4.3.3.1.