3GPP TSG-RAN5 Meeting #90-e	R5-211165

# Electronic Meeting, 22nd Feb 2021 – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1781 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of support for URLLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no support for URLLC related test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of URLLC is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for URLLC will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	PDU session establishment accept

Table 4.7.2-2: PDU SESSION ESTABLISHMENT ACCEPT

| Derivation Path: 24.501 clause 8.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI |  | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PDU SESSION ESTABLISHMENT ACCEPT message identity |  | ‘1100 0010’B |  |  |
| Selected PDU session type |  | '001'B |  | IPv4 |
|  |  | '010'B |  | IPv6 |
|  |  | '011'B |  | IPv4v6 |
| Selected SSC mode |  | ‘001’B | SSC mode 1 |  |
| Authorized QoS rules |  | 5GC QoS rule of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| Session AMBR |  |  |  |  |
| Unit for Session-AMBR for downlink |  | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for downlink |  | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| Unit for Session-AMBR for uplink |  | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for uplink |  | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| 5GSM cause |  | Not Present |  |  |
| PDU address |  |  |  | IPv4 |
| Length of PDU address contents |  | 5 octets |  |  |
| PDU type value |  | ‘001’B | IPv4 |  |
| PDU address information |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDU address |  |  |  | IPv6 |
| Length of PDU address contents |  | 9 octets |  |  |
| PDU type value |  | ‘010’B | IPv6 |  |
| PDU address information |  | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDU address |  |  |  | IPv4v6 |
| Length of PDU address contents |  | 13 octets |  |  |
| PDU type value |  | ‘011’B | IPv4v6 |  |
| PDU address information (Octets 4 to 11) |  | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDU address information (Octets 12 to 15) |  | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  |  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| RQ timer value |  | Not Present |  |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘0000 0001’B | SST |  |
| SST |  | ‘0000 0001’B | SST value 1 (eMBB) |  |
| SST |  | ‘0000 0010’B | SST value 2 (URLLC) | SST_URLLC |
| SST |  | ‘0000 0011’B | SST value 3 (MIoT) | SST_MIoT |
| SST |  | ‘0000 0100’B | SST value 4 (V2X) | SST_V2X |
| Always-on PDU session indication |  | Not Present |  |  |
| Always-on PDU session indication |  |  |  |  |
| APSI |  | ‘0’B | Always-on PDU session not allowed | Always_On_Requested |
| APSI |  | ‘1’B | Always-on PDU session required | Always_On_Requested AND SST_URLLC |
| Mapped EPS bearer contexts |  | Not Present |  |  |
| Mapped EPS bearer contexts |  |  |  | Interworking_with_EPS |
| Mapped EPS bearer context |  |  |  |  |
| EPS bearer identity |  | The same value as the one specified in the Reference QoS flow referred to from the Reference QoS rule indicated in the IE Authorized QoS rules |  |  |
| Operation code |  | ‘001’B | Create new EPS bearer |  |
| E bit |  | '1'B | Parameters list is included |  |
| Number of EPS parameters |  | ’0001’B | 1 parameter |  |
| Mapped EPS QoS parameters |  | EPC default bearer context of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| EAP message |  | Not Present |  |  |
| Authorized QoS flow descriptions |  | The QoS flow referred to in the relevant Authorized QoS rules IE |  |  |
| Extended protocol configuration options |  | Not Present |  |  |
| Extended protocol configuration options |  |  |  | P-CSCF_IPv6 OR P-CSCF_IPv4 |
| Container ID 1 |  | ‘0001’H |  | P-CSCF_IPv6 |
| Length of container ID 1 contents |  |  | Length value determined by testimplementation |  |
| Container ID 1 contents |  | IPv6 address | P-CSCF IPv6 Address |  |
| Container ID 2 |  | ‘000C’H |  | P-CSCF_IPv4 |
| Length of container ID 2 contents |  |  | Length value determined by test implementation |  |
| Container ID 2 contents |  | IPv4 address | P-CSCF IPv4 Address |  |
| DNN |  | The DNN/APN ID of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| 5GSM network feature support |  | Not Present |  |  |
| Serving PLMN rate control |  | Not Present |  |  |
| ATSSS container |  | Not Present |  |  |
| Control plane only indication |  | Not Present |  |  |
| IP header compression configuration |  | Not Present |  |  |
| Ethernet header compression configuration |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '001'B |
| IPv6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '010'B |
| IPv4v6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '011'B |
| IPv4-DHCP | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the IE Extended protocol configuration options contains a configuration protocol option = '000BH' ("IPv4 address allocation via DHCPv4", length of contents = 0). Note: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the "PDU address information" just above. |
| Always_On_Requested | If the last PDU SESSION ESTABLISHMENT REQUEST message included the Always-on PDU session requested IE |
| P-CSCF_IPv6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message the IE Extended protocol configuration options contains a configuration protocol option = '0001H' (“P-CSCF IPv6 Address Request", length of contents = 0) |
| P-CSCF_IPv4 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message the IE Extended protocol configuration options contains a configuration protocol option = '000CH' (“P-CSCF IPv4 Address Request", length of contents = 0) |
| Interworking_with_EPS | If the UE has indicated support of S1, then the SS shall include this IE to provide details for the interworking with EPS being supported for a PDU session. This requirement is set up for the purpose of facilitating the test description. It is not mandatory for the Network to support Mapped EPS bearer contexts. |
| SST_URLLC | Slice suitable for the handling of ultra- reliable low latency communications. |
| SST_MIoT | Slice suitable for the handling of massive IoT |
| SST_V2X | Slice suitable for the handling of V2X services. |

<End of modified section>

<Start of next modified section>

#### –	PDU session modification command

Table 4.7.2-9: PDU SESSION MODIFICATION COMMAND

| Derivation Path: 24.501 clause 8.3.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PDU session ID |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  | UE_Initiated_Modification |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PTI |  | The value indicated in PDU SESSION MODIFICATION REQUEST message. |  | UE_Initiated_Modification |
| PDU SESSION MODIFICATION COMMAND message identity |  | ‘1100 1011’B |  |  |
| 5GSM cause |  | Not Present |  |  |
| Session AMBR |  | Not Present |  |  |
| RQ timer value |  | Not Present |  |  |
| Always-on PDU session indication |  | Not Present |  |  |
| Always-on PDU session indication |  |  |  |  |
| APSI |  | ‘0’B | Always-on PDU session not allowed | Always_On_Requested |
| APSI |  | ‘1’B | Always-on PDU session required | Always_On_Requested AND SST_URLLC |
| Authorized QoS rules |  | Not Present |  |  |
| Mapped EPS bearer contexts |  | Not Present |  |  |
| Authorized QoS flow descriptions |  | Not Present |  |  |
| Extended protocol configuration options |  | Not Present |  |  |
| ATSSS container |  | Not Present |  |  |
| IP header compression configuration |  | Not Present |  |  |
| Port management information container |  | Not Present |  |  |
| Serving PLMN rate control |  | Not Present |  |  |
| Ethernet header compression configuration |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Always_On_Requested | If the last PDU SESSION MODIFICATION REQUEST message included the Always-on PDU session requested IE |
| UE_Initiated_Modification | If this message was triggered by a PDU SESSION MODIFICATION REQUEST message sent by the UE |

<End of modified section>

<Start of next modified section>

### 4.8.4	DNN/APN configurations

The present subclause provides DNN/APN configurations required for flexible PDU/PDN handling. Table 4.8.4-1 provides configurations for the types on DNN/APN handled in the present version of the test specification. If in the future new PDU types need to be handled then new DNN/APN configuration(s) may be added.

Table 4.8.4-1: DNN/APN configurations

| Configurations | Config #1 | Config #2 | Config #3 |
| --- | --- | --- | --- |
| DNN/APN type | internet | ims | FFS |
| DNN/APN ID | pc_APN_ID_Internet | pc_APN_ID_IMS | FFS |
| 5GC QoS rule | Reference QoS rule #1 as specified in subclause 4.8.2.1. | Reference QoS rule #2 as specified in subclause 4.8.2.1. | Reference QoS rule #8 as specified in subclause 4.8.2.1. |
| EPC default bearer context | Reference default EPS bearer context #1 as specified in TS 36.508 [10], Table 6.6.1-1. | Reference default EPS bearer context #2 as specified in TS 36.508 [10], Table 6.6.1-1. | N/A |
| EPC dedicated bearer context | Reference dedicated EPS bearer context #1 as specified in TS 36.508 [10], Table 6.6.2-1. | Reference dedicated EPS bearer context #4 as specified in TS 36.508 [10], Table 6.6.2-1. | N/A |
| IP address allocation | Yes | Yes | Yes |
| IMS registration | No | Yes NOTE 1 | No |
| NOTE 1: For PDN establishment the Procedure for IMS signalling according to TS 36.508 [2], subclause 4.5A.3 applies; for PDU establishment the Procedure for IMS signalling according to TS 34.229-5 [47], Annex A.2 applies. |  |  |  |

Editor’s Note: It is FFS how to determine DNN for URLLC.

<End of modified section>