3GPP TSG-RAN5 Meeting #90-e	R5-211181

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1192 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC1 and PC3 Updates for Band n14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) NR Band n14 PC1 requirements have been updated in the core specification. 2) The PC3 maximum output power requirement for NR Band n14 should not include the deltaTC relaxation. 3) The indication of when UTRAACLR is not applicable for certain NR operating bands has been included in the core specification. 4) The test requirements for configured transmitted power test point 4 are missing for NR Bands n14 and n30. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Updated the associated PC1 requirements and test requirements for NR Band n14. 2) Updated the PC3 maximum output power requirement for NR Band n14 to remove the deltaTC relaxation note and updated the associated test requirements. Removed the corresponding editor’s note in the UE MOP test case. 3) Added the core requirement statement concerning when UTRAACLR is not applicable for certain NR operating bands. 4) Added the test requirements for configured transmitted power test point 4 for NR Bands n14 and n30. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The NR Band n14 PC1 test requirements would not be specified. 2) The PC3 maximum output power test requirement for NR Band 14 would remain incorrect. 3) The test specification would not be clear as to when UTRAACLR is not applicable for certain NR operating bands. 4) The test requirement for configured transmitted power test point 4 for NR Bands n14 and n30 would not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.2.3, 6.2.4, 6.5.2.2, 6.5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged content omitted >>

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

#### 6.2.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

#### 6.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1, 2 and 3 UE release 15 and forward.

#### 6.2.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1.3-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ± 2 |
| n2 |  |  |  |  | 23 | ± 23 |
| n3 |  |  |  |  | 23 | ± 23 |
| n5 |  |  |  |  | 23 | ± 2 |
| n7 |  |  |  |  | 23 | ± 23 |
| n8 |  |  |  |  | 23 | ± 23 |
| n12 |  |  |  |  | 23 | ± 23 |
| n14 | 31 | +2/-3 |  |  | 23 | ± 2 |
| n20 |  |  |  |  | 23 | ± 23 |
| n25 |  |  |  |  | 23 | ± 23 |
| n26 |  |  |  |  | 23 | ± 23 |
| n28 |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  | 23 | ± 2 |
| n34 |  |  |  |  | 23 | ± 2 |
| n38 |  |  |  |  | 23 | ± 2 |
| n39 |  |  |  |  | 23 | ± 2 |
| n40 |  |  |  |  | 23 | ± 2 |
| n41 |  |  | 26 | +2/-33 | 23 | ± 23 |
| n48 |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  | 23 | ± 2 |
| n51 |  |  |  |  | 23 | ± 2 |
| n53 |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  | 23 | ±2 |
| n66 |  |  |  |  | 23 | ± 2 |
| n70 |  |  |  |  | 23 | ± 2 |
| n71 |  |  |  |  | 23 | +2/-2.5 |
| n74 |  |  |  |  | 23 | ± 2 |
| n77 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n78 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 23 | ± 2 |
| n81 |  |  |  |  | 23 | ± 2 |
| n82 |  |  |  |  | 23 | ± 2 |
| n83 |  |  |  |  | 23 | ± 2/-2.5 |
| n84 |  |  |  |  | 23 | ± 2 |
| n86 |  |  |  |  | 23 | ± 2 |
| n95 |  |  |  |  | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: FFS NOTE 5: FFS NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

- if the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%  (The exact evaluation period is no less than one radio frame); or

- if the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in sub-clause 6.2.4;

- else if (the IE P-Max as defined in TS 38.331 [6] is not provided or set to the higher value than the maximum output power of the default power class and the percentage of uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle as defined in TS 38.331; or

the IE P-Max as defined in TS 38.331 [6] is not provided or set to the higher value than the maximum output power of the default power class and the percentage of uplink symbols transmitted in a certain evaluation period is less than or equal to 50% when maxUplinkDutyCycle is absent. The exact evaluation period is no less than one radio frame):

- shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2.4;

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.1.

#### 6.2.1.4	Test description

##### 6.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest (NOTE 3) |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for Ues which supports half Pi BPSK in FR1. NOTE 3: For band n28, the Highest test channel bandwidth is replaced by 20MHz due to MPR is always larger than 0dB for 30MHz bandwidth. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

##### 6.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

4.	For Ues supporting Power Class 2 or Power Class 1, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2.1.4.3-2.

##### 6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.2.1.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1 UE |

#### 6.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1 for Power Class 3, Table 6.2.1.5-2 for Power Class 2, and Table 6.2.1.5-2a for Power Class 1.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.1.5-1.

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±2±TT |
| n2 |  |  |  |  | 23 | ±23±TT |
| n3 |  |  |  |  | 23 | ±23±TT |
| n5 |  |  |  |  | 23 | ±2±TT |
| n7 |  |  |  |  | 23 | ±23±TT |
| n8 |  |  |  |  | 23 | ±23±TT |
| n12 |  |  |  |  | 23 | ±23±TT |
| n14 |  |  |  |  | 23 | ±2±TT |
| n20 |  |  |  |  | 23 | ±23±TT |
| n25 |  |  |  |  | 23 | ±23±TT |
| n26 |  |  |  |  | 23 | ±23±TT |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±2±TT |
| n34 |  |  |  |  | 23 | ±2±TT |
| n38 |  |  |  |  | 23 | ±2±TT |
| n39 |  |  |  |  | 23 | ±2±TT |
| n40 |  |  |  |  | 23 | ±2±TT |
| n41 |  |  |  |  | 23 | ± 23±TT |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±2±TT |
| n51 |  |  |  |  | 23 | ±2±TT |
| n53 |  |  |  |  | 23 | ±2±TT |
| n65 |  |  |  |  | 23 | ±2±TT |
| n66 |  |  |  |  | 23 | ±2±TT |
| n70 |  |  |  |  | 23 | ±2±TT |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±2±TT |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n80 |  |  |  |  | 23 | ±2±TT |
| n81 |  |  |  |  | 23 | ±2±TT |
| n82 |  |  |  |  | 23 | ±2±TT |
| n83 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n84 |  |  |  |  | 23 | ±2±TT |
| n86 |  |  |  |  | 23 | ±2±TT |
| n95 |  |  |  |  | 23 | ±2±TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n14 | 31 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.0.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4] for the applicable operating bands.

### 6.2.2	UE maximum output power reduction

Editor’s Note: The following aspects are either missing or not yet determined:

- PC1 requirements for NR operating bands other than n14 are not defined in RAN4 Rel-15 and Rel-16 specifications.

- Seeking some clarification from RAN4 on how ΔTIB,c in TS 38.101-1 [2] subclause 6.2A.4.2/6.2C.2 and TS 38.101-3 [4] subclause 6.2A.4.2/6.2B.4.2 shall be applied for applicable bands.

- The UE capability [DMRS-pi2BPSK-supported] and the corresponding IE [DMRSPi2BPSK] are still not finally determined by RAN2.

#### 6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

#### 6.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 2 and 3 UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed.

#### 6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 1.5, 2 and 3 and UE power class 1 in n14, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-4, 6.2.2.3-2, Table 6.2.2.3-1 and Table 6.2.2.3-5, respectively for channel bandwidths that meets both following criteria:

-	Channel bandwidth ≤ 100 MHz.

-	Relative channel bandwidth ≤ 4 % for TDD bands and ≤ 3 % for FDD bands. Unless otherwise stated, the ∆MPR is set to zero.

If the relative channel bandwidth is larger than 4% for TDD bands or 3% for FDD bands, the ∆MPR is defined in Table 6.2.2.3-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low  + FUL_high).

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 |  | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 |  | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0dB MPR is 26dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40% of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Table 6.2.2.3-2: Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2.2.3-3: ∆MPR

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 | Power class 3 | 30 MHz | 0.5 |

Table 6.2.2-4 Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6.5 | ≤ 3.5 | ≤ 1.5 |
|  | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 5 | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 6 | ≤ 3 |
|  | 16 QAM | ≤ 6.5 | ≤ 6 | ≤ 3.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 5 |
|  | 256 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1 for n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | ≤ 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High, and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ 2 RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in sub-clause 6.1.2.2 of TS 38.214 [12]. For these almost contiguous signals in power class 2 and 3, the allowed maximum power reduction defined in Table 6.2.2.3-1 is increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameters of RBStart,Low and RBStart,High to specify valid RB allocation ranges for Outer and Inner RB allocations are defined as following:

RBStart,Low = max(1, floor((NRB_alloc + NRB_gap)/2))

RBStart,High = NRB – RBStart,Low – NRB_alloc –NRB_gap

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.2.

#### 6.2.2.4	Test description

##### 6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a and Table 6.2.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Ues supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. |  |  |  |  |

Table 6.2.2.4.1-1a: Void

Table 6.2.2.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports half Pi BPSK in FR1. |  |  |  |  |

Table 6.2.2.4.1-2a: Test Configuration Table for power class 1 for n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports half Pi BPSK in FR1. NOTE 3: Ues supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. |  |  |  |  |

Table 6.2.2.4.1-3: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4. NOTE 2: Test applies only for Ues which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

Table 6.2.2.4.1-4: Uplink configuration for almost contiguous allocation

| Channel Bandwidth（MHz） | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 64@0 | 64@96 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 40 | 15 | CP | 80@0 | 88@128 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 50 | 15 | CP | 96@0 | 110@160 | 48@64 | 48@144 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 64@0 | 66@96 | 32@32 | 16@80 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 89@128 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 96@0 | 101@144 | 32@32 | 16@80 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 112@0 | 97@176 | 48@64 | 48@144 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 , and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a and Table 6.2.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

##### 6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a and Table 6.2.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control “up” commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For Ues supporting Power Class 1 in n14 and Power Class 2, repeat steps 1~3 for Test ID 22 and 36 in Table 6.2.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2.2.4.3-2.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.2.4.1-1 and Table 6.2.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

##### 6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2.2.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1 UE |

Table 6.2.2.4.3-3: DMRS-UplinkConfig (Test ID 37 — 39 in Table 6.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 and Table 6.2.2.5-3.

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n80, n81, n82, n84, n86) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 2.0 |  | 2 | （3.52） | 28.0 + TT | 23.8 — TT | （22.3 — TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 28.0 + TT | 20.5 — TT | （19.0 — TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 28.0 + TT | 20.5 — TT | （19.0 — TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 2.0 |  | 2 | （3.52） | 28.0 + TT | 22.8 — TT | （21.3 — TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 21.0 — TT | （19.5 — TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 — TT | （19.0 — TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 — TT | （19.0 — TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 — TT | （19.0 — TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 21.0 — TT | （19.5 — TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 20.0 — TT | （18.0 — TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 20.0 — TT | （18.0 — TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 20.0 — TT | （18.0 — TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 20.0 — TT | （18.0 — TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | （3.52） | 25.0 + TT | 19.0 — TT | （16.0 — TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | （3.52） | 25.0 + TT | 19.0 — TT | （16.0 — TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | （3.52） | 25.0 + TT | 19.0 — TT | （16.0 — TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | （3.52） | 25.0 + TT | 18.0 — TT | （15.5 — TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | （3.52） | 25.0 + TT | 18.0 — TT | （15.5 — TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | （3.52） | 25.0 + TT | 18.0 — TT | （15.5 — TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | （3.52） | 25.0 + TT | 14.5 — TT | （12.0 — TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | （3.52） | 25.0 + TT | 14.5 — TT | （12.0 — TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | （3.52） | 25.0 + TT | 14.5 — TT | （12.0 — TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 19.5 — TT | （17.5 — TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 — TT | （14.5 — TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 — TT | （14.5 — TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 — TT | （14.5 — TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | （3.52） | 25.0 + TT | 19.0 — TT | （16.0 — TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 — TT | （14.5 — TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 — TT | （14.5 — TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 — TT | （14.5 — TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 16.0 — TT | （14.0 — TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 16.0 — TT | （14.0 — TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 16.0 — TT | （14.0 — TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 — TT | （10.0 — TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 — TT | （10.0 — TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 — TT | （10.0 — TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 — TT | （19 — TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 — TT | （19 — TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 21 — TT | （19.5 — TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71, n83) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 — TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 — TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 — TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 — TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 — TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 — TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 — TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 — TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 — TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 — TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 — TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 — TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 — TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 — TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 — TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 — TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 — TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 — TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 — TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 — TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 — TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 — TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 — TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 — TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 — TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 — TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 — TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 — TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 — TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 — TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 — TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 — TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 — TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 — TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2a: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 — TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 — TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-3: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | 25.8 | 2.0 | 3 | 28.0 + TT | 22.8 — TT |
| 2 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 — TT |
| 3 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 — TT |
| 4 | 23 | -3 | 1.2 | 0 | 24.8 | 2.0 | 3 | 28.0 + TT | 21.8 — TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 — TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 — TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 — TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 — TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 — TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 — TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 — TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 — TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 — TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 — TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 — TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 — TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 — TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 — TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 — TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 — TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 — TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 — TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 3 | 25.0 + TT | 18.5 — TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 — TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 — TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 — TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 — TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 — TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 — TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 — TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 — TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 — TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 — TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 — TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 — TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 — TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 — TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 — TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 3 | 25.0 + TT | 20.0 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4: UE Power Class test requirements (for Bands n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 23.0 — TT | （21.5 — TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 22.5 — TT | （21.0 — TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 23.0 — TT | （21.5 — TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 22.0 — TT | （20.5 — TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 22.0 — TT | （20.5 — TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 21.0 — TT | （19.5 — TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 20.5 — TT | （19.0 — TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | （4.52） | 28.0 + TT | 18.5 — TT | （17.0 — TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | （4.52） | 28.0 + TT | 18.5 — TT | （17.0 — TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | （4.52） | 28.0 + TT | 18.5 — TT | （17.0 — TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 21.5 — TT | （20.0 — TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 20.0 — TT | （18.5 — TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 21.0 — TT | （19.5 — TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 20.0 — TT | （18.5 — TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 — TT | （18.0 — TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | （4.52） | 28.0 + TT | 16.0 — TT | （14.0 — TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | （4.52） | 28.0 + TT | 16.0 — TT | （14.0 — TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | （4.52） | 28.0 + TT | 16.0 — TT | （14.0 — TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4a: UE Power Class test requirements for Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 2 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 3 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 4 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 5 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 6 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 7 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 8 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 9 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 10 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 11 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 12 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 13 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 14 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 15 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 16 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 17 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 18 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 19 | 31 | 0 | 1.5 | 0 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 — TT |
| 20 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 21 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 22 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 23 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 24 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 25 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 26 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 27 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 28 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 29 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 30 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 31 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 32 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 33 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 34 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 35 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.2.5-6: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74, n80, n81, n82, n84, n86) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 3.52 | 25.0 + TT | 17.5 — TT | 14.5 — TT2 |
| 15 | 23 | 0 | 1.5 | 1 | 0 | 1.52 | 20.5 | 192 | 2.5 | 3.52 | 2 | 3.52 | 25.0 + TT | 18 — TT | 15.5 — TT2 |
| 26 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 3.52 | 25.0 + TT | 14.5 — TT | 12 — TT2 |
| 27 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 3.52 | 25.0 + TT | 15.5 — TT | 12.5 — TT2 |
| 34 | 23 | 0 | 2 | 1.5 | 0 | 1.52 | 19.5 | 182 | 3.5 | 42 | 2 | 3.52 | 25.0 + TT | 16 — TT | 14 — TT2 |
| 35 | 23 | 0 | 2 | 1 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 3.52 | 25.0 + TT | 17.5 — TT | 14.5 — TT2 |
| 46 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 3.52 | 25.0 + TT | 14.5 — TT | 12 — TT2 |
| 47 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 3.52 | 25.0 + TT | 15.5 — TT | 12.5 — TT2 |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 2 | 3.52 | 25.0 + TT | 14 — TT | 11.5 — TT2 |
| 57 | 23 | 0 | 3.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 3.52 | 25.0 + TT | 14.5 — TT | 12 — TT2 |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | 1.52 | 15 | 13.52 | 5 | 52 | 2 | 3.52 | 25.0 + TT | 10 — TT | 8.5- TT2 |
| 67 | 23 | 0 | 6.5 | 1 | 0 | 1.52 | 15.5 | 142 | 5 | 52 | 2 | 3.52 | 25.0 + TT | 10.5 — TT | 9 — TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-6a: Void

Table 6.2.2.5-7: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71, n83) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR ncrease (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 — TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18 — TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 — TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 — TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16 — TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 — TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 — TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 — TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 — TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 — TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 — TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 2.5 | 25.0 + TT | 10.5 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-7a: Void

Table 6.2.2.5-7b: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR ncrease (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 — TT |
| 24 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 — TT |
| 43 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 — TT |
| 44 | 23 | 0 | 3.5 | 1 | 0 | 17.5 | 4 | 2.5 | 25.0 + TT | 14.5 — TT |
| 53 | 23 | 0 | 4 | 1.5 | 0 | 17.5 | 5 | 2.5 | 25.0 + TT | 12.5 — TT |
| 54 | 23 | 0 | 4 | 1 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 — TT |
| 63 | 23 | 0 | 7 | 1.5 | 0 | 14.5 | 5 | 2.5 | 25.0 + TT | 9.5 — TT |
| 64 | 23 | 0 | 7 | 1 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW=30MHz when SCS=60kHz. NOTE 4: Applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz when SCS=30kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR ncrease (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 — TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 — TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 — TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 — TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16 — TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 — TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 — TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 — TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 3 | 25.0 + TT | 14 — TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 — TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 3 | 25.0 + TT | 10 — TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 3 | 25.0 + TT | 10.5 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Test applies only for Ues which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8a: Void

Table 6.2.2.5-9: UE Power Class test requirements (for Bands n41, n77, n78, n79) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 1.5 | 1.5 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 4.52 | 28.0 + TT | 20 — TT | 17 — TT2 |
| 15 | 26 | 0 | 1.5 | 1 | 0 | 1.52 | 23.5 | 222 | 2 | 22 | 3 | 4.52 | 28.0 + TT | 20.5 — TT | 17.5 — TT2 |
| 26 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 18.5 — TT | 15.5 — TT2 |
| 27 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 19 — TT | 16 — TT2 |
| 34 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 212 | 2 | 22 | 3 | 4.52 | 28.0 + TT | 19.5 — TT | 16.5 — TT2 |
| 35 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 4.52 | 28.0 + TT | 20 — TT | 17 — TT2 |
| 46 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 18.5 — TT | 15.5 — TT2 |
| 47 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 19 — TT | 16 — TT2 |
| 56 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21 | 19.52 | 2 | 3.52 | 3 | 4.52 | 28.0 + TT | 18 — TT | 15 — TT2 |
| 57 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 18.5 — TT | 15.5 — TT2 |
| 66 | 26 | 0 | 6.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 3 | 4.52 | 28.0 + TT | 14 — TT | 11.5 — TT2 |
| 67 | 26 | 0 | 6.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 3 | 4.52 | 28.0 + TT | 14.5 — TT | 12 — TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for Ues which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-9a: Void

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_10, NS_07, NS_40, NS_09 and NS_49 not complete.

- PC1 requirements for NR operating bands other than n14 are not fully defined in RAN4 Rel-15 and Rel-16 specifications due to the lack of MPR requirements.

## << Unchanged content omitted >>

### 6.2.4	Configured transmitted power

#### 6.2.4.1	Test purpose

To verify the measured UE configured maximum output power PUMAX,f,c is within the specified bounds.

#### 6.2.4.2	Test applicability

This test case applies to all types of NR Power Class 2 and 3 UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

#### 6.2.4.3	Minimum conformance requirements

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1.3-1 without taking into account the tolerance specified in the Table 6.2.1.3-1;

When the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 capable UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 capable UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass = 3 dB for a power class 2 capable UE, when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); otherwise ΔPPowerClass = 0 dB;

∆TIB,c is the additional tolerance for serving cell c as specified in 6.2A.4.0.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 [4] clause 6.2B.4.2 for EN-DC ;  = 0 dB otherwise; In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.0.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [4] for the applicable operating bands.

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise ∆TC,c  = 0dB;

MPRc and A-MPRc for serving cell c are specified in subclause 6.2.2.3 and subclause 6.2.3.3, respectively; ∆MPRc for serving cell c is specified in clause 6.2.2.

∆TRxSRS is applied when UE transmits SRS to other than first SRS port when the SRS-TxSwitch capability is indicated as  ‘1T2R’, ‘1T4R’ or, ‘1T4R/2T4R’ with UE configured with 4 SRS resources in the SRS resource set, and when UE transmits SRS to other than first or second SRS port when the SRS-TxSwitch capability is indicated as  ‘2T4R’ or ‘1T4R/2T4R’ with the UE configured with 2 SRS resources in the SRS resource set. The value of ∆TRxSRS is 4.5dB for n79 and 3 dB for bands whose FUL_high is lower than the FUL_low  of n79.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self defence requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4.3-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4.3-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4.3-2. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1.3-1.

Table 6.2.4.3-2: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.4.

#### 6.2.4.4	Test description

##### 6.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| 33 |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 PBSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: For NR band n28, 30MHz test channel bandwidth is tested with High range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.4.4.3.

##### 6.2.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE reaches the Pumax level of the test point.

3.	Measure the mean power of the UE in the channel bandwidth for each test point in table 6.2.4.5-1 according to the test configuration from table 6.2.4.4.1-1. The period of measurement shall be at least the continuous duration of one active slot and in the uplink symbols. For TDD slots with transient periods are not under test.

##### 6.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.2.4.4.3-1: FrequencyInfoUL-SIB: Test point 1

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2.4.4.3-2: FrequencyInfoUL-SIB: Test point 2

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2.4.4.3-3: FrequencyInfoUL-SIB: Test point 3

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

Table 6.2.4.4.3-4: FrequencyInfoUL-SIB: Test point 4

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  |  |

Table 6.2.4.4.3-5: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  | Test ID 1, 2 |
|  | 1 |  | Test ID 3 |
| } |  |  |  |
| } |  |  |  |

#### 6.2.4.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2.4.5-1.

Table 6.2.4.5-1: PCMAX configured UE output power

|  | Maximum output power |  |
| --- | --- | --- |
|  | Test ID 1,2 | Test ID 3 |
| Measured UE output power test point 1 | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) |
| Measured UE output power test point 2 | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) |
| Measured UE output power test point 3 | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) |
| Measured UE output power test point 4 | Note 3 | Note 4 |
| Note 1: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. Note 2: Power class 3 is default power class unless otherwise stated. Note 3: The maximum output power shall be within the range in Table 6.2.4.5-1a. Note 4: The maximum output power shall be within the range in Table 6.2.4.5-1b. |  |  |

Table 6.2.4.5-1a: Measured UE output power test point 4 for Test ID 1,2

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 20 dBm ±(2.5+TT) |
| n2 | 20 dBm ±(2.51+TT) |
| n3 | 20 dBm ±(2.51+TT) |
| n5 | 20 dBm ±2.5±TT |
| n7 | 20 dBm ±(2.51+TT) |
| n8 | 20 dBm ±(2.51+TT) |
| n12 | 20 dBm ±(2.51+TT) |
| n14 | 20 dBm ±(2.5+TT) |
| n20 | 20 dBm ±(2.51+TT) |
| n25 | 20 dBm ±(2.5+TT) |
| n26 | 20 dBm ±(2.51+TT) |
| n28 | 20 dBm ±(2.5+TT) |
| n30 | 20 dBm ±(2.5+TT) |
| n34 | 20 dBm ±(2.5+TT) |
| n38 | 20 dBm ±(2.5+TT) |
| n39 | 20 dBm ±(2.5+TT) |
| n40 | 20 dBm ±(2.5+TT) |
| n41 | 20 dBm ±(2.51+TT) |
| n50 | 20 dBm ±(2.5+TT) |
| n51 | 20 dBm ±(2.5+TT) |
| n65 | 20 dBm ±(2.5+TT) |
| n66 | 20 dBm ±(2.5+TT) |
| n70 | 20 dBm ±(2.5+TT) |
| n71 | 20 dBm ±(2.5+TT) |
| n74 | 20 dBm ±(2.5+TT) |
| n77 | 20 dBm + 2.5+TT/-3-TT |
| n78 | 20 dBm + 2.5+TT/-3-TT |
| n79 | 20 dBm + 2.5+TT/-3-TT |
| n80 | 20 dBm ±(2.5+TT) |
| n81 | 20 dBm ±(2.5+TT) |
| n82 | 20 dBm ±(2.5+TT) |
| n83 | 20 dBm ±(2.5+TT) |
| n84 | 20 dBm ±(2.5+TT) |
| n86 | 20 dBm ±(2.5+TT) |
| n95 | 20 dBm ±(2.5+TT) |
| NOTE 1: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.0 dB. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-1b: Measured UE output power test point 4 for Test ID 3

| NR band | Tolerance (dB) |
| --- | --- |
| n1 | 23 dBm ±(2+TT) |
| n2 | 23 dBm ±(21+TT) |
| n3 | 23 dBm ±(21+TT) |
| n5 | 23 dBm ±(2+TT) |
| n7 | 23 dBm ±(21+TT) |
| n8 | 23 dBm ±(21+TT) |
| n12 | 23 dBm ±(21+TT) |
| n20 | 23 dBm ±(21+TT) |
| n25 | 23 dBm ±(2+TT) |
| n26 | 23 dBm ±(21+TT) |
| n28 | 23 dBm +2+TT/-2.5-TT |
| n34 | 23 dBm ±(2+TT) |
| n38 | 23 dBm ±(2+TT) |
| n39 | 23 dBm ±(2+TT) |
| n40 | 23 dBm ±(2+TT) |
| n41 | 23 dBm ±(21+TT) |
| n50 | 23 dBm ±(2+TT) |
| n51 | 23 dBm ±(2+TT) |
| n65 | 23 dBm ±(2+TT) |
| n66 | 23 dBm ±(2+TT) |
| n70 | 23 dBm ±(2+TT) |
| n71 | 23 dBm +2+TT/-2.5-TT |
| n74 | 23 dBm ±(2+TT) |
| n77 | 23 dBm +2+TT/-3-TT |
| n78 | 23 dBm +2+TT/-3-TT |
| n79 | 23 dBm +2+TT/-3-TT |
| n80 | 23 dBm ±(2+TT) |
| n81 | 23 dBm ±(2+TT) |
| n82 | 23 dBm ±(2+TT) |
| n83 | 23 dBm +2+TT/-2.5-TT |
| n84 | 23 dBm ±(2+TT) |
| n86 | 23 dBm ±(2+TT) |
| n95 | 23 dBm ±(2+TT) |
| NOTE 1: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.4.5-2. |  |

Table 6.2.4.5-2: Test Tolerance (Configured transmitted power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB |

## << Unchanged content omitted >>

### 6.5.2	Out of band emission

#### 6.5.2.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2.2	Spectrum Emission Mask

Editor’s Note: The following aspects are either missing or not yet determined:

- The UE capability [DMRS-pi2BPSK-supported] and the corresponding IE [DMRSPi2BPSK] are still not finally determined by RAN2.

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in subclause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

##### 6.5.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth.

##### 6.5.2.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

##### 6.5.2.2.3	Minimum conformance requirements

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2.3-1 for the specified channel bandwidth.

Table 6.5.2.2.3-1: General NR spectrum emission mask

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 |  | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 |  | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 |  | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.2

##### 6.5.2.2.4	Test description

6.5.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5.2.2.4.1-1, 6.5.2.2.4.1-2, and 6.5.2.2.4.1-2a. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Default Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | Low range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Freq |  | ChBw |  | SCS |  | Downlink Configuration |  |  | Uplink Configuration |  |  |
|  |  |  | Default |  | Default |  | N/A for Spectrum Emission Mask test case |  |  | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 13 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 23 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 33 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer_Full |
| 4 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  | Edger_1RB_Left |
| 5 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  | Outer_Full |
| 7 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  | Outer_Full |
| 10 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  | Outer_Full |
| 13 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  | Outer_Full |
| 16 | Low |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  | Outer_Full |
| 19 | Low |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  | Outer_Full |
| 22 | Low |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  | Outer_Full |
| 25 | Low |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  | Outer_Full |
| 284,5 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 294,5 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 304,5 |  | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For Power Class 3 testing, include two steps for UE operating in bands n40, n41, n77, n78 and n79, with IE powerBoostPi2BPSK set to 1 and 0 separately. NOTE 4: For Power Class 3 testing, UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 5: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5.2.2.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: It is essential that all test points in this table also exist in table 6.2.2.4.1-2. |  |  |  |  |

Table 6.5.2.2.4.1-2a: Test Configuration Table for power class 1 for n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Spectrum Emission Mask test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. NOTE 4: It is essential that all test points in this table also exist in table 6.2.2.4.1-2. |  |  |  |  |

Table 6.5.2.2.4.1-3: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.5.2.2.4.1-4. NOTE 2: It is essential that all test points in this table also exist in table 6.2.2.4.1-3. NOTE 3: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

Table 6.5.2.2.4.1-4: Uplink configuration for almost contiguous allocation

| Channel Bandwidth（MHz） | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 64@0 | 64@96 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 40 | 15 | CP | 80@0 | 88@128 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 50 | 15 | CP | 96@0 | 110@160 | 48@64 | 48@144 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 64@0 | 66@96 | 32@32 | 16@80 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 89@128 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 96@0 | 101@144 | 32@32 | 16@80 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 112@0 | 97@176 | 48@64 | 48@144 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5.2.2.4.3

6.5.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.2.4.1-1, Table 6.5.2.2.4.1-2, Table 6.5.2.2.4.1-2a and Table 6.5.2.2.4.1-3. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.2.5-1 to 6.2.2.5-9. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5.2.2.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, table 6.5.2.2.4.1-2, and table 6.5.2.2.4.1-2a, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.5.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.5.2.2.4.3-2: DMRS-UplinkConfig (Test ID 28 – 30 in Table 6.5.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 6.5.2.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.2.5-1 to 6.2.2.5-9 as appropriate, and the power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1.

Table 6.5.2.2.5-1: General NR spectrum emission mask

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  |  | -24 + TT | -24 + TT | -24 + TT | -24 + TT | -24 + TT | 30 kHz |
| ± 1-5 |  | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | -10 + TT | 1 MHz |
| ± 5-6 |  | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT | -13 + TT |  |
| ± 6-10 |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |  |
| ± 65-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 + TT |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5.2.2.5-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

## << Unchanged content omitted >>

#### 6.5.2.4	Adjacent channel leakage ratio

Adjacent channel leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

##### 6.5.2.4.1	NR ACLR

Editor’s Note: The following aspects are either missing or not yet determined:

- The UE capability [DMRS-pi2BPSK-supported] and the corresponding IE [DMRSPi2BPSK] are still not finally determined by RAN2.

6.5.2.4.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5.2.4.1.2	Test applicability

This test case applies to all types of NR Power Class 2 and 3 UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

6.5.2.4.1.3	Minimum conformance requirements

NR adjacent channel leakage power ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1.3-1.

If the measured adjacent channel power is greater than –50dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1.3-2.

Table 6.5.2.4.1.3-1: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |

Table 6.5.2.4.1.3-2: NR ACLR requirement

|  | Power class 11 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB1 | 31 dB | 31 dB | 30 dB |
| NOTE 1: Applicable for power class 1 UE operating in n14. |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.4.1.

6.5.2.4.1.4	Test description

6.5.2.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5.2.4.1.4.1-1, 6.5.2.4.1.4.1-2, and 6.5.2.4.1.4.1-2a. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5.2.4.1.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full |
| 23 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 33 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 43 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 54 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Inner_Full |
| 64 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 74 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 84 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 9 | Default |  |  |  | DFT-s-OFDM QPSK | Inner_Full |
| 10 | Low |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 13 | Default |  |  |  | DFT-s-OFDM 16 QAM | Inner_Full |
| 14 | Low |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 17 | Low |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 18 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 20 | Low |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  |  |  | DFT-s-OFDM 256 QAM | Outer_Full |
| 23 | Default |  |  |  | CP-OFDM QPSK | Inner_Full |
| 24 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 27 | Default |  |  |  | CP-OFDM 16 QAM | Inner_Full |
| 28 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 31 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 34 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| 374,6 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,6 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,6 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For Power Class 3 testing, UE operating in TDD mode with PI/2 BPSK modulation, and UE indicating support for UE capability powerBoosting-pi2BPSK, the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: For Power Class 3 testing, UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 5: It is essential that all test points in this table also exist in table 6.2.2.4.1-1. NOTE 6: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. |  |  |  |  |  |  |

Table 6.5.2.4.1.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: It is essential that all test points in this table also exist in table 6.2.2.4.1-2. |  |  |  |  |

Table 6.5.2.4.1.4.1-2a: Test Configuration Table for power class 1 for n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Adjacent Channel Leakage Ratio test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. NOTE 4: It is essential that all test points in this table also exist in table 6.2.2.4.1-2. |  |  |  |  |

Table 6.5.2.4.1.4.1-3: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.5.2.4.1.4.1-4. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

Table 6.5.2.4.1.4.1-4: Uplink configuration for almost contiguous allocation

| Channel Bandwidth（MHz） | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 64@0 | 64@96 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 40 | 15 | CP | 80@0 | 88@128 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 50 | 15 | CP | 96@0 | 110@160 | 48@64 | 48@144 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 64@0 | 66@96 | 32@32 | 16@80 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 89@128 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 96@0 | 101@144 | 32@32 | 16@80 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 112@0 | 97@176 | 48@64 | 48@144 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.4.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5.2.4.1.4.3.

6.5.2.4.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.4.1.4.1-1, Table 6.5.2.4.1.4.1-2, Table 6.5.2.4.1.4.1-2a and Table 6.5.2.4.1.4.1-3. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.2.5-1 to 6.2.2.5-9 as appropriate. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the rectangular filtered mean power for the assigned NR channel.

5.	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR channel, respectively.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

7.	For UEs supporting Power Class 1 in n14 and Power Class 2, repeat steps 1~6 for Test ID 22 and 36 in Table 6.5.2.4.1.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.5.2.4.1.4.3-1.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.4.1.4.1-1, 6.5.2.4.1.4.1-2, and table 6.5.2.4.1.4.1-2a, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5.2.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.5.2.4.1.4.3-1: P-Max (Step 7)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE |

Table 6.5.2.4.1.4.3-2: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.5.2.4.1.4.3-3: DMRS-UplinkConfig (Test ID 37 – 39 in Table 6.5.2.4.1.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.2.4.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.2.5-1 to 6.2.2.5-9 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6, shall be higher than the limits in Table 6.5.2.4.1.5-2.

The measured UE mean power in the channel bandwidth, derived in step 7, shall fulfil power class 3 requirements in Tables 6.2.2.5-1 and 6.2.2.5-3 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 7, shall be higher than the power class 3 limits in Table 6.5.2.4.1.5-2.

Table 6.5.2.4.1.5-1: NR ACLR measurement bandwidth

| NR channel bandwidth / NR ACLR measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| NR ACLR measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 78.15 | 88.23 | 98.31 |

Table 6.5.2.4.1.5-2: NR ACLR requirement

|  | Power class 12 | Power class 2 | Power class 3 |
| --- | --- | --- | --- |
| NR ACLR | 37 - TT dB2 | 31 - TT dB | 30 - TT dB |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5.2.4.1.5-3. NOTE 2: Applicable for power class 1 UE operating in n14. |  |  |  |

Table 6.5.2.4.1.5-3: Test Tolerance (NR ACLR)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 0.8 dB | 0.8 dB | 0.8 dB |

##### 6.5.2.4.2	UTRA ACLR

6.5.2.4.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5.2.4.2.2	Test applicability

This test case applies for network signalling values NS_3U, NS_5U ,NS_43U, and NS_100 to all types of NR Power Class 3 UE release 15 and forward.

6.5.2.4.2.3	Minimum conformance requirements

UTRA adjacent channel leakage power ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRAACLR is specified for the first adjacent UTRA channel (UTRAACLR1) which centre frequency is ± 2.5 MHz from NR channel edge and for the 2nd adjacent UTRA channel (UTRAACLR2) which centre frequency is ± 7.5 MHz from NR channel edge.

The UTRA channel power is measured with an RRC filter with roll-off factor =0.22 and bandwidth of 3.84 MHz. The assigned NR channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.5.2.4.1.3-1.

If the measured adjacent channel power is greater than - 50dBm then the UTRAACLR1 and UTRAACLR2 shall be higher than the value specified in Table 6.5.2.4.2.3-1.

UTRAACLR is not applicable to the power class 3 UE operating in Band n12, n14, n17, and n30.

UTRAACLR is not applicable to the power class 1 UE operating in Band n14.

Table 6.5.2.4.2.3-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.4.2.

## << End of changes >>