3GPP TSG-RAN5 Meeting #90-e	R5-211186
Electronic Meeting, 22 February – 5 March 2021

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On Larger Quiet Zone Sizes with Grey Box

Document for:	Discussion and Endorsement

# Introduction

This contribution addresses the grey-box approach [1] to test devices that cannot be tested in the largest 30cm diameter QZ currently defined in [2][3].

# Discussion

In the last few meetings, the topic of larger quiet zone sizes were discussed in [4][5][6][7][1][8][9]. In [1], the grey-box approach was endorsed to test devices that currently cannot be tested in the largest 30cm diameter QZ

Proposal 1: Consider the grey-box approach, i.e., declaration of a reference point to be aligned with the centre of the quiet zone and the maximum antenna panel displacement from this reference point, for devices that do not fit into the existing 30cm quiet zones.

Based on the discussion of larger QZ sizes in RAN5#87-e [5], three action points related to quiet zones were created in RAN5#87-e. One of which was closed in RAN5#88-e, while the remaining two (AP#87e.23 and AP#87e.24) were kept open.

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#87e.23 | RF | OEM vendors and Operators to provide feedback on how to handle test coverage for large devices in case 100% coverage cannot be ensured. Any TP allowance, MU relaxations allowed for meeting the requirements | OEMs and Operators | R5-202082 R5-202083 R5-202084 R5-202085 R5-198262 R5-198262 R5-204191 R5-204200 | RAN5#89e | Open |
| AP#87e.24 | RF | OEM vendors and operators to provide data on max antenna separation distances and device size for NR FR2 devices that can currently not be tested with the largest QZ size of 30cm in diameter. | OEMs and Operators | R5-202082 R5-202083 R5-202084 R5-202085 R5-198262 R5-204191 R5-204200 | RAN5#89e | Open |

Especially the latter action point on the maximum antenna separation of mm-wave antennas would be important to address before finalizing the larger quiet zones. This contribution provides a way forward in absence of OEM feedback on this action point.

Contributions from RAN5#89-e [1][8][9] focused on discussions of 35-40cm quiet zone diameters which triggered the creation of another action point

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#89e.21 | RF | TE vendors to provide preliminary QoQZ and XPD MU and MTSU estimates, using MOP and REFSENS as reference, for QZ sizes of 35 and 40cm (others not precluded) assessed on existing IFF test systems originally designed for 30cm QZ. | KS, R&S, Anritsu | R5-205713 R5-206068 R5-206130 | RAN5#90e | Open |

Discussions during the RAN5#89-e meeting on “extend current QZ size from 30 cm to 35 cm” focused on whether the existing 30cm quiet zone should be replaced or augmented. Excerpts of email discussion are shown below

[Keysight]: This seems to be reflected somewhat in proposals to potentially “extend” the existing 30cm QZ to 35cm. My interpretation of these proposals is to replace the black-box 30cm QZ with the grey-box 35cm QZ which has an MTSU of up to 1dB (Apple) or ≥0.6dB (R&S) higher than the existing 30cm MTSU. This would in turn wipe out the existing MTSUs and TTs for smartphones and tablets that fit into the 30cm QZ?! I don’t think this is desirable given the industry’s previous request to evolve MU (aka improve) over time?! [Apple] Our proposal is to have minimum impact to current MTSU, if extending QZ size from 30 to 35 cm with grey-box approach wouldn't increase MTSU at all, that would be our preferable. I believe R&S shared that if QZ size increases to 35 cm, there will be 0.56dB in the MTSU for MOP. We would like to ask Keysight share similar MTSU estimation. If both TE vendors shared that MTSU will be increased less than 0.6 dB, and OEMs and Operators are ok with 0.6 dB impact for device size 30-40 cm with 35 cm QZ, that would help the industry. … [Keysight] We therefore suggest not to consider an extension/replacement of the existing 30cm QZ but rather a new QZ to augment/complement the 30cm QZ. [Apple] We agree to have complement QZ based on 30 cm [R&S] We support the idea of a “complement QZ” based on 30cm QZ systems to solve the coverage issue Apple is raising for tablet-type devices. Further work will be required for the next QZ size based on the feedback to AP#87e.24.

This agreement should be clearly captured, i.e.,

Proposal 1: The next larger QZ based on the grey-box approach shall augment/complement the existing 30cm QZ

# Grey-Box Approach

Instead of defining a quiet zone size that fully encloses the entire device, the proposed grey-box approach to reduce the sizes of applicable OTA systems is to make sure that all antenna panels integrated in the DUT are enclosed within a fixed and previously assessed quiet zone while allowing the device to extend beyond the quiet zone.

This concept is further illustrated and explained in Figure 1-Figure 3. In Figure 1, a large DUT is shown with the geometric centre aligned with the centre of the 30cm QZ. Regardless of where the antenna(s) of the DUT are located, the fact that portions of the DUT extend beyond the 30cm QZ would prevent this DUT to be tested in a system with a 30cm QZ based on the current definition of the quiet zone, per B.2.2.2 of [3]

B.2.2.2 Quiet zone dimension The quiet zone shall be large enough to fully contain the DUT.

In Figure 2, the DUT is shown with its geometric centre aligned with the centre of the QZ (black box). Clearly, for the example antenna locations and placements within the device, the black-box approach does not allow the antennas to be fully contained by this QZ size. However, for the grey-box approach illustrated in Figure 3 where a reference point on the DUT, shown schematically with the red x which was chosen to be the geometric centre of the outermost dimensions of the two antenna arrays, is aligned with the centre of the QZ, both antennas are well contained within the QZ. It should be noted that devices that can be contained completely within the sample 40cm quiet zone can continue to be tested using the black box approach and without a vendor declaration.

Figure 1: Black-Box Approach for a DUT exceeding the 30cm QZ size

Figure 2: Black-Box Approach for a DUT in system with a sample 40cm QZ

Figure 3: Grey-Box Approach for a DUT in system with a sample 40cm QZ

Figure 4: Choice between black-box (left) and grey-box approach (right)

This approach would not require the declaration of the exact location of each antenna, typically required for a white box test approach. Instead, the declaration of

- a positioning reference point anywhere within the device to be aligned with the centre of the quiet zone and

- the maximum antenna panel displacement from the reference point

is sufficient for the grey-box approach.

While the declaration of the maximum antenna panel displacement from the reference point is not necessarily required as long it is confirmed that the maximum displacement is smaller or equal to the quiet zone diameter. However, this declaration would allow DUTs to be tested in bands

- that have maximum displacements ≤30cm with systems supporting just the 30cm QZ supporting the black-box approach

- that have maximum displacements >30cm (but smaller than the larger QZ) with systems supporting just the next larger QZ with the grey-box approach

This is further illustrated in Figure 4 for the same device discussed in Figure 1-Figure 3 for a different frequency band that have the antennas displaced by less than 30cm. The device in this band can now be tested using the black box approach (left portion of Figure 4) with a better MU and more conventional device positioning approach or the grey-box approach (right portion of Figure 4) with the same test setup as the other band with requires the grey-box approach in Figure 3.

Observation 1: A declaration of a positioning reference point and the maximum antenna panel displacement from the reference point (per band) is the most flexible approach

A sample vendor declaration for the grey-box approach is highlighted in Table 1

Table 1: Sample vendor declaration for grey-box approach

| Band | Positioning Reference Point: Offset (x/y/z) from geometric centre of DUT | Maximum displacement of active antenna panels |
| --- | --- | --- |
| n257 |  |  |
| n258 |  |  |
| … |  |  |
| n261 |  |  |

Proposal 2: Adopt the vendor declaration of positioning reference point and maximum antenna panel displacement from the reference point (per band)

This approach would therefore greatly increase test case coverage while preventing a significant increase in size of FR2 OTA test systems and IFF reflector sizes.

Furthermore, discussions were held during RAN5#89-e, specifically based on [9],whether the maximum device size should be specified with the next larger QZ size utilizing the grey-box approach. We do not believe that the grey-box approach should limit the device size, i.e., as long the chamber and positioner can support very large devices and the device’s antennas fit into the quiet zone this should be permissible.

Proposal 3: Do not limit the device size for the grey-box approach

# Larger QZ Sizes

Based on preliminary discussions held during the last two meetings, QZ sizes between 35cm and 40cm were discussed. Given the largest existing quiet zone of 30cm, a 35cm QZ diameter would hardly be a significant improvement in size while the 40cm would allow an increase of 10cm which matches the difference of the two QZ diameters currently defined, i.e., 20cm and 30cm diameter.

Observation 2: A 35cm QZ diameter is a very small improvement in QZ size when compared to the 40cm QZ which complements the existing QZ sizes of 20cm and 30cm well.

The action point AP#89e.21 was meant to determine the impact on extending the quiet zone of existing systems on the quiet zone performance, specifically QoQZ and XPD MU.

This contribution is based on a set of abbreviated EIRP&TRP QoQZ MU campaigns with 14 measurement points (instead of 238); it was previously shown that this limited campaign approximates the QoQZ MUs well. The quiet zone size evaluated in this contribution is a 35cm and 40cm sphere (with radius R=17.5cm and 20cm in x, y, and z) and the procedure is following the procedure and requirements outlined in [3] other than the number of measurements. The measurement results are shown in Table 2 for frequencies up to 49GHz (n262). These results show that a 0.2dB increase in QoQZ MU is expected for the 40cm QZ and that the 35cm QZ will likely have the same QoQZ MU as the 30cm QZ.

Observation 3: A 0.2dB increase in QoQZ MU is expected for the 40cm QZ and that the 35cm QZ will likely have the same QoQZ MU as the 30cm QZ

Clearly, the results for the frequencies greater than 40GHz are better than those at 23.45 and 32.125GHz; it is expected that the low-end results will improve with a revision of the measurement probe with improved beamwidth but with no loss in dynamic range/increase in influence of noise MU. Revised results will be presented at the next RAN5 meeting.

Observation 4: Even better QoQZ MUs are expected for larger QZs with a revised probe feed.

Table 2: QoQZ MUs for 35cm and 40cm QZ based on abbreviated QoQZ campaign

| QZ Size | f [GHz] → | 23.450 | 32.125 | 40.800 | 44.300 | 49.000 |
| --- | --- | --- | --- | --- | --- | --- |
| 40cm | EIRP | 0.75 | 0.76 | 0.25 | 0.36 | 0.16 |
|  | TRP | 0.52 | 0.73 | 0.18 | 0.25 | 0.21 |
| 35cm | EIRP | 0.57 | 0.49 | 0.32 | 0.58 | 0.47 |
|  | TRP | 0.39 | 0.59 | 0.13 | 0.26 | 0.19 |

It is proposed to adopt the spherical 40cm QZ with the assumption that the dynamic range/influence of noise MU is the same as for the 30cm QZ and just a very small QoQZ MU increase of less of equal than 0.2dB. A maximum increase of 0.2dB in QoQZ MU would correspond to an increase of MOP and REFSENS Maximum Test System Uncertainty (MTSU) of 0.12dB.

Proposal 3: Adopt the spherical 40cm QZ as the next larger quiet zone with the assumption that the dynamic range/influence of noise MU is the same as for the 30cm QZ.

In case PC3 and PC1 devices get introduced with antenna separation distances that exceed the largest quiet zone, 100% test coverage could be guaranteed various ways:

- Introduction of white box testing, i.e., declaration of exact antenna locations and the applicability of active antenna panels, which would allow active antenna arrays to be placed within the largest quiet zone

- Definition of larger quiet zone

- Definition of non-spherical quiet zone, e.g., cylindrical with device positioning declaration that would guarantee the active antenna panels are always enclosed.

As a last resort, waiving of conformance testing should be considered for PC3 and PC1 devices with antenna separations (grey box) larger than a specific quiet zone/antenna separation.

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: A declaration of a positioning reference point and the maximum antenna panel displacement from the reference point (per band) is the most flexible approach

Observation 2: A 35cm QZ diameter is a very small improvement in QZ size when compared to the 40cm QZ which complements the existing QZ sizes of 20cm and 30cm well.

Observation 3: A 0.2dB increase in QoQZ MU is expected for the 40cm QZ and that the 35cm QZ will likely have the same QoQZ MU as the 30cm QZ

Observation 4: Even better QoQZ MUs are expected for larger QZs with a revised probe feed.

Proposal 1: The next larger QZ based on the grey-box approach shall augment/complement the existing 30cm QZ

Proposal 2: Adopt the vendor declaration of positioning reference point and maximum antenna panel displacement from the reference point (per band)

Proposal 3: Do not limit the device size for the grey-box approach

Proposal 3: Adopt the spherical 40cm QZ as the next larger quiet zone with the assumption that the dynamic range/influence of noise MU is the same as for the 30cm QZ.

# References

- R5-206068, On Larger Quiet Zone Sizes, Keysight Technologies, 3GPP TSG-RAN5 Meeting #89-e, November 2020

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment, V16.4.0 (2020-06)

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.40 (2020-06)

- R5-198262, On Larger Quiet Zone Sizes, Keysight Technologies, 3GPP TSG-RAN WG5 #85, November 2019

- R5-202082, Discussion on size of Quiet Zone above 30cm, Rohde & Schwarz, 3GPP TSG RAN5 Meeting #87-e, May 2020

- R5-204747, On Larger Quiet Zone Sizes, Keysight Technologies, 3GPP TSG-RAN5 Meeting #88-e, August 2020

- R5-204200, Discussion on size of Quiet Zone above 30cm, Rohde & Schwarz, 3GPP TSG-RAN5 Meeting #88-e, August 2020

- R5-206130, Discussion on size of Quiet Zone above 30cm, Rohde & Schwarz, 3GPP TSG-RAN5 Meeting #89-e, November 2020

- R5-205713, AP#87e.23 Test coverage for large devices, Apple Inc., 3GPP TSG-RAN5 Meeting #89-e, November 2020