`3GPP TSG-RAN5 Meeting #90-e	R5-211193
Electronic Meeting, 22 February – 5 March 2021

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	On the MU Element “Uncertainty of an absolute gain of the calibration antenna” for n259

Document for:	Discussion and Endorsement

# Introduction

This contribution is addressing the MU element “Uncertainty of an absolute gain of the calibration antenna” for the n259 band.

# Discussion

In the RAN4#97-e WF on n262 [1], it was noted to complete the work on Band n259 (40-43.5GHz)

Observation 1 : In order to introduce 5G (Band n259; 40GHz band) service in Japan, it is required to complete RAN5 work by Feb-2021 at the latest.

The MU evaluation requires a maximum test frequency of 44.3GHz [2] for FR2c.

In this contribution, we revisit an earlier contribution [3] that defined the MU element “Uncertainty of an absolute gain of the calibration antenna” for FR2a and FR2b, i.e., from 23.45GHz through 40.8GHz with uncertainty value of 0.60dB (normal distribution).

Given the calibration certificate that was leveraged to define the 0.6dB sample value indicates that the same uncertainty is applicable up to 50GHz

it is proposed to assign the same example value for n259 band with max test frequency of 44.3GHz.

Proposal 1: For n259, use the same 0.6dB uncertainty for MU Element “Uncertainty of an absolute gain of the calibration antenna”

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: For n259, use the same 0.6dB uncertainty for MU Element “Uncertainty of an absolute gain of the calibration antenna”

# References

- R5-204851, Update of WF on Band n259, NTT DOCOMO, 3GPP TSG-RAN5 Meeting #88-e, August 2020

- R5-197733, Frequency range of Band n259, NTT DOCOMO, 3GPP TSG-RAN5 Meeting #85, November 2019

- R5-185954, On the MU Element “Uncertainty of an absolute gain of the calibration antenna,” 3GPPRAN5#3-5G-NR Adhoc, October 2018