3GPP TSG-RAN5 Meeting #90-e

# Electronic meeting, 22nd Feb to 05th March 2021

Title:		TP Analysis for FR2 Tx Additional Spurious emissions NS 202 & 203 test case

Source:			Qualcomm Inc.

Document for:		Discussion and endorsement

# Introduction

The purpose of this paper is to provide test point selection proposal for NR FR2 Tx additional spurious emission NS 202 and NS 203 test cases in 38.521-2 -> section 6.5.3.3.

# Discussion

The test point analysis for FR2 Tx additional spurious emissions (TS38.521-2 -> Section 6.5.3.3) test case shall be the same as that agreed for FR2 Tx Spurious emissions (TS38.521-2 -> Section 6.5.3.1) test case. The proposals for FR2 Tx additional spurious emission NS 202 and NS 203 test cases in 38.521-2 -> Section 6.5.3.3 is stated below.

## Test Environment

Proposal 1: Propose Test Environment as Normal Conditions (NC) for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

## Test Frequencies

Proposal 2: Test low-range & high-range frequency only for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

## Test Channel Bandwidth

Proposal 3: Select highest channel bandwidth only for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

## Test Subcarrier Spacing

Proposal 4: Select lowest supported SCS only for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

## Uplink configuration

Proposal 5: Inner Full and Inner_1RB for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Proposal 6: Proposed that DFT-s-OFDM be selected for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Proposal 7: Select QPSK modulation scheme for TX spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Implementing the above proposals, the proposed updated test configuration table for TS38.521-2 -> Section 6.5.3.3 is as shown below:

Table 6.5.3.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s -OFDM QPSK | Inner_Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s -OFDM QPSK | Inner_1RB (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_1RB_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_1RB_Region1 for PC1. |  |  |  |

The maximum number of test points for NR FR2 TX additional spurious emission = 4:

| Environment conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 NC | 1 (120kHz) | 1 (H) | 2 (L/H) | 2 (Full/1RB) | 4 |

# Conclusion

In conclusion, the following selection of test points are proposed TX additional spurious emission measurement in FR2:

Proposal 1: Propose Test Environment as Normal Conditions (NC) for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Proposal 2: Test low-range & high-range frequency only for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Proposal 3: Select highest channel bandwidth only for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Proposal 4: Select lowest supported SCS only for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Proposal 5: Inner Full and Inner_1RB for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Proposal 6: Proposed that DFT-s-OFDM be selected for TX additional spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.

Proposal 7: Select QPSK modulation scheme for TX spurious emission measurement in FR2. This is the same as that in FR2 general spurious emission test point.