3GPP TSG-RAN5 Meeting #90-e	R5-211257

# Electronic Meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5274 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for Band 48 in test case 6.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_TDD_3550_CBRS_US-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test requirement for Band 48 is missing in MOP test case Table 6.2.2.5-1 although the work item has been completed in 2017. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test requirement for Band 48 has been specified in Table 6.2.2.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Maximum output power cannot be tested for Band 48. Other test cases referring to the MOP test requirement of Band 48 cannot be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 6.2.2.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1.

Table 6.2.2.5-1: UE Power Class test requirements

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2.7 |  |  |
| 2 |  |  |  |  | 23 | ±2.72 |  |  |
| 3 |  |  |  |  | 23 | ±2.72 |  |  |
| 4 |  |  |  |  | 23 | ±2.7 |  |  |
| 5 |  |  |  |  | 23 | ±2.7 |  |  |
| 6 |  |  |  |  | 23 | ±2.7 |  |  |
| 7 |  |  |  |  | 23 | ±2.72 |  |  |
| 8 |  |  |  |  | 23 | ±2.72 |  |  |
| 9 |  |  |  |  | 23 | ±2.7 |  |  |
| 10 |  |  |  |  | 23 | ±2.7 |  |  |
| 11 |  |  |  |  | 23 | ±2.7 |  |  |
| 12 |  |  |  |  | 23 | ±2.72 |  |  |
| 13 |  |  |  |  | 23 | ±2.7 |  |  |
| 14 |  |  |  |  | 23 | ±2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  | 23 | ±2.7 |  |  |
| 18 |  |  |  |  | 23 | ±2.76 |  |  |
| 19 |  |  |  |  | 23 | ±2.7 |  |  |
| 20 |  |  |  |  | 23 | ±2.72 |  |  |
| 21 |  |  |  |  | 23 | ±2.7 |  |  |
| 22 |  |  |  |  | 23 | +3.0/-4.52 |  |  |
| 23 |  |  |  |  | 235 | ±2.75 |  |  |
| 24 |  |  |  |  | 23 | ±2.7 |  |  |
| 25 |  |  |  |  | 23 | ±2.72 |  |  |
| 26 |  |  |  |  | 23 | ±2.72 |  |  |
| 27 |  |  |  |  | 23 | ±2.7 |  |  |
| 28 |  |  |  |  | 23 | +2.7/-3.2 |  |  |
| 30 |  |  |  |  | 23 | ±2.7 |  |  |
| 31 |  |  |  |  | 23 | ±2.7 |  |  |
| ... |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2.7 |  |  |
| 34 |  |  |  |  | 23 | ±2.7 |  |  |
| 35 |  |  |  |  | 23 | ±2.7 |  |  |
| 36 |  |  |  |  | 23 | ±2.7 |  |  |
| 37 |  |  |  |  | 23 | ±2.7 |  |  |
| 38 |  |  |  |  | 23 | ±2.7 |  |  |
| 39 |  |  |  |  | 23 | ±2.7 |  |  |
| 40 |  |  |  |  | 23 | ±2.7 |  |  |
| 41 |  |  |  |  | 23 | ±2.72 |  |  |
| 42 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| 43 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| 44 |  |  |  |  | 23 | +2.7/[-3.7] |  |  |
| 45 |  |  |  |  | 23 | ±2.7 |  |  |
| 47 |  |  | 26 | ±3.3 | 23 | ±3.3 |  |  |
| 48 |  |  |  |  | 23 | +3.0/-4.0 |  |  |
| … |  |  |  |  |  |  |  |  |
| 53 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| 66 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2.7/-2.7 |  |  |
| 71 |  |  |  |  | 23 | +2.7/-3.2 |  |  |
| 72 |  |  |  |  | 23 | ±2.7 |  |  |
| 73 |  |  |  |  | 23 | ±2.7 |  |  |
| 74 |  |  |  |  | 23 | ±2.7 |  |  |
| 85 |  |  |  |  | 23 | ±2.72 |  |  |
| 87 |  |  |  |  | 23 | ±2.7 |  |  |
| 88 |  |  |  |  | 23 | ±2.7 |  |  |
| Note 1: Void Note 2: For transmission bandwidths (Figure 5.4.2-1, Table 5.4.4-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB Note 3: PPowerClass is the maximum UE power specified without taking into account the tolerance. Note 4: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. Note 5: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. Note 6: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5dB for transmission bandwidths confined within 815 MHz and 818 MHz. |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c in Tables 6.2.5.3-2, 6.2.5.3-3 and 6.2.5.3-4 shall be applied for applicable bands.

In case the UE supports more than one of the above inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the tolerances in Table 6.2.5.3-2, truncated to one decimal place for that operating band among the supported CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance in Table 6.2.5.3-2 that applies for that operating band among the supported CA configurations.

< End of changes >