3GPP TSG RAN Meeting #90-e	 	R5-211266

Electronic Meeting, 22th February – 5th  March 2021

Source:		Rohde & Schwarz

Title:	On the MU of FR2 OBW

Agenda Item:		5.3.2.17

Document for:	Discussion and Endorsement

# Introduction

The measurement uncertainty (MU) of occupied bandwidth (OBW) has been discussed during the last meetings [1] to [20]. However, no final conclusion has been reached. This contribution is a resubmission of the paper submitted to RAN5#89-e [19].

# Discussion

During RAN5 #88-e it was observed that the commonly agreed simulation assumption of 28000 MHz as center for the data sets provided in [9] has been incorrect. The new simulation assumption is 27998.4 MHz. With respect to the new center frequency the power spectrum of the UE is much better centered than with the old assumption. This results in lower occupied bandwidth deviations since an evaluation window is applied whose center is aligned to that of the channel.

Instead of simulating the frequency response by three nodes at the channel center and at the edges, we base our simulation on measurements of the frequency response of a test system.

At RAN5#88-e the metric to define the OBW MU has been agreed to be [16,17]

(OBWmeas - OBWsignal) / CBW *100, 		(1)

where OBWsignal is the OBW of data in [9] without any additional impairments due to signal flatness or noise. OBWmeas is the OBW which would be measured when the assumptions of ripple and noise apply.

The spectra of each polarization and of the total power are plotted for 100 MHz CHBW and 4x100 MHz in Figures 1 and 2, respectively. During RAN5#85, it has been confirmed that the 4x100 MHz CA dataset can be applied to derive the MU for OBW for 400 MHz single carrier. Due the CA nature of the 400 MHz spectrum a channel bandwidth of 394.92 MHz instead of 380.16 MHz (single carrier TxBW) should be used as measurement BW to determine the channel power [16]. 
In our understanding CBW in equation (1) is 400 MHz for the 4x100 MHz dataset.

Figure 1: Spectrum for 100 MHz CHBW from [9]

Figure 2: Spectrum for 400 MHz CHBW from [9]

Table 1 summarizes some parameters of the dataset for the assumption of 27998.4 MHz as center frequency. Interpolation of the provided data has been applied in order to improve the resolution of the OBW results. The OBW has been calculated applying an evaluation window whose center is aligned on the center of the channel.

Table 1: Results of the two data sets

|  | 4x100 MHz CA dataset |
| --- | --- |
| center frequency (MHz) | 27998.4 |
| OBW (span=1.5⋅CHBW) | 394.64 MHz |
| CP (dBm) | 18.41 |

For the channel power CP, it is assumed that the RAW data has been measured with a resolution bandwidth of 1 MHz. If the RBW has been different, there will be a constant shift in the absolute power level which should not be relevant for the OBW MU derivation.

Compared to our contribution to RAN5#88-e [16], we have measured also the noise power spectral density when no signal is applied. In this way, both the ripple and the noise of the test system are accounted for in the OBW simulations. The measurements have been performed at a power level equivalent to minEIRP taken into account MBR, MPR, power tolerance and TT. The measurements have been repeated for each of the FR2 bands at low, mid and high frequency range and the four channel bandwidths.

Based on measured frequency characteristics and SNR of our test systems, we obtain the measurement uncertainties summarized in Table 2.

Table 2: Occupied bandwidth MU

| Frequency range | OBW MU for 50MHz and 100MHz CHBW | OBW MU for 200 MHz CHBW | OBW MU for 400 MHz |
| --- | --- | --- | --- |
| FR2a | ±0.4% | ±1.1% | ±1.2% |
| FR2b | ±0.4% | ±0.8% | ±1.3% |
| FR2c | ±0.4% | -1.6%/+1.25% | work in progress |

Proposal 1: For the MU of the single carrier occupied bandwidth TC apply the values in Table 2 for  PC3.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, the impact of SNR and frequency flatness on the OBW MU has been analyzed.

Table 2: Occupied bandwidth MU

| Frequency range | OBW MU for 50MHz and 100MHz CHBW | OBW MU for 200 MHz CHBW | OBW MU for 400 MHz |
| --- | --- | --- | --- |
| FR2a | ±0.4% | ±1.1% | ±1.2% |
| FR2b | ±0.4% | ±0.8% | ±1.3% |
| FR2c | ±0.4% | -1.6%/+1.25% | work in progress |

Proposal 1: For the MU of the single carrier occupied bandwidth TC apply the values in Table 2 for  PC3.

# References

- R5-188189, On the FR2 MU for occupied BW and ACLR, Anritsu, 3GPP TSG RAN WG 5 Meeting #81, November 2018

- R5-190390, On QoQZ MU Impact on OBW, Keysight, 3GPP TSG RAN WG 5 Meeting #4-5G-NR Adhoc, January 2019

- R5-192660, On the FR2 OBW MU, Anritsu, 3GPP TSG RAN WG 5 Meeting #82, February 2019

- R5-191617, On OBW measurement uncertainty, Keysight, 3GPP TSG RAN WG 5 Meeting #82, February 2019

- R5-194353, On the noise impact to FR2 OBW, Anritsu, 3GPP TSG RAN WG 5 Meeting #83, May 2019

- R5-192984, On the SNR for FR2 TRx test cases, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #5-5G-NR Adhoc, April 2019

- R5-193187, On noise impact, Keysight, 3GPP TSG RAN WG 5 Meeting #5-5G-NR Adhoc, April 2019

- R5-197626, Impact of noise for FR2 OBW, Anritsu, 3GPP TSG RAN WG 5 Meeting #83, May 2019

- R5-198175, Discussion on AP#84.22 Tx Spectrum Shape for FR2 OBW MU, Qualcomm, 3GPP TSG RAN WG 5 Meeting #85, November 2019

- R5-200869, On the MU of FR2 OBW, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #86-e, February 2020

- R5-200739, On FR2 OBW MU, Anritsu, 3GPP TSG RAN WG 5 Meeting #86-e, February 2020

- R5-200874, On FR2 OBW MU, Keysight Technologies, 3GPP TSG RAN WG 5 Meeting #86-e, February 2020

- R5-202458, On the MU of FR2 OBW, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #87-e, May 2020

- R5-202318, On FR2 OBW MU, Anritsu, 3GPP TSG RAN WG 5 Meeting #87-e, May 2020

- R5-202460; On FR2 OBW MU, Keysight Technologies, 3GPP TSG RAN WG 5 Meeting #87-e, May 2020

- R5-204270, On the MU of FR2 OBW, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #88-e, August 2020

- R5-204849, On FR2 OBW MU, Anritsu, 3GPP TSG RAN WG 5 Meeting #88-e, August 2020

- R5-204850; On OBW measurement uncertainty, Keysight Technologies, 3GPP TSG RAN WG 5 Meeting #88-e, August 2020

- R5-206617, On the MU of FR2 OBW, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #89-e, November 2020

- R5-206175, On FR2 OBW MU, Anritsu, 3GPP TSG RAN WG 5 Meeting #89-e, November 2020