3GPP TSG-RAN5 Meeting #90-e                                                      R5-211276

Electronic Meeting, 22nd February – 5th March

Title:		        FR2 Extreme testing conditions applicability

Source:	        Keysight Technologies

Agenda Item:	5.3.2.17

Document for:	Discussion and Endorsement

# 1.	Introduction

Applicability of extreme testing condition (ETC) is not fully clear in FR2 test specifications ([1], [2]), probably due to testability concerns raised in previous RAN4 and RAN5 meetings.

Now that ETC testability has progressed ([3], [4], [5], [6]) and some details are being finalized on measurement uncertainties under ETC ([7], [8]), a full revision of ETC applicability to FR2 test cases is required.

This contribution includes an analysis on FR2 core requirements and compares FR1 and FR2 test specifications with respect to extreme temperature conditions.

# 2.	ETC in core requirements

Core requirements for FR2 Standalone ([9]) includes the following statement in Annex E:

Observation 1: Unless otherwise stated, all core requirements are applicable under nominal and extreme environmental testing conditions.

[9] only includes the following explicit exceptions to extreme testing conditions:

Proposal 1: Send an LS to RAN4 to clarify whether explicit exceptions to ETC were related to testability issues or to lack of core requirement relaxation for extreme temperature conditions.

# 3.	ETC in FR1 test specification vs FR2 test specification

To understand which core requirement could have dependencies with the temperature, FR1 test specifications for standalone can be used as a reference ([10]).

Table 1 provides:

- a list of FR1 SA test cases requiring ETC testing

- the mapping of those FR1 SA test cases to equivalent ones in FR2 SA specification ([2])

- some comments on current ETC configuration in FR2 SA equivalent test case

- proposed action per FR2 test case regarding ETC.

Table 1 ETC FR1 test specification vs FR test specification

| FR1 Test case requiring ETC |  |  | Equivalent FR2 |  |  | Comment | Action |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Spec. | Test Case ID | Test case Title | Test Spec. | Test case ID | Test case Title |  |  |
| 38.521-1 | 6.2.1 | UE maximum output power | 38.521-2 | 6.2.1.1 | UE maximum output power - EIRP and TRP | ETC already considered in 38.521-2 although NOTE 2 indicating Test environment for UE Max TRP is normal only | Remove NOTE 2 |
| 38.521-1 | 6.2.2 | UE maximum output power reduction | 38.521-2 | 6.2.2 | UE maximum output power reduction | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.2.4 | Configured transmitted power | 38.521-2 | 6.2.4 | Configured transmitted power | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.2A.1.1 | UE maximum output power for CA (2 UL CA) | 38.521-2 | 6.2A.1.1.1 | UE maximum output power - EIRP and TRP for CA (2UL CA) | ETC already considered in 38.521-2 although NOTE 2 indicating Test environment for UE Max TRP is normal only | Remove NOTE 2 |
| 38.521-1 | 6.2A.2.1 | Maximum power reduction (MPR) for CA (2 UL CA) | 38.521-2 | 6.2A.2.1 | UE maximum power reduction for CA (2 UL CA) | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.2A.4.1 | Configured transmitted power for CA (2 UL CA) | 38.521-2 | 6.2A.4 | Configured transmitted power for CA | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.2C.1 | Configured transmitted power for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.2C.3 | UE maximum output power for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.2C.4 | UE maximum output power reduction for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.2D.1 | UE maximum output power for UL-MIMO | 38.521-2 | 6.2D.1 | UE maximum output power for UL-MIMO | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.2D.2.4 | Configured transmitted power for UL-MIMO | 38.521-2 | 6.2D.2.4 | Configured transmitted power for UL-MIMO | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.3.1 | Minimum output power | 38.521-2 | 6.3.1 | Minimum output power | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.3.3.2 | General ON/OFF time mask | 38.521-2 | 6.3.3.2 | General ON/OFF time mask | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.3.3.4 | PRACH time mask | 38.521-2 | 6.3.3.4 | PRACH time mask | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.3.3.6 | SRS time mask | 38.521-2 | 6.3.3.6 | SRS time mask | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.3.4.3 | Power Control Relative power tolerance | 38.521-2 | 6.3.4.3 | Relative power tolerance | Explicit ETC exception in core requirements | Clarify with RAN4 whether ETC exception was related to testability issues or to lack of ETC tolerance definition |
| 38.521-1 | 6.3A.1.1 | Minimum output power for CA (2 UL CA) | 38.521-2 | 6.3A.1.1 | Minimum output power for CA (2 UL CA) | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.3A.3.1 | Transmit ON/OFF time mask for CA (2 UL CA) | 38.521-2 | 6.3A.3.1 | Transmit ON/OFF time mask for CA (2UL CA) | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.3C.1 | Minimum output power for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.3C.3 | Transmit ON/OFF time mask for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.3C.4.2 | Power control tolerance for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.3D.1 | Minimum output power for UL-MIMO | 38.521-2 | 6.3D.1 | Minimum output power for UL-MIMO | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.3D.3 | Transmit ON/OFF time mask for UL-MIMO | 38.521-2 | 6.3D.3.1 | Transmit ON/OFF time mask for UL-MIMO | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.4.1 | Frequency error | 38.521-2 | 6.4.1 | Frequency error | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.4.2.4 | EVM equalizer spectral flatness | 38.521-2 | 6.4.2.4 | EVM equalizer spectral flatness | Explicit ETC exception in core requirements | Clarify with RAN4 whether ETC exception was related to testability issues or to lack of ETC tolerance definition |
|  |  |  | 38.521-2 | 6.4.2.4.1 | EVM equalizer spectral flatness for pi/2 BPSK modulation | Explicit ETC exception in core requirements | Clarify with RAN4 whether ETC exception was related to testability issues or to lack of ETC tolerance definition |
| 38.521-1 | 6.4A.1.1 | Frequency error for CA (2 UL CA) | 38.521-2 | 6.4A.1.1 | Frequency error for CA (2 UL CA) | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.4C.1 | Frequency error for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.4C.2.2 | Carrier leakage for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.4C.2.4 | EVM equalizer spectrum flatness for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.4D.1.4 | Frequency error for UL MIMO | 38.521-2 | 6.4D.1 | Frequency error for UL MIMO | - Not added to the test specification yet but defined in Work plan and core requirements - ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.4D.2.4 | EVM equalizer spectrum flatness for UL MIMO | 38.521-2 | 6.4D.2.4 | EVM equalizer spectrum flatness for UL MIMO | - Not added to the test specification yet but defined in Work plan and core requirements - No explicit ETC exception in core requirements but UL-SISO requirements applicable | Clarify with RAN4 whether ETC exception was related to testability issues or to lack of ETC tolerance definition |
| 38.521-1 | 6.5.2.4.1 | NR ACLR | 38.521-2 | 6.5.2.3 | Adjacent channel leakage ratio | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 6.5C.2.4.1 | NR ACLR for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 6.5D.2.4.1 | NR ACLR for UL MIMO | 38.521-2 | 6.5D.2.2 | Adjacent channel leakage ratio for UL MIMO | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 7.3.2 | Reference sensitivity power level | 38.521-2 | 7.3.2 | Reference sensitivity power level | ETC already considered in 38.521-2 | None |
| 38.521-1 | 7.3A.1 | Reference sensitivity for 2DL CA without exception | 38.521-2 | 7.3A.2.1 | Reference sensitivity power level for CA (2DL CA) | ETC already considered in 38.521-2 | None |
| 38.521-1 | 7.3A.1_1 | Reference sensitivity for 2DL CA exceptions | 38.521-2 |  |  |  |  |
| 38.521-1 | 7.3A.2 | Reference sensitivity power level for 3 DL CA | 38.521-2 | 7.3A.2.2 | Reference sensitivity power level for CA (3DL CA) | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 7.3C.2 | Reference sensitivity power level for SUL | 38.521-2 | N/A | N/A | N/A | None |
| 38.521-1 | 7.3D.2 | Reference sensitivity power level for UL MIMO | 38.521-2 | 7.3D.1 | Reference sensitivity power level for UL MIMO | ETC conditions in FR1 should be used | To be tested under ETC |
| 38.521-1 | 7.3E.2 | Reference sensitivity for V2X / non-concurrent operation | 38.521-2 | N/A | N/A | N/A | None |

Observation 2: Extreme temperature conditions are not aligned between FR1 and FR2 Standalone technical specifications ([10], [1]).

Proposal 2: Align FR2 Standalone Extreme testing conditions per test case with those in FR1 Standalone (as shown in Table 1 above) if corresponding core requirements do not include any explicit exception to ETC.

Additionally, in 38.521-1 ([10]) only test cases up to 2 UL CA have been defined so far for most of the test cases. However, environmental conditions should be aligned for any number of CCs.

Table 2 describes the list of test cases already defined in [2] that would require environmental conditions update to include ETC. To be noticed that additional CA test cases will be defined in the short term and should follow the same approach.

Table 2 Additional UL CA test cases already defined in FR2 test specification requiring ETC (as per FR1 vs FR2 comparison)

| Additional FR2 test case requiring ETC testing |  |  | Comment | Action |
| --- | --- | --- | --- | --- |
| Test Spec. | Test case ID | Test case Title |  |  |
| 38.521-2 | 6.2A.1.1.2 | UE maximum output power - EIRP and TRP for CA (3UL CA) | - Related to UE maximum output power - EIRP and TRP for CA (2UL CA) - ETC already considered in 38.521-2 although NOTE 2 indicating Test environment for UE Max TRP is normal only | Remove NOTE 2 |
| 38.521-2 | 6.2A.1.1.3 | UE maximum output power - EIRP and TRP for CA (4UL CA) | - Related to UE maximum output power - EIRP and TRP for CA (2UL CA) - ETC already considered in 38.521-2 although NOTE 2 indicating Test environment for UE Max TRP is normal only | Remove NOTE 2 |
| 38.521-2 | 6.3A.1.2 | Minimum output power for CA (3UL CA) | Related to Minimum output power for CA (2 UL CA) | To be tested under ETC |
| 38.521-2 | 6.3A.1.3 | Minimum output power for CA (4UL CA) | Related to Minimum output power for CA (2 UL CA) | To be tested under ETC |
| 38.521-2 | 6.3A.1.4 | Minimum output power for CA (5UL CA) | Related to Minimum output power for CA (2 UL CA) | To be tested under ETC |
| 38.521-2 | 6.3A.1.5 | Minimum output power for CA (6UL CA) | Related to Minimum output power for CA (2 UL CA) | To be tested under ETC |
| 38.521-2 | 6.3A.1.6 | Minimum output power for CA (7UL CA) | Related to Minimum output power for CA (2 UL CA) | To be tested under ETC |
| 38.521-2 | 6.3A.1.7 | Minimum output power for CA (8UL CA) | Related to Minimum output power for CA (2 UL CA) | To be tested under ETC |
| 38.521-2 | 6.4A.1.2 | Frequency error for CA (3UL CA) | Related to Frequency error for CA (2 UL CA) | To be tested under ETC |
| 38.521-2 | 6.4A.1.3 | Frequency error for CA (4UL CA) | Related to Frequency error for CA (2 UL CA) | To be tested under ETC |
| 38.521-2 | 7.3A.2.3 | Reference sensitivity power level for CA (4DL CA) | Related to Reference sensitivity power level for CA (2DL CA) | To be tested under ETC |
| 38.521-2 | 7.3A.2.4 | Reference sensitivity power level for CA (5DL CA) | Related to Reference sensitivity power level for CA (2DL CA) | To be tested under ETC |
| 38.521-2 | 7.3A.2.5 | Reference sensitivity power level for CA (6DL CA) | Related to Reference sensitivity power level for CA (2DL CA) | To be tested under ETC |
| 38.521-2 | 7.3A.2.6 | Reference sensitivity power level for CA (7DL CA) | Related to Reference sensitivity power level for CA (2DL CA) | To be tested under ETC |
| 38.521-2 | 7.3A.2.7 | Reference sensitivity power level for CA (8DL CA) | Related to Reference sensitivity power level for CA (2DL CA) | To be tested under ETC |

Proposal 3: Use the same environmental testing condition for any number of CCs in FR2 Standalone test cases.

Finally, there are other FR2 test cases not having a direct mapping with corresponding FR1 test case but should require ETC testing due to the relation with other core requirements requiring ETC. These are listed in Table 3.

Table 3 Additional test cases requiring ETC not mapping directly with FR1 test cases

| Additional FR2 test case requiring ETC testing |  |  | Comment | Action |
| --- | --- | --- | --- | --- |
| Test Spec. | Test case ID | Test case Title |  |  |
| 38.521-2 | 6.2.1.2 | UE maximum output power – Spherical coverage | Related to UE-maximum output power in core requirements | To be tested under ETC |
| 38.521-2 | 6.2A.1.2.1 | Spherical coverage for CA (2UL CA) | Related to UE-maximum output power for CA in core requirements | To be tested under ETC |
| 38.521-2 | 6.2A.1.2.2 | Spherical coverage for CA (3UL CA) | Related to UE-maximum output power for CA in core requirements | To be tested under ETC |
| 38.521-2 | 6.2A.1.2.3 | Spherical coverage for CA (4UL CA) | Related to UE-maximum output power for CA in core requirements | To be tested under ETC |
| 38.521-2 | 6.2A.1.2.4 | Spherical coverage for CA (5UL CA) | Related to UE-maximum output power for CA in core requirements | To be tested under ETC |
| 38.521-2 | 6.2A.1.2.5 | Spherical coverage for CA (6UL CA) | Related to UE-maximum output power for CA in core requirements | To be tested under ETC |
| 38.521-2 | 6.2A.1.2.6 | Spherical coverage for CA (7UL CA) | Related to UE-maximum output power for CA in core requirements | To be tested under ETC |
| 38.521-2 | 6.2A.1.2.7 | Spherical coverage for CA (8UL CA) | Related to UE-maximum output power for CA in core requirements | To be tested under ETC |
| 38.521-2 | 6.6.1.3.3 | Beam correspondence for PC3 | - Related to UE-maximum output power in core requirements - Explicit ETC exception in core requirements | Clarify with RAN4 whether ETC exception was related to testability issues or to lack of ETC tolerance definition |
| 38.521-2 | 7.3.4 | EIS spherical coverage | - Related to Reference sensitivity power level in core requirements - Explicit ETC exception in core requirements | Clarify with RAN4 whether ETC exception was related to testability issues or to lack of ETC tolerance definition |
| 38.521-2 | 7.3D.2 | EIS spherical coverage for UL MIMO | - Related to Reference sensitivity power level in core requirements - No explicit ETC exception in core requirements but UL-SISO requirements applicable | Clarify with RAN4 whether ETC exception was related to testability issues or to lack of ETC tolerance definition |

Proposal 4: Add ETC testing to specific FR2 SA test cases (not applicable to FR1) listed in table 3 including action “To be tested under ETC”.

# 4.	NSA vs SA specifications

Extreme testing conditions should be aligned for FR2 NSA and SA test specifications.

Proposal 5: Align ETC testing in FR2 SA and NSA test cases.

# 5.	Conclusion

This document includes some proposals to progress on FR2 SA ETC testing.

Following observations and proposals are made:

Observation 1: Unless otherwise stated, all core requirements are applicable either under nominal or extreme environmental testing conditions.

Proposal 1: Send an LS to RAN4 to clarify whether explicit exceptions to ETC were related to testability issues or to lack of core requirement relaxation for extreme temperature conditions.

Observation 2: Extreme testing conditions is not aligned between FR1 and FR2 Standalone test specifications ([10], [1]).

Proposal 2: Align FR2 Standalone Extreme testing conditions per test case with those in FR1 Standalone (as shown in Table 1 above) if corresponding core requirements don’t include any explicit exception to ETC.

Proposal 3: Use the same environmental testing condition for any number of CCs in FR2 Standalone test cases.

Proposal 4: Add ETC testing to specific FR2 SA test cases (not applicable to FR1) listed in table 3 including action “To be tested under ETC”.

Proposal 5: Align ETC testing in FR2 SA and NSA test cases.

# 6.	References

[1] 38.521-2 v16.6.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”

[2] 38.521-3 v16.6.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[3] R5-204739, “Preliminary QoQZ and EIRP MU results for ETC testing “, Keysight Technologies, RAN5#88-e

[4] R5-206819,“QoQZ and EIRP/EIS MU results for ETC testing”, Keysight Technologies, RAN5#89-e

[5] R5-206821,“CR to 38.521-2 on ETC Testing”, Keysight Technologies, RAN5#89-e

[6] R5-206835, “CR to 38.903 on ETC Testing”, Keysight Technologies, RAN5#89-e

[7] R5-211184, “On ETC MUs”, Keysight Technologies, Rohde & Schwarz, RAN5#90-e

[8] R5-211185, “CR to 38.903 on ETC Testing”, Keysight Technologies, RAN5#90-e

[9] 38.101-2 v16.6.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[10] 38.521-1 v16.6.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”