3GPP TSG-RAN5 Meeting #90-e	R5-211298

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0142 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for support of multiple GPS signals |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2021-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the case where a UE supports multiple GPS signals, previous CRs have missed some text in clauses 4.4.1 and 4.4.3 that needs modification. For sub-tests 4, 8 and 10, in the case where the UE supports multiple GPS signals, the GANSS Auxiliary Information needs to be sent for multiple GPS signals. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Tables in 4.4.1.4, 4.4.1.4A, 4.4.1.6, 4.4.3.4, 4.4.3.4A and 4.4.3.6, in the case where the UE supports multiple GPS signals, an additional UE positioning GANSS assistance data message is added which will carry the additional GANSS Auxiliary Information for multiple GPS signals. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the case where a UE supports multiple GPS signals, errors will remain and the tests will not correctly test UEs that support multiple GPS signals. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.4, 4.4.1.4A, 4.4.1.6, 4.4.3.4, 4.4.3.4A, 4.4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-5, CR 0203 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.4	A-GNSS assistance data sets

This subclause defines the assistance data sets supplied by the SS in A-GNSS test cases specified in subclause 6.2. For A-GNSS sub-test cases which include the GPS L1 C/A signal, the A-GPS assistance data as defined in clause 4.3 apply.

Throughout this subclause, "adequate assistance data" means the assistance data used in test cases where it is expected that a UE supporting A-GNSS will be able to perform the requested positioning operation using the supplied assistance data, and "inadequate assistance data" is the assistance data used in test cases that expect that the UE will be unable to perform the requested operation. The values of all the fields in all cases are defined in 3GPP TS 37.571-5 [12] subclause 6.1.3.

### 4.4.1	Adequate assistance data for UE-based A-GNSS

For UE-based GNSS test cases requiring adequate assistance data, the IE "UE positioning GANSS assistance data" is spread across one or more (dependent on the sub-test) separate MEASUREMENT CONTROL messages, and set as follows:

#### 4.4.1.1	Sub-Test 1

First MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time |  |  |
| - GANSS Day | Set according to 4.2 |  |
| - GANSS Day Cycle Number | Set according to 4.2 | Rel-10 UE or later |
| - GANSS TOD | Set according to 4.2 |  |
| - GANSS TOD Uncertainty | Set according to 4.2 |  |
| - GANSS Time ID | 2 (GLONASS) |  |
| - UTRAN GANSS reference time | Not present |  |
| - TUTRAN-GANSS drift rate | Not present |  |
| - UE positioning GANSS reference UE position | Set according to 4.2 |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | 3 (GLONASS) |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Set according to 4.2 |  |
| - Non-Broadcast Indication | Not present |  |
| - Satellite information | For satellites 1-6 |  |
| - GANSS additional clock models | Model-4 |  |
| - GANSS additional orbit models | Model-4 |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Not present |  |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Set according to 4.2 |  |
| - GANSS-ID-3 |  |  |
| - Aux Info List | For satellites 1-6 |  |

#### 4.4.1.2	Sub-Test 2

First MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time |  |  |
| - GANSS Day | Set according to 4.2 |  |
| - GANSS Day Cycle Number | Set according to 4.2 | Rel-10 UE or later |
| - GANSS TOD | Set according to 4.2 |  |
| - GANSS TOD Uncertainty | Set according to 4.2 |  |
| - GANSS Time ID | Not present |  |
| - UTRAN GANSS reference time | Not present |  |
| - TUTRAN-GANSS drift rate | Not present |  |
| - UE positioning GANSS reference UE position | Set according to 4.2 |  |
| - UE positioning GANSS ionospheric model | Set according to 4.2 |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | Not present |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Set according to 4.2 |  |
| - Non-Broadcast Indication | Not present |  |
| - Satellite information | For satellites 1-N |  |
| - GANSS clock model | Model-1 |  |
| - GANSS orbit model | Model-1 |  |
| - UE positioning GANSS additional navigation models | Not present |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Not present |  |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Not present |  |

Second MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | Not present |
| UE positioning GANSS assistance data |  |
| - UE positioning GANSS reference time | Not present |
| - UE positioning GANSS reference UE position | Not present |
| - UE positioning GANSS ionospheric model | Not present |
| - UE positioning GANSS additional ionospheric Model | Not present |
| - UE positioning GANSS Earth orientation Parameters | Not present |
| - GANSS Generic Assistance Data |  |
| - GANSS ID | Not present |
| - UE positioning GANSS SBAS ID | Not present |
| - GANSS Time Models | Not present |
| - UE positioning DGANSS corrections | Not present |
| - UE positioning GANSS navigation model | Set according to 4.2 |
| - Non-Broadcast Indication | Not present |
| - Satellite information | For satellites (N+1)-6 |
| - GANSS clock model | Model-1 |
| - GANSS orbit model | Model-1 |
| - UE positioning GANSS additional navigation models | Not present |
| - UE positioning GANSS real-time integrity | Not present |
| - UE positioning GANSS data bit assistance | Not present |
| - UE positioning GANSS reference measurement information | Not present |
| - UE positioning GANSS almanac | Not present |
| - UE positioning GANSS UTC model | Not present |
| - UE positioning GANSS additional UTC models | Not present |
| - UE positioning GANSS auxiliary information | Not present |

#### 4.4.1.3	Sub-Test 3

First MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.1, First Measurement Control Message |
| UE positioning GANSS assistance data |  |
| - UE positioning GANSS reference time | Not present |
| - UE positioning GANSS reference UE position | Not present |
| - UE positioning GANSS ionospheric model | Not present |
| - UE positioning GANSS additional ionospheric Model | Not present |
| - UE positioning GANSS Earth orientation Parameters | Not present |
| - GANSS Generic Assistance Data |  |
| - GANSS ID | 1 (Modernized GPS) |
| - UE positioning GANSS SBAS ID | Not present |
| - GANSS Time Models | Not present |
| - UE positioning DGANSS corrections | Not present |
| - UE positioning GANSS navigation model | Not present |
| - UE positioning GANSS additional navigation models | Not present |
| - UE positioning GANSS real-time integrity | Not present |
| - UE positioning GANSS data bit assistance | Not present |
| - UE positioning GANSS reference measurement information | Not present |
| - UE positioning GANSS almanac | Not present |
| - UE positioning GANSS UTC model | Not present |
| - UE positioning GANSS additional UTC models | Not present |
| - UE positioning GANSS auxiliary information |  |
| - GANSS-ID-1 |  |
| - Aux Info List | For satellites 1-3 |

Second MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.1, Second Measurement Control Message |
| UE positioning GANSS assistance data |  |
| - UE positioning GANSS reference time | Not present |
| - UE positioning GANSS reference UE position | Not present |
| - UE positioning GANSS ionospheric model | Not present |
| - UE positioning GANSS additional ionospheric Model | Not present |
| - UE positioning GANSS Earth orientation Parameters | Not present |
| - GANSS Generic Assistance Data |  |
| - GANSS ID | 1 (Modernized GPS) |
| - UE positioning GANSS SBAS ID | Not present |
| - GANSS Time Models | Not present |
| - UE positioning DGANSS corrections | Not present |
| - UE positioning GANSS navigation model | Not present |
| - UE positioning GANSS additional navigation models | Not present |
| - UE positioning GANSS real-time integrity | Not present |
| - UE positioning GANSS data bit assistance | Not present |
| - UE positioning GANSS reference measurement information | Not present |
| - UE positioning GANSS almanac | Not present |
| - UE positioning GANSS UTC model | Not present |
| - UE positioning GANSS additional UTC models | Not present |
| - UE positioning GANSS auxiliary information |  |
| - GANSS-ID-1 |  |
| - Aux Info List | For satellites 4-6 |

#### 4.4.1.4	Sub-Test 4

First MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.1, First Measurement Control Message |
| UE positioning GANSS assistance data | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.1.3, First Measurement Control Message, UE positioning GANSS assistance data. |

Second MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.1, Second Measurement Control Message |
| - UE positioning GPS UTC model | Set according to 4.2. |
| UE positioning GANSS assistance data | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.1.3, Second Measurement Control Message, UE positioning GANSS assistance data. |

Third MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | 3 (GLONASS) |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Set according to 4.2 |  |
| - GANSS Time Model |  |  |
| - GANSS Time Model Reference Time | Set according to 4.2 |  |
| - TA0 | Set according to 4.2 |  |
| - TA1 | Not present |  |
| - TA2 | Not present |  |
| - GNSS_TO_ID | 0 (GPS) |  |
| - Week Number | Not present |  |
| - Delta_T | Set according to 4.2 | Rel-10 UE or later |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Set according to 4.2 |  |
| - Non-Broadcast Indication | Not present |  |
| - Satellite information | For satellites 1-6 |  |
| - GANSS additional clock models | Model-4 |  |
| - GANSS additional orbit models | Model-4 |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Not present |  |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Set according to 4.2 |  |
| - GANSS-ID-3 |  |  |
| - Aux Info List | For satellites 1-6 |  |

#### 4.4.1.4A	Sub-Test 8

First MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.1, First Measurement Control Message |
| UE positioning GANSS assistance data | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.1.3, First Measurement Control Message, UE positioning GANSS assistance data. |

Second MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.1, Second Measurement Control Message |
| - UE positioning GPS UTC model | Set according to 4.2. |
| UE positioning GANSS assistance data | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.1.3, Second Measurement Control Message, UE positioning GANSS assistance data. |

Third MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | Not present | (Galileo) |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Set according to 4.2 |  |
| - GANSS Time Model |  |  |
| - GANSS Time Model Reference Time | Set according to 4.2 |  |
| - TA0 | Set according to 4.2 |  |
| - TA1 | Not present |  |
| - TA2 | Not present |  |
| - GNSS_TO_ID | 0 (GPS) |  |
| - Week Number | Not present |  |
| - Delta_T | Set according to 4.2 |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Not present |  |
| - Non-Broadcast Indication | Not present |  |
| - Satellite information | For satellites 1-6 |  |
| - GANSS additional clock models | Model 1 |  |
| - GANSS additional orbit models | Model 1 |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Not present |  |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Not present |  |

#### 4.4.1.5	Sub-Test 9

First MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time |  |  |
| - GANSS Day | Set according to 4.2 |  |
| - GANSS Day Cycle Number | Set according to 4.2 |  |
| - GANSS TOD | Set according to 4.2 |  |
| - GANSS TOD Uncertainty | Set according to 4.2 |  |
| - GANSS Time ID | 3 (BDS) |  |
| - UTRAN GANSS reference time | Not present |  |
| - TUTRAN-GANSS drift rate | Not present |  |
| - UE positioning GANSS reference UE position | Set according to 4.2 |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | 4 (BDS) |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Set according to 4.2 |  |
| - Non-Broadcast Indication | Not present |  |
| - Satellite information | For satellites 1-6 |  |
| - GANSS additional clock models | Model 6 |  |
| - GANSS additional orbit models | Model 6 |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Not present |  |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Not present |  |

#### 4.4.1.6	Sub-Test 10

First MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.1, First Measurement Control Message |
| UE positioning GANSS assistance data | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.1.3, First Measurement Control Message, UE positioning GANSS assistance data. |

Second MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.1, Second Measurement Control Message |
| - UE positioning GPS UTC model | Set according to 4.2. |
| UE positioning GANSS assistance data | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.1.3, Second Measurement Control Message, UE positioning GANSS assistance data. |

Third MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | 4 (BDS) |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Set according to 4.2 |  |
| - GANSS Time Model |  |  |
| - GANSS Time Model Reference Time | Set according to 4.2 |  |
| - TA0 | Set according to 4.2 |  |
| - TA1 | Not present |  |
| - TA2 | Not present |  |
| - GNSS_TO_ID | 0 (GPS) |  |
| - Week Number | Not present |  |
| - Delta_T | Set according to 4.2 |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Set according to 4.2 |  |
| - Non-Broadcast Indication | Not present |  |
| - Satellite information | For satellites 1-6 |  |
| - GANSS additional clock models | Model 6 |  |
| - GANSS additional orbit models | Model 6 |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Not present |  |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Not present |  |

### 4.4.2	Inadequate assistance data for UE-based A-GNSS

For UE-based test cases requiring inadequate assistance data, the IE "UE positioning GPS assistance data" and "UE positioning GANSS assistance data" is set to "Not present" in the MEASUREMENT CONTROL message.

### 4.4.3	Adequate assistance data for UE-assisted A-GNSS

For UE-assisted test cases requiring adequate assistance data, the IEs "UE positioning GPS assistance data" and "UE positioning GANSS assistance data" are set as follows:

#### 4.4.3.1	Sub-Test 1

MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time |  |  |
| - GANSS Day | Set according to 4.2 |  |
| - GANSS Day Cycle Number | Set according to 4.2 | Rel-10 UE or later |
| - GANSS TOD | Set according to 4.2 |  |
| - GANSS TOD Uncertainty | Set according to 4.2 |  |
| - GANSS Time ID | 2 (GLONASS) |  |
| - UTRAN GANSS reference time | Not present |  |
| - TUTRAN-GANSS drift rate | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | 3 (GLONASS) |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Not present |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Set according to 4.2 |  |
| - GANSS Signal ID | Not present |  |
| - Satellite Information | For satellites 1-6 |  |
| - Extra Doppler | Set according to 4.2 |  |
| - Azimuth and Elevation | Set according to 4.2 |  |
| - Azimuth and Elevation LSB | Set according to 4.2 | Rel-10 UE or later |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Set according to 4.2 |  |
| - GANSS-ID-3 |  |  |
| - Aux Info List | For satellites 1-6 |  |

#### 4.4.3.2	Sub-Test 2

MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time |  |  |
| - GANSS Day | Set according to 4.2 |  |
| - GANSS Day Cycle Number | Set according to 4.2 | Rel-10 UE or later |
| - GANSS TOD | Set according to 4.2 |  |
| - GANSS TOD Uncertainty | Set according to 4.2 |  |
| - GANSS Time ID | Not present |  |
| - UTRAN GANSS reference time | Not present |  |
| - TUTRAN-GANSS drift rate | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | Not present |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Not present |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Set according to 4.2 |  |
| - GANSS Signal ID | Not present |  |
| - Satellite Information | For satellites 1-6 |  |
| - Extra Doppler | Set according to 4.2 |  |
| - Azimuth and Elevation | Set according to 4.2 |  |
| - Azimuth and Elevation LSB | Set according to 4.2 | Rel-10 UE or later |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Not present |  |

#### 4.4.3.3	Sub-Test 3

MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |
| --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.3 |
| UE positioning GANSS assistance data |  |
| - UE positioning GANSS reference time | Not present |
| - UE positioning GANSS reference UE position | Not present |
| - UE positioning GANSS ionospheric model | Not present |
| - UE positioning GANSS additional ionospheric Model | Not present |
| - UE positioning GANSS Earth orientation Parameters | Not present |
| - GANSS Generic Assistance Data |  |
| - GANSS ID | 1 (Modernized GPS) |
| - UE positioning GANSS SBAS ID | Not present |
| - GANSS Time Models | Not present |
| - UE positioning DGANSS corrections | Not present |
| - UE positioning GANSS navigation model | Not present |
| - UE positioning GANSS additional navigation models | Not present |
| - UE positioning GANSS real-time integrity | Not present |
| - UE positioning GANSS data bit assistance | Not present |
| - UE positioning GANSS reference measurement information | Not present |
| - UE positioning GANSS almanac | Not present |
| - UE positioning GANSS UTC model | Not present |
| - UE positioning GANSS additional UTC models | Not present |
| - UE positioning GANSS auxiliary information | Set according to 4.2 |
| - GANSS-ID-1 |  |
| - Aux Info List | For satellites 1-6 |

#### 4.4.3.4	Sub-Test 4

MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.3 |  |
| - UE positioning GPS UTC model | Set according to 4.2 |  |
| UE positioning GANSS assistance data (1) | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.3.3, Measurement Control Message, UE positioning GANSS assistance data. |  |
| UE positioning GANSS assistance data (2) |  | If UE positioning GANSS assistance data (1) is present, then UE positioning GANSS assistance data (2) is sent in a second MEASUREMENT CONTROL MESSAGE |
| - UE positioning GANSS reference time | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | 3 (GLONASS) |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Not present |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information |  |  |
| - GANSS Signal ID | Not present |  |
| - Satellite Information | For satellites 1-6 |  |
| - Extra Doppler | Set according to 4.2 |  |
| - Azimuth and Elevation | Set according to 4.2 |  |
| - Azimuth and Elevation LSB | Set according to 4.2 | Rel-10 UE or later |
| - UE positioning GANSS auxiliary information | Set according to 4.2 |  |
| - GANSS-ID-3 |  |  |
| - Aux Info List | For satellites 1-6 |  |

#### 4.4.3.4A	Sub-Test 8

MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.3 |  |
| - UE positioning GPS UTC model | Set according to 4.2 |  |
| UE positioning GANSS assistance data (1) | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.3.3, Measurement Control Message, UE positioning GANSS assistance data. |  |
| UE positioning GANSS assistance data (2) |  | If UE positioning GANSS assistance data (1) is present, then UE positioning GANSS assistance data (2) is sent in a second MEASUREMENT CONTROL MESSAGE |
| - UE positioning GANSS reference time | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | Not present | (Galileo) |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Not present |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information |  |  |
| - GANSS Signal ID | Not present |  |
| - Satellite Information | For satellites 1-6 |  |
| - Extra Doppler | Set according to 4.2 |  |
| - Azimuth and Elevation | Set according to 4.2 |  |
| - Azimuth and Elevation LSB | Set according to 4.2 |  |
| - UE positioning GANSS auxiliary information | Not present |  |

#### 4.4.3.5	Sub-Test 9

MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Not present |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time |  |  |
| - GANSS Day | Set according to 4.2 |  |
| - GANSS Day Cycle Number | Set according to 4.2 |  |
| - GANSS TOD | Set according to 4.2 |  |
| - GANSS TOD Uncertainty | Set according to 4.2 |  |
| - GANSS Time ID | 3 (BDS) |  |
| - UTRAN GANSS reference time | Not present |  |
| - TUTRAN-GANSS drift rate | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | 4 (BDS) |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Not present |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Set according to 4.2 |  |
| - GANSS Signal ID | Not present |  |
| - Satellite Information | For satellites 1-6 |  |
| - Extra Doppler | Set according to 4.2 |  |
| - Azimuth and Elevation | Set according to 4.2 |  |
| - Azimuth and Elevation LSB | Set according to 4.2 |  |
| - UE positioning GANSS almanac | Not present |  |
| - UE positioning GANSS UTC model | Not present |  |
| - UE positioning GANSS additional UTC models | Not present |  |
| - UE positioning GANSS auxiliary information | Not present |  |

#### 4.4.3.6	Sub-Test 10

MEASUREMENT CONTROL MESSAGE:

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| UE positioning GPS assistance data | As defined in 4.3.3 |  |
| - UE positioning GPS UTC model | Set according to 4.2 |  |
| UE positioning GANSS assistance data (1) | If for GPS only L1 C/A supported, not present. If multiple GPS signals supported, as defined in 4.4.3.3, Measurement Control Message, UE positioning GANSS assistance data. |  |
| UE positioning GANSS assistance data (2) |  | If UE positioning GANSS assistance data (1) is present, then UE positioning GANSS assistance data (2) is sent in a second MEASUREMENT CONTROL MESSAGE |
| - UE positioning GANSS reference time | Not present |  |
| - UE positioning GANSS reference UE position | Not present |  |
| - UE positioning GANSS ionospheric model | Not present |  |
| - UE positioning GANSS additional ionospheric Model | Not present |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | 4 (BDS) |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Not present |  |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Not present |  |
| - UE positioning GANSS additional navigation models | Not present |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information |  |  |
| - GANSS Signal ID | Not present |  |
| - Satellite Information | For satellites 1-6 |  |
| - Extra Doppler | Set according to 4.2 |  |
| - Azimuth and Elevation | Set according to 4.2 |  |
| - Azimuth and Elevation LSB | Set according to 4.2 |  |
| - UE positioning GANSS auxiliary information | Not present |  |

If the UE requests further assistance data, the SS sends subsequent MEASUREMENT CONTROL messages containing the assistance data fields requested by the UE that are available in the SS as specified in TS 37.571-5 [12] subclause 6.1.3 and in clause 4.4.5.

### 4.4.4	Inadequate assistance data for UE-assisted A-GNSS

For UE-assisted test cases requiring inadequate assistance data, the IEs "UE positioning GPS assistance data" and "UE positioning GANSS assistance data" are set to "Not present" in the MEASUREMENT CONTROL message.

### 4.4.5	Response to assistance data requests from UE

If the SS needs to send assistance data in response to a request for additional assistance data from the UE, or in response to an MO-LR request for assistance data, the IEs "UE positioning GPS assistance data" and "UE positioning GANSS assistance data" are set as follows:

| Information Element | Value/Remark |  |
| --- | --- | --- |
| UE positioning GPS assistance data | Set according to 4.3.5 |  |
| - UE positioning GPS UTC model | Set according to 4.2 |  |
| UE positioning GANSS assistance data |  |  |
| - UE positioning GANSS reference time | Set according to 4.2 |  |
| - GANSS Day | Set according to 4.2 |  |
| - GANSS Day Cycle Number | Set according to 4.2 | Rel-10 UE or later |
| - GANSS TOD | Set according to 4.2 |  |
| - GANSS TOD Uncertainty | Set according to 4.2 |  |
| - GANSS Time ID | Set according to 4.2 |  |
| - UTRAN GANSS reference time | Not present |  |
| - TUTRAN-GANSS drift rate | Not present |  |
| - UE positioning GANSS reference UE position | Set according to 4.2 |  |
| - UE positioning GANSS ionospheric model | Set according to 4.2 |  |
| - UE positioning GANSS additional ionospheric Model | Set according to 4.2 |  |
| - UE positioning GANSS Earth orientation Parameters | Not present |  |
| - GANSS Generic Assistance Data |  |  |
| - GANSS ID | Set according to 4.2 |  |
| - UE positioning GANSS SBAS ID | Not present |  |
| - GANSS Time Models | Set according to 4.2 |  |
| - GANSS Time Model | Set according to 4.2 |  |
| - GANSS Time Model Reference Time | Set according to 4.2 |  |
| - TA0 | Set according to 4.2 |  |
| - TA1 | Not present |  |
| - TA2 | Not present |  |
| - GNSS_TO_ID | Set according to 4.2 |  |
| - Week Number | Not present |  |
| - Delta_T | Set according to 4.2 | Rel-10 UE or later |
| - UE positioning DGANSS corrections | Not present |  |
| - UE positioning GANSS navigation model | Set according to 4.2 |  |
| - UE positioning GANSS additional navigation models | Set according to 4.2 |  |
| - UE positioning GANSS real-time integrity | Not present |  |
| - UE positioning GANSS data bit assistance | Not present |  |
| - UE positioning GANSS reference measurement information | Set according to 4.2 |  |
| - GANSS Signal ID | Set according to 4.2 |  |
| - Satellite Information | Set according to 4.2 |  |
| - Extra Doppler | Set according to 4.2 |  |
| - Azimuth and Elevation | Set according to 4.2 |  |
| - Azimuth and Elevation LSB | Set according to 4.2 | Rel-10 UE or later |
| - UE positioning GANSS almanac | Set according to 4.2 |  |
| - Complete Almanac Provided | True | Rel-10 UE or later |
| - UE positioning GANSS UTC model | Set according to 4.2 |  |
| - UE positioning GANSS additional UTC models | Set according to 4.2 |  |
| - UE positioning GANSS auxiliary information | Set according to 4.2 |  |

If the UE requests the GPS navigation model or the GANSS navigation model Model-1, Model-2, Model-3 or Model 6 then the SS provides navigation model satellite information for at most three satellites in any one MEASUREMENT CONTROL or ASSISTANCE DATA DELIVERY message; additional satellites are sent in subsequent MEASUREMENT CONTROL or ASSISTANCE DATA DELIVERY messages.

If the UE requests the GPS or GANSS almanac then the SS provides almanac information spread across at least two MEASUREMENT CONTROL or ASSISTANCE DATA DELIVERY messages.

If the UE requests both navigation model and almanac then the SS provides them in different MEASUREMENT CONTROL or ASSISTANCE DATA DELIVERY messages.