3GPP TSG-RAN5 Meeting #90-e	R5-211299

# Electronic Meeting, 22nd February - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0281 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability of ‘Re-registration after capability update / 5GS’ test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Adjusting applicability statement and adding PICs for enabling/disabling SMS over IMS since not all UEs support this feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adjusted applicability statement for 34.229-5 TC 6.6 and added PICs for enabling/disabling SMS over IMS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Device that does not support enabling/disabling SMS over IMS will unfairly fail this test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, A.4.5, 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

## 4.2	Applicabilities for test cases specified in TS 34.229-5

Table 4.2: Applicability of TS 34.229-5 tests

| Clause | Title |  | Release | Applicability | Comments |
| --- | --- | --- | --- | --- | --- |
| Registration |  |  |  |  |  |
| 6.1 | Initial Registration / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR (see Note 1 below) |
| 6.2 | Initial Registration Failures / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.3 | Re-Registration Scenarios / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.4 | De-Registration Scenarios / 5GS |  | Rel-15 | C13 | UE supports IMS security and NR and SM-over-IP receiver |
| 6.5 | Refresh for ISIM parameters / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.6 | Re-Registration after capability update / 5GS |  | Rel-15 | C15 | UE supports IMS security and NR and Enabling/Disabling SMS over IMS |
| 6.7 | Authentication / MAC Parameter Invalid / Only two consecutive challenges / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.8 | Authentication / Security Server missing / SQN out of range / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| 6.9 | Subscription / 503 Service Unavailable / 5GS |  | Rel-15 | C01 | UE supports IMS security and NR |
| Call Control |  |  |  |  |  |
| 7.1 | MTSI MO Voice Call / 503 Service Unavailable / 5GS | Rel-15 |  | C03 | UE supports IMS security and NR and MTSI and MTSI speech and initiating a session |
| 7.2 | MTSI MO Voice Call / 503 Service Unavailable / 5GS | Rel-15 |  | C03 | UE supports IMS security and NR and MTSI and MTSI speech and initiating a session |
| 7.3 | MTSI MO Voice Call / 421 Extension Required / 5GS | Rel-15 |  | C03 | UE supports IMS security and NR and MTSI and MTSI speech and initiating a session |
| 7.4 | MTSI MO Voice call with preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C02 | UE supports IMS security and NR and MTSI and MTSI speech and initiating a session and preconditions |
| 7.5 | MTSI MO Voice call without preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C02 | UE supports IMS security and NR and MTSI and MTSI speech and initiating a session and preconditions |
| 7.6 | MTSI MT Voice Call with preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C04 | UE supports IMS security and NR and MTSI and MTSI speech and preconditions |
| 7.7 | MTSI MT Voice Call without preconditions at both originating UE and terminating UE / 5GS |  | Rel-15 | C04 | UE supports IMS security and NR and MTSI and MTSI speech and preconditions |
| 7.8 | MTSI MT Voice Call without preconditions at originating UE and with preconditions at terminating UE / 5GS | Rel-15 |  | C04 | UE supports IMS security and NR and MTSI and MTSI speech and preconditions |
| 7.9 | MTSI MT Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS | Rel-15 |  | C05 | UE supports IMS security and NR and MTSI and MTSI speech |
| 7.10 | MTSI MT Voice call without preconditions and without SDP offer in MT INVITE / 5GS | Rel-15 |  | C05 | UE supports IMS security and NR and MTSI and MTSI speech |
| 7.11 | MTSI MT Voice call without preconditions at terminating UE and originating UE requiring them / 5GS | Rel-15 |  | C05 | UE supports IMS security and NR and MTSI and MTSI speech |
| 7.12 | MTSI MO Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS | Rel-15 |  | C02 | UE supports IMS security and NR and MTSI and MTSI speech and initiating a session and preconditions |
| Supplementary Services |  |  |  |  |  |
| 8.1 | Originating Identification Presentation / Configuration / 5GS |  | Rel-15 | C06 | UE supports IMS security and NR and MTSI and MTSI Originating Identification Presentation and GBA for XCAP authentication |
| 8.18 | Barring of All Incoming Calls / except for a specific user / 5GS |  | Rel-15 | C07 | UE supports IMS security and NR and MTSI and MTSI Communication Barring and GBA for XCAP authentication |
| SMS |  |  |  |  |  |
| 9.1 | Mobile Originating SMS / 5GS |  | Rel-15 | C08 | UE supports IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| 9.2 | Mobile Terminating SMS / 5GS |  | Rel-15 | C09 | UE supports IMS security and NR and SM-over-IP receiver and UE configured to use SMS over IP |
| 9.3 | Mobile Originating Concatenated SMS / 5GS |  | Rel-15 | C10 | UE supports IMS security and NR and concatenated SM-over-IP sender and UE configured to use SMS over IP |
| 9.4 | Mobile Terminating Concatenated SMS / 5GS |  | Rel-15 | C11 | UE supports IMS security and NR and concatenated SM-over-IP receiver and UE configured to use SMS over IP |
| 9.5 | Mobile Originating SMS / RP-ERROR / 5GS |  | Rel-15 | C08 | UE supports IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| Emergency Calls |  |  |  |  |  |
| 10.1 | Emergency Call with emergency registration / Success / Location information available / 5GS |  | Rel-15 | C12 | UE supports IMS security and NR and IMS emergency services and is capable of obtaining location information |

NOTE 1:	If any one of the other Registration test cases 6.2 until 6.9 is executed, this test case is optional.

| Conditions/Options |  |  |
| --- | --- | --- |
| C01 | IF A.6a/2 AND A.18/5 THEN R ELSE N/A | IMS security and NR |
| C02 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.4/2B AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and initiating a session and preconditions |
| C03 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.4/2B THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and initiating a session |
| C04 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 AND A.4/16 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech and preconditions |
| C05 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.15/1 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI speech |
| C06 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.16/1 AND A.6a/3 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI Originating Identification Presentation and GBA for XCAP authentication |
| C07 | IF A.6a/2 AND A.18/5 AND A.3A/50 AND A.16/7 AND A.6a/3 THEN R ELSE N/A | IMS security and NR and MTSI and MTSI Communication Barring and GBA for XCAP authentication |
| C08 | IF A.6a/2 AND A.18/5 AND A.3A/61 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and SM-over-IP sender and UE configured to use SMS over IP |
| C09 | IF A.6a/2 AND A.18/5 AND A.3A/62 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and SM-over-IP receiver and UE configured to use SMS over IP |
| C10 | IF A.6a/2 AND A.18/5 AND A.3A/64 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and concatenated SM-over-IP sender and UE configured to use SMS over IP |
| C11 | IF A.6a/2 AND A.18/5 AND A.3A/65 AND [73] A.4.4-2/32 THEN R ELSE N/A | IMS security and NR and concatenated SM-over-IP receiver and UE configured to use SMS over IP |
| C12 | IF A.6a/2 AND A.18/5 AND A.12/26 AND A.12/27 THEN R ELSE N/A | IMS security and NR and IMS emergency services and is capable of obtaining location information |
| C13 | IF A.6a/2 AND A.18/5 AND A.3A/62 THEN R ELSE N/A | IMS security and NR and SM-over-IP receiver |
| C15 | IF A.6a/2 AND A.18/5 AND A.12/58 THEN R ELSE N/A | IMS security and NR and Enabling/Disabling SMS over IMS |

# *** Next Change ***

### A.4.5	Additional information

Table A.12: Additional information

| Item | Additional information | Reference | Status | Release | Mnemonic | Support |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Void |  |  |  |  |  |
| 2 | Void |  |  |  |  |  |
| 3 | Void |  |  |  |  |  |
| 4 | UE capable of being configured to initiate Dedicated PDP Context | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiatesDedicatedPDPContext |  |
| 5 | UE capable of being configured to initiate P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaPCO |  |
| 6 | Void |  |  |  |  |  |
| 7 | UE capable of being configured to initiate P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_InitiateP_CSCFDiscovery_viaDHCPv6 |  |
| 8 | UE supports P-CSCF discovery via PCO | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaPCO |  |
| 9 | Void |  |  |  |  |  |
| 10 | UE supports P-CSCF discovery via DHCPv6 | 24.229 [10], 9.2.1 | o | Rel-5 | pc_P_CSCFDiscovery_viaDHCPv6 |  |
| 11 | Void |  |  |  |  |  |
| 12 | UE capable of initiating a bidirectional voice session over IMS | 24.229 [10], 5.1.6 | o | Rel-5 | pc_BidirecVoiceOverIMS |  |
| 13 | Void |  |  |  |  |  |
| 14 | Void |  |  |  |  |  |
| 15 | Void |  |  |  |  |  |
| 16 | UE Supports " IPv6 address with embedded IPv4 address" in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 17 | UE Supports IPv4 address in PCO IE | 23.981 [18], 5.2.1 | o | Rel-6 |  |  |
| 18 | Void |  |  |  |  |  |
| 19 | UE supports UI capable of showing user notification for Message Waiting Indication | 24.173 [55], Annex F | o | Rel-7 |  |  |
| 20 | Void |  |  |  |  |  |
| 21 | Void |  |  |  |  |  |
| 22 | Void |  |  |  |  |  |
| 23 | Void |  |  |  |  |  |
| 24 | UE supports no reply timer setting | 24.604 [68], 4.9.1.4 | o | Rel-8 |  |  |
|  |  | IR.92 [83], 2.3.8 | m |  |  |  |
| 25 | UE supports sending DTMF events over RTP | 26.114 [56], Annex G | o | Rel-7 |  |  |
|  |  | IR.92 [83], 3.3 | m |  |  |  |
| 26 | UE supports IMS emergency services | 24.229 [10], 5.1.6 | o | Rel-9 | pc_IMS_EmergencyCall |  |
| 27 | UE is capable of obtain ing location information | 24.229 [10] 4.7, 36.509 [79] 4.1, 5.5.2, 34.109 [78] 5.4.2 | o | Rel-9 | pc_IMS_Geolocation |  |
| 28 | Void |  |  |  |  |  |
| 29 | UE supports emergency speech call over 1xRTT | C.S0005-E [74] | o |  | pc_CS_Em_Call_in_1xRTT |  |
| 30 | Void |  |  |  |  |  |
| 31 | UE supports end-to-access-edge media security using SDES | 24.229 [10], 4.2.B.2 | o | Rel-9 |  |  |
| 32 | UE supports video media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | o | Rel-13 | pc_IMS_Video_FeatureTag |  |
|  |  | IR.94 [75], 2.2 | m |  |  |  |
| 33 | UE supports Communication Hold during emergency call | 23.167 [76], 7.1.1 | o | Rel-9 | pc_CommunicationHold_DuringEmergencyCall |  |
| 34 | UE indicates g.3gpp.srvcc-alerting media feature tag in INVITE request or 180 (Ringing) response | 24.237 [77], 12.2.3B.1 | o | Rel-10 | pc_IMS_SRVCCAlert |  |
| 35 | Void |  |  |  |  |  |
| 36 | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request | 24.237 [77] 6A.2.2.2 | o | Rel-12 | pc_BSRVCC |  |
| 37 | UE indicates OMA-TS-XDM_MO-V1_1-20080627-A.doc, section 5.2.8 "Node: /<X>/ AAUTHNAME" is configured | 24.623 [81], Annex B.2 | o | Rel-8 | pc_XCAP_UsernameIsConfiguredInUE |  |
| 38 | The UE use the default public user identity received in P-Associated -URI header in 200 OK for REGISTER as XCAP User Identity (XUI) | IR.92 [83] | m |  | pc_XCAP_XUIisDefaultPublicUserId |  |
|  |  | 24.623 [81] | o | Rel-8 |  |  |
| 39 | UE has the method that support IMS deregistration | 24.229 [10], 5.1.1.6 | o | Rel-5 | pc_IMS_Deregistration |  |
| 40 | UE supports Cs to PS SRVCC | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCC |  |
| 41 | UE supports Cs to PS SRVCC in alerting state | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCAlert |  |
| 42 | UE supports Cs to PS SRVCC and the MSC server assisted mid-call feature | 24.237 [77], 6.2.3 | o | Rel-11 | pc_IMS_CS_PS_SRVCCMidCall |  |
| 43 | Void |  |  |  |  |  |
| 44 | Void |  |  |  |  |  |
| 45 | UE supports early media | 24.229 [10], 5.1.3 | o | Rel-8 | pc_EarlyMedia |  |
| 46 | UE indicates g.3gpp.accesstype media feature tag in REGISTER | 24.237 [77], 6.2.2 | c1 | Rel-11 |  |  |
| 47 | Void |  |  |  |  |  |
| 48 | Void |  |  |  |  |  |
| 49 | The UE uses XCAP without PDN connection in WLAN | IR.51 [84], 4.6 | o | Rel-11 | pc_WLAN_XCAP_without_PDN | If PICS is true, Wi-Fi access without PDN connection applies. Otherwise, EPC-integrated WLAN applies |
| 50 | The UE uses XCAP in fixed broadband access |  | o | Rel-12 | pc_fixed_broadband_XCAP | In the context of the present specification, a UE supporting fixed broadband, A.18/3, shall set this PICS to true. |
| 51 | The UE uses same public user Identity for From/To as in initial REGISTER | 24.229 [10] | o | Rel-8 | pc_Same_IMPU |  |
|  |  | IR.92 [83], 2.2.1 | m |  |  |  |
| 52 | UE reattempts IMS emergency service without registration after rejection of emergency registration. (For Rel-14, this is applicable only when the rejection indicates an anonymous emergency call support) | 23.167 [76], 4.1 | o | Rel-9 | pc_IMS_after_EmReg_rejection |  |
|  |  |  | m | Rel-14 |  |  |
| 53 | UE supports SIP session timer | 24.229 [10], RFC 4028 [86] | o | Rel-8 |  |  |
|  |  | IR.92 [83], 2.2.8 | m |  |  |  |
| 54 | UE supports IMS eCall type of emergency services | 24.229 [10], 5.1.6.11 | o | Rel-14 | pc_IMS_eCall_Capable |  |
| 55 | UE supports IMS eCall Only type of emergency services | 24.229 [10], 5.1.6.11 | o | Rel-14 | pc_IMS_eCall_Only |  |
| 56 | UE supports audio media feature tag | 24.229 [10], 5.1.1.2.1, 5.1.3.1, 5.1.4 | m | Rel-13 | pc_IMS_Audio_FeatureTag |  |
|  |  | IR.92 [83], 2.2.1, 2.2.4 | m |  |  |  |
| 57 | UE supports Session Timer | 24.229 [10], 5.1.2A, 5.2.7.2, 5.2.7.3, 5.4.5.3 | o | Rel-6 | pc_IMS_Session_Timer |  |
|  |  | IR.92 [83], 2.2.8 | m |  |  |  |
| 58 | UE supports enabling/disabling SMS over IMS | 27.007 [97], 8.72 | o | Rel-15 | pc_Enable_Disable_SMSoverIMS |  |
| Conditions/Options |  |  |  |  |  |  |
| c1: IF [73] A.4.4-1/32 THEN m ELSE o - - SC UE indicates accesstype. |  |  |  |  |  |  |

# *** Next Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[3]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[4]	3GPP TS 34.123-3: "User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[5]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[6]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[7]	ISO/IEC 9646-1: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[8]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[9]	ETSI ETS 300 406: "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[10]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[11]	Void.

[12]	3GPP TS 33.203: "Access security for IP-based services".

[13]	3GPP TS 23.221: "Architectural requirements".

[14]	Void.

[15]	RFC 3261: "SIP: Session Initiation Protocol".

[16]	Void.

[17]	3GPP TS 24.247: "Messaging using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[18]	3GPP TR 23.981: "Interworking aspects and migration scenarios for IPv4-based IP Multimedia Subsystem (IMS) implementations".

[19]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[20]	Void.

[21]	Void.

[22]	Void.

[23]	Void.

[24]	Void.

[25]	Void.

[26]	RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)".

[27]	RFC 3262: "Reliability of provisional responses in Session Initiation Protocol (SIP)".

[28]	Void.

[29]	Void.

[30]	Void.

[31]	Void.

[32]	Void.

[33]	Void.

[34]	Void.

[35]	Void.

[36]	Void.

[37]	Void.

[38]	Void.

[39]	Void.

[40]	Void.

[41]	Void.

[42]	Void.

[43]	Void.

[44]	Void.

[45]	Void.

[46]	Void.

[47]	Void.

[48]	Void.

[49]	Void.

[50]	Void.

[51]	Void.

[52]	Void.

[53]	Void.

[54]	Void.

[55]	3GPP TS 24.173: "IMS Multimedia Telephony Communication Service and supplementary services; stage 3".

[56]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[57]	Void.

[58]	Void.

[59]	Void.

[60]	Void.

[61]	Void.

[62]	Void.

[63]	3GPP TS 33.222: “Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)”.

[64]	3GPP TS 24.109: “Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details”.

[65]	RFC 2617; “HTTP Authentication: Basic and Digest Access Authentication”.

[66]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3".

[67]	Void.

[68]	3GPP TS 24.604: “Communication Diversion (CDIV) using IP Multimedia (IM)”.

[69]	3GPP TS 24.615: "Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem".

[70]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”.

[71]	3GPP TR 21.904: "UE capability requirements".

[72]	Void.

[73]	3GPP TS 36.523-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS)proforma specification".

[74]	3GPP2 C.S0005-E: “Upper Layer (Layer 3) Signalling Standard for cdma2000 Spread Spectrum Systems”.

[75]	GSMA PRD IR.94: "IMS Profile for Conversational Video Service".

[76]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[77]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[78]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[79]	3GPP TS 36.509: "Special conformance testing functions for User Equipment (UE)".

[80]	Void.

[81]	3GPP TS 24.623: "Extensible Markup Language (XML) Configuration Access Protocol (XCAP) over the Ut interface for Manipulating Supplementary Services".

[82]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[83]	GSMA PRD IR.92: "IMS Profile for Voice and SMS".

[84]	GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi".

[85]	3GPP TS 24.238: "Session Initiation Protocol (SIP) based user configuration; Stage3".

[86]	IETF RFC 4028 (April 2005): "Session Timers in the Session Initiation Protocol (SIP)".

[87]	GSMA PRD NG.108: "IMS Profile for Voice and SMS for UE category M1".[88]	IETF RFC 8147 (May 2017): "Next-Generation Pan-European eCall".

[88]	FFS

[89]	GSMA PRD NG.102: "IMS Profile for Converged IP Communications".

[90]	3GPP TS 24.390: " Unstructured Supplementary Service Data (USSD) using IP Multimedia (IM) Core Network (CN) subsystem IMS; Stage 3"

[91]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[92]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[93]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma specification".

[94]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[95]	3GPP TS 34.229-5: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)".

[96]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[97]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

# *** Next Change ***