3GPP TSG-RAN5 Meeting #90-e	R5-211338

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1741 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PC5-RRC message UECapabilityEnquirySidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-2-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add default configuration for PC5-RRC message UECapabilityEnquirySidelink |  |  |  |  |  |  |  |  |
| Summary of change: |  | Default configuration for PC5-RRC message UECapabilityEnquirySidelink is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-210716r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	UECapabilityEnquirySidelink

Table 4.6.1A-6: UECapabilityEnquirySidelink

| Derivation Path: TS 38.331 [6], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityEnquirySidelink ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier-r16 |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueCapabilityEnquirySidelink-r16 SEQUENCE { |  |  |  |  |
| frequencyBandListFilterSidelink-r16 |  | Not checked |  |  |
| ue-CapabilityInformationSidelink-r16 |  | Not present |  |  |
|  |  | OCTET STRING containing UECapabilityInformationSidelink specified in Table Table 4.6.1A-7 |  | TWO_WAY_ENQUIRY |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| criticalExtensionsFuture |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| TWO_WAY_ENQUIRY | For two-way SL UE capability enquiry procedure |

<End of modified section 1>