3GPP TSG-RAN WG5 Meeting #90-e	R5-211373

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1762 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of procedure for clearing UEs Last Registered PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To clear UE’s Last Registered PLMN (RPLMN), Registration Reject procedure having cause #6 (Illegal ME) defined in cl 4.9.8 was referred in multiple test cases of 38.523-1. However, per 24.501, cause #6 (Illegal ME) does not actually clear UE’s RPLMN. [24.501] #6 (Illegal ME). The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, last visited registered TAI, TAI list and ngKSI. In case of PLMN, the UE shall consider the USIM as invalid for 5GS services until switching off or the UICC containing the USIM is removed; In case of SNPN, the UE shall consider the entry of the "list of subscriber data" with the SNPN identity of the current SNPN as invalid until the UE is switched off or the entry is updated. Additionally, if EAP based primary authentication and key agreement procedure using EAP-AKA' or 5G AKA based primary authentication and key agreement procedure was performed in the current SNPN, the UE shall consider the USIM as invalid for the current SNPN until switching off or the UICC containing the USIM is removed. The UE shall delete the list of equivalent PLMNs (if any) and enter the state 5GMM-DEREGISTERED.NO-SUPI. If the message has been successfully integrity checked by the NAS, then the UE shall: 1) set the counter for "SIM/USIM considered invalid for GPRS services" events and the counter for "USIM considered invalid for 5GS services over non-3GPP access" events in case of PLMN; or 2) set the counter for "the entry for the current SNPN considered invalid for 3GPP access" events and the counter for "the entry for the current SNPN considered invalid for non-3GPP access" events in case of SNPN; to a UE implementation-specific maximum value. 3) delete the 5GMM parameters stored in non-volatile memory of the ME as specified in annex C. If the message was received via 3GPP access and the UE is operating in single-registration mode, the UE shall handle the EMM parameters EMM state, EPS update status, 4G-GUTI, last visited registered TAI, TAI list and eKSI as specified in 3GPP TS 24.301 [15] for the case when the EPS attach request procedure is rejected with the EMM cause with the same value. The USIM shall be considered as invalid also for non-EPS services until switching off or the UICC containing the USIM is removed. If the message has been successfully integrity checked by the NAS and the UE maintains a counter for "SIM/USIM considered invalid for non-GPRS services", then the UE shall set this counter to a UE implementation-specific maximum value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added procedure 4.9.8A for clearing UEs last registered PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE with stored RPLMN may unfairly fail the test case by camping on to it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.8A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-211373 is the final version of R5-211038 with below change: Removed (RPLMN) and “‘” from title. |  |  |  |  |  |  |  |  |

<Start of modified section>

### 4.9.8A	Procedure for clearing UE’s Last Registered PLMN (RPLMN)

4.9.8A.1	Scope

The purpose of this procedure is to ensure UE’s last registered PLMN (RPLMN) is empty, which may allow fields to be cleared in the USIM.

4.9.8A.2	Procedure description

4.9.8A.2.1	Initial conditions

As specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.8A.2.2	Procedure sequence

Table 4.9.8A.2.2-1: Procedure for clearing UE’s RPLMN

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | Power on the UE | - | - |
| 2 | Clear last registered PLMN of UE (Note 1) | - | - |
| 3 | Test procedure for Switch off / Power off in State DEREGISTERED as specified in subclause 4.9.6.4 | - | - |
| Note 1: MMI command “CLEAR_RPLMN” to be used. |  |  |  |

4.9.8A.2.3				Specific message contents

None.

<End of modifed section>