3GPP TSG-RAN5 Meeting #90-e	R5-211386

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2014 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR TC 7.1.2.3.3 and 7.1.2.3.4-RLC SN sequence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm CDMA Technologies, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, it’s easier for reading to combine Note 3 into Note 4. 2, The "and it shall be repeated FLOOR(Maximum_RLC_SN/iteration_size)" in Note 4 should be deleted, because it is redundant with EXCEPTION above step 1 and may cause confusion, i.e: to repeat FLOOR(Maximum_RLC_SN/iteration_size) times or to repeat FLOOR(Maximum_RLC_SN/iteration_size)+1 times? |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Combine Note 3 into Note 4. 2, Delete "and it shall be repeated FLOOR(Maximum_RLC_SN/iteration_size)" in Note 4. 3, Some other editorial modification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There maybe some confusion of reading this TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.3, 7.1.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-210640r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.2.3.3	AM RLC / 12-bit SN / Correct use of sequence numbering

7.1.2.3.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }
ensure that {

when { UE transmits the PDU corresponding to first SDU }
    then { UE includes the SN field equal to 0 in PDU }
            }

(2)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }
ensure that {

when{ UE transmits subsequent SDUs }
    then { UE includes the SN field incremented by 1 per SDU of each PDU transmitted }
            }

(3)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }
ensure that {

with { UE transmits more than 4096 SDUs}
    then { UE wraps the SN after transmitting the 4096 SDUs}

}

(4)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN
ensure that {

with { more than 4096 SDUs are sent to UE }
    then { UE accepts PDUs with SNs that wrap around every 4096 SDUs }

}

7.1.2.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.2.3.1.1, 5.2.3.2.1, 5.2.3.2.2, 6.2.2.4 and 7.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.2.3.2.1]

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see sub clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see sub clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly as needed (see sub clause 5.2.3.2.4).

[TS 38.322, clause 5.2.3.2.2]

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

[TS 38.322, clause 6.2.2.4]

AMD PDU consists of a Data field and an AMD PDU header. The AMD PDU header is byte aligned.

An AM RLC entity is configured by RRC to use either a 12 bit SN or a 18 bit SN. The length of the AMD PDU header is two and three bytes respectively.

An AMD PDU header contains a D/C, a P, a SI, and a SN. An AMD PDU header contains the SO field only when the Data field consists of an RLC SDU segment which is not the first segment, in which case a 16 bit SO is present.

Figure 6.2.2.4-1: AMD PDU with 12 bit SN (No SO)

Figure 6.2.2.4-2: AMD PDU with 18 bit SN (No SO)

Figure 6.2.2.4-3: AMD PDU with 12 bit SN with SO

Figure 6.2.2.4-4: AMD PDU with 18 bit SN with SO

[TS 38.322, clause 7.1]

c) RETX_COUNT – Counter

This counter counts the number of retransmissions of an RLC SDU or RLC SDU segment (see subclause 5.3.2). There is one RETX_COUNT counter maintained per RLC SDU.

The receiving side of each AM RLC entity shall maintain the following state variables:

a) RX_Next – Receive state variable

This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

b) RX_Next_Status_Trigger – t-Reassembly state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Reassembly.

c) RX_Highest_Status – Maximum STATUS transmit state variable

This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

d) RX_Next_Highest – Highest received state variable

This state variable holds the value of the SN following the SN of the RLC SDU with the highest SN among received RLC SDUs. It is initially set to 0.

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables and constant:

b) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0.

c) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

d) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0.

7.1.2.3.3.3	Test description

7.1.2.3.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.3.3.1-1.

Table 7.1.2.3.3.3.1-1: RLC parameters

| Uplink SN-FieldLength-AM | size12 |
| --- | --- |
| Downlink SN-FieldLength-AM | size12 |
| pollPDU | p2048 |
| pollByte | kB25 |

7.1.2.3.3.3.2	Test procedure sequence

Table 7.1.2.3.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 – 3a1 shall be repeated from j=0 to j= FLOOR(Maximum_RLC_SN/iteration_size). (Note 1, 4, 6) | - | - | - | - |
| 1 | SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. (Note 4, 5). Each RLC PDU contains one RLC SDU. | <-- | RLC PDUs | - | - |
| - | EXCEPTION: In Step 2, SS may receive a RLC PDU or several RLC PDUs, then step 2 may be repeated multiple times until all RLC PDUs with SN=j*iteration_size to SN=(((j+1)*iteration_size)-1) for each iteration are received. | - | - | - | - |
| 2 | CHECK: Does UE transmit RLC PDUs with SN=0 for the first iteration and all RLC PDUs for each iteration? (Note 2) | --> | RLC PDUs | 1,2 | P |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the contents of the AMD PDU transmitted at Step 2. | - | - | - | - |
| 3a1 | IF the UE has set the poll bit in the AMD PDU transmitted at Step 2 THEN the SS transmits a Status Report. | <-- | STATUS PDU | - | - |
| 4 | SS transmits a RLC PDU containing one RLC SDU. | <-- | RLC PDU | - | - |
| 5 | CHECK: Does UE transmit a RLC PDU with SN=0? | --> | RLC PDU | 3,4 | P |
| 6 | The SS transmits a STATUS PDU with ACK_SN = 1. | <-- | STATUS PDU | - | - |
| Note 1: Maximum_RLC_SN = 2[RLC-SN-SizeUL] -1. Note 2: The verdict shall be provided each time [(SN+1) mod 256 = 0] for 12 bit SN and [(SN+1) mod 4096 = 0] for 18 bit SN respectively. Note 3: Void Note 4: Iteration will be incremented by iteration_size of 1 for 12 bit SN and iteration_size of 25 for 18 bit SN. Small RLC SDU size will be used. Note 5: SS shall transmit a RLC PDU list with size equal to iteration_size and incrementing SN by 1 till SN = ((j + 1) * iteration_size)-1. Note 6: The RLC SDU size shall be 4 octets(3 octets of PDCP header + 1 octet PDCP SDU). |  |  |  |  |  |

7.1.2.3.3.3.3	Specific message contents

None.

7.1.2.3.4	AM RLC / 18-bit SN / Correct use of sequence numbering

7.1.2.3.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN }
ensure that {

when { UE transmits the PDU corresponding to first SDU }
    then { UE includes the SN field equal to 0 in PDU }
            }

(2)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN }
ensure that {

when{ UE transmits subsequent SDUs }
    then { UE includes the SN field incremented by 1 per SDU of each PDU transmitted }
            }

(3)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN }
ensure that {

with { UE transmits more than 262144 SDUs }
    then { UE wraps the SN after transmitting the 262144 SDUs }

}

(4)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN
ensure that {

with { more than 262144 SDUs are sent to UE }
    then { UE accepts PDUs with SNs that wrap around every 262144 SDUs }

}

7.1.2.3.4.2	Conformance requirements

Same as conformance requirements in clause 7.1.2.3.3.2

7.1.2.3.4.3	Test description

7.1.2.3.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.4.3.1-1.

Table 7.1.2.3.4.3.1-1: RLC parameters

| Uplink SN-FieldLength-AM | size18 |
| --- | --- |
| Downlink SN-FieldLength-AM | size18 |
| pollPDU | p2048 |
| pollByte | kB25 |

7.1.2.3.4.3.2	Test procedure sequence

Same as test procedure in clause 7.1.2.3.3.3.2

7.1.2.3.4.3.3	Specific message contents

None.

<End of modified section 2>