3GPP TSG-RAN5 Meeting #90-e	R5-211444

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0111 | rev | 1 | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR IMS TC 8.32-Inviting user to conference by REFER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently there is no test coverage to verfiy that an UE could correctly handle inviting user to conference by REFER. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TC 8.32 for inviting user to conference by REFER. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No testing of UE compliance for such use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.32 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2... CR 0278... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This document is the outcome of R5-210666r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.32	Inviting user to conference by sending a REFER request to the conference focus / 5GS

8.32.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to start a conference call }

then { UE sends INVITE to the conference factory and completes the conference call initiation and subscribes to conference event }

}

(2)

with { Conference call going on }

ensure that {

when { UE is made to invite another user to the conference call }

then { UE sends REFER to the conference focus }

}

(3)

with { UE having invited another user to the conference call }

ensure that {

when { UE receives 202 Accepted followed by notification messages for the REFER request, the confirmation on the other user and conditional conference event package }

then { UE sends 200 OK for each received NOTIFY request }

}

(4)

with { Conference call going on }

ensure that {

when { UE is made to leave the call }

then { UE sends BYE and processes notification for conf event if any }

}

8.32.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.147, clause 5.3.1.5.3]:

Upon generating a REFER request in accordance with the procedures specified in 3GPP TS 24.229 [5], IETF RFC 3515 [17] as updated by IETF RFC 6665 [10] and IETF RFC 7647 [39] that is destined to the conference focus in order to invite another user to a specific conference, the conference participant shall:

1)	set the request URI of the REFER request to the conference URI to which the user is invited to;

2)	set the Refer-To header of the REFER request to the SIP URI or tel URL of the user who is invited to the conference;

3)	either include the "method" URI parameter with the value "INVITE" or omit the "method" URI parameter in the Refer-To header; and

NOTE:	Other headers of the REFER request will be set in accordance with 3GPP TS 24.229 [5].

4)	send the REFER request towards the conference focus that is hosting the conference.

The UE may additionally include the Referred-By header to the REFER request and set it to the URI of the conference participant that is sending the REFER request.

In case of an active session the UE may additionally include the Replaces header in the header portion of the SIP URI of the Refer-to header field of the REFER request. If the user involved in the active session is identified by a tel URI, the UE shall convert the tel URI to an SIP URI as described in RFC 3261 [7] before including the Replaces header field. The included Replaces header field shall refer to the active dialog that is replaced by the ad-hoc conference. The Replaces header field shall comply with RFC 3891 [33].

Afterwards the UE shall treat incoming NOTIFY requests that are related to the previously sent REFER request in accordance with RFC 3515 [17] as updated by RFC 6665 [10] and may indicate the received information to the user.

8.32.3	Test description

8.32.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

-	SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1) and registered to IMS.

8.32.3.2	Test procedure sequence

Table 8.32.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is made to attempt an IMS Conference call | - | - | - | - |
| 2 | Check: Does the UE send INVITE with the first SDP offer? | --> | INVITE | 1 | P |
| 3-8 | Steps 2-7 of annex A.4.1 happen | - | - | - | - |
| 9 | The SS responds INVITE with 200 OK and gives the final conference URI within the response | <-- | 200 OK | - | - |
| 10 | The UE acknowledges the receipt of 200 OK for INVITE | --> | ACK | - | - |
| 11 | EXCEPTION: steps 11 – 14 describe optional behaviour depending on UE configuration. The SS shall wait up to 3s for the SUBSCRIBE of step 11 | - | - | - | - |
| 12 | UE subscribes the conference event | --> | SUBSCRIBE | - | - |
| 13 | SS responds to the subscription | <-- | 200 OK | - | - |
| 14 | The UE is made to invite another user to the conference | - | - | - | - |
| 15 | The UE sends REFER to SS referring to the conference | --> | REFER | 2 | P |
| 16 | The SS responds with a 202 final response | <-- | 202 Accepted | - | - |
| 17 | The SS sends initial NOTIFY for the implicit subscription created by the REFER request | <-- | NOTIFY | - | - |
| 18 | The UE responds the NOTIFY with 200 OK | --> | 200 OK | 3 | P |
| 19 | The SS sends a NOTIFY related to REFER request to confirm that the invited user was able to join the conference | <-- | NOTIFY |  |  |
| 20 | The UE responds the NOTIFY with 200 OK | --> | 200 OK | 3 | P |
| 21 | Optional: If the UE has subscribed the conference event package, the SS sends a NOTIFY for conference event package to inform that the invited user was able to join the conference | <-- | NOTIFY | - | - |
| 22 | Optional: The UE responds the NOTIFY with 200 OK | --> | 200 OK | 3 | P |
| 23 | The UE is made to leave the conference | - | - | - | - |
| 24 | The UE sends BYE to ler leaving the conference |  | BYE | 4 | P |
| 25 | The SS sends 200 OK for BYE | <-- | 200 OK | - | - |
| 26 | If the UE had subscribed to the conference event package, the SS notifies the UE that its subscription to conference event package is terminated | --> | NOTIFY | - | - |
| 27 | The UE sends 200 OK for NOTIFY (if sent by SS) | --> | 200 OK | - | - |

8.32.3.3	Specific message contents

Table 8.32.3.3-1: INVITE (step 2, table 8.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Step 2 of C.21, Condition A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |  |  |
| To addr-spec |  | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |  |  |

Table 8.32.3.3-2: 200 OK for INVITE (step 9, table 8.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1, Condition A22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Record-Route rec-route |  | Same value as in the 183 response |  |  |
| Contact addr-spec feature-param |  | sip:final@conf-factory appended with px_IMS_HomeDomainName isfocus |  |  |

Table 8.32.3.3-3: ACK (step 10, table 8.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Step 2 of C.21, Condition A28 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | sip:final@conf-factory appended with px_IMS_HomeDomainName |  |  |

Table 8.32.3.3-4: SUBSCRIBE for conference event package (step 11, table 8.32.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.7, Condition A1

Table 8.32.3.3-5: 200 OK for SUBSCRIBE (step 12, table 8.32.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A5.2

Table 8.32.3.3-6: REFER (step 15, table 8.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.10, Condition A5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Request-Line Request-URI |  | sip:final@conf-factory appended with px_IMS_HomeDomainName |  |  |
| Refer-To addr-spec |  | SIP URI or tel URI of the user invited to the conference. If an active session exists, the Replaces header in the header portion of the SIP URI shall be included (mandatory inclusion is stated in IR.92 ) and set to the dialog ID of the active session according to RFC 3891. In this case, if the user has been invited with a tel URI, the UE shall convert the tel URI to a SIP URI according to RFC 3261 [6] clause 19.1.6. (NOTE: the dialog ID is percent encoded according to RFC 3986). |  |  |
| To addr-spec tag |  | remote SIP URI as used in To header in step 12 remote tag of the dialog with the conference focus created in step 12 |  |  |
| Route Route-param |  | URIs of the Record-Route header of 183 response sent in step 4 (step 3 of A.4.1) |  |  |

Table 8.32.3.3-7: 202 Accepted (step 16, table 8.32.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.3.3

Table 8.32.3.3-8: NOTIFY (step 17, table 8.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | SIP/2.0 100 Trying |  |  |

Table 8.32.3.3-9: 200 OK (step 18/20/22/27, table 8.32.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.3.1, Condition A22

Table 8.32.3.3-10: NOTIFY (step 19, table 8.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Subscription-State substate-value expires reason |  | terminated omitted from the request noresource |  |  |
| Message-body |  | SIP/2.0 100 Trying |  |  |

Table 8.32.3.3-11: NOTIFY (step 21, table 8.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | <?xml version="1.0" encoding="UTF-8"?> <conference-info xmlns="urn:ietf:params:xml:ns:conference-info"> entity="sip:final@conf-factory. appended with px_IMS_HomeDomainName" state="partial" version=" value as in previous notification for conference event package but incremented by one" <users> <user entity=" SIP URI or tel URI of the invited user"> <endpoint entity=" Contact URI of the invited user"> <status>connected</status> <joining-method>dialed-in</joining-method> <media id="1"> <type>audio</type> <label> unique identifier for the media stream between the focus and the endpoint of the invited user (e.g. 11223)</label> <src-id>random SSRC value</src-id> <status>sendrecv</status> </media> </endpoint> </users> </conference-info> |  |  |

Table 8.32.3.3-12: BYE (step 24, table 8.32.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.8, Condition A8 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Request-Line Request-URI |  | sip:final@conf-factory appended with px_IMS_HomeDomainName |  |  |

Table 8.32.3.3-10: NOTIFY for conference event package (step 17, table 8.32.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.5.3, Condition A4

<End of modified section 1>