3GPP TSG-RAN5 Meeting #90-e	R5-211450

# Electronic Meeting, 22nd Feb 2021 – 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1348 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates to RACH-ConfigCommon IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The RACH-ConfigCommon-DEFAULT IE in Table 4.6.3-12 contains editorial errors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RACH-ConfigCommon-DEFAULT IE in Table 4.6.3-12 have been updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RACH-ConfigCommon-DEFAULT will not be correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-211172. |  |  |  |  |  |  |  |  |

#### –	RACH-ConfigCommon-DEFAULT

Table 4.6.3-12: RACH-ConfigCommon-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |  |  |  |  |
| preambleInfo SEQUENCE { |  |  |  |  |  |  |  |
| numberOfRA-Preambles |  | n52 |  | Assuming the number of dedicated preambles is 12. |  |  |  |
| preamblesGroupAConfig SEQUENCE {} |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |  |  |  |  |
| powerRampingStep |  | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower |  | dBm-104 (default) |  | Thermal noise = -113 dBm NF = 5 dB IoT = 6 dB Required SNR = -8 dB (See table 8.4.2-1 in TS 36.104 [30]) -> -110 dB (default value is acceptable) |  |  |  |
| } |  |  |  |  |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |  |  |  |  |
| preambleTransMax |  | n6 |  | Under the condition of Case 1 in RAN1 simulation assumptions, an UE with pathloss of CDF = 90% reaches the maximum transmit power in 4 successive retransmissions. 6 has been selected considering the margin of 2. |  |  |  |
| ra-ResponseWindowSize |  | sf10 |  | The maximum value is preferable. |  |  |  |
| mac-ContentionResolutionTimer |  | sf48 |  | Allows for a sufficient number of msg3 retransmissions. |  |  |  |
| } |  |  |  |  |  |  |  |
| maxHARQ-Msg3Tx |  | 4 |  | Under the condition of Target SIR = 2 dB, the probability of 2 transmissions is less than 1%. 4 has been selected considering the margin of 2. |  |  |  |
| preambleTransMax-CE-r13 |  | Not present |  |  |  |  |  |
| preambleTransMax-CE-r13 |  | n6 |  | Same as preambleTransMax |  | CEmodeA or CEmodeB |  |
| RACH-CE-LevelInfoList-r13 |  | Not present |  |  |  |  |  |
| RACH-CE-LevelInfoList-r13 SEQUENCE (SIZE (1..maxCE-Level-r13)) OF RACH-CE-LevelInfo-r13 |  | 2 entries |  | first two 2 entries [1] and [2] of list to be used |  | CEmodeA |  |
| RACH-CE-LevelInfoList-r13 SEQUENCE (SIZE (1..maxCE-Level-r13)) OF RACH-CE-LevelInfo-r13 |  | 4 entries |  |  |  | CEmodeB |  |
| RACH-CE-LevelInfo-r13[1] SEQUENCE { |  |  |  |  |  |  |  |
| preambleMappingInfo-r13 SEQUENCE { |  |  |  |  |  |  |  |
| firstPreamble-r13 |  | 0 |  |  |  |  |  |
| lastPreamble-r13 |  | 14 |  |  |  |  |  |
|  |  | 13 |  |  |  | EDT |  |
| } |  |  |  |  |  |  |  |
| ra-ResponseWindowSize-r13 |  | sf20 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r13 |  | sf80 |  | Allows for a sufficient number of msg3 retransmissions. Longer time for increasing CE levels. |  |  |  |
| rar-HoppingConfig-r13 |  | off |  |  |  |  |  |
| edt-Parameters-r15 |  | Not present |  |  |  |  |  |
| edt-Parameters-r15 SEQUENCE { |  |  |  |  |  | EDT |  |
| edt-LastPreamble-r15 |  | 14 |  |  |  |  |  |
| edt-SmallTBS-Enabled-r15 |  | False |  |  |  |  |  |
| edt-TBS-r15 |  | b408 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| RACH-CE-LevelInfo-r13[2] SEQUENCE { |  |  |  |  |  |  |  |
| preambleMappingInfo-r13 SEQUENCE { |  |  |  |  |  |  |  |
| firstPreamble-r13 |  | 15 |  |  |  |  |  |
| lastPreamble-r13 |  | 29 |  |  |  |  |  |
|  |  | 28 |  |  |  | EDT |  |
| } |  |  |  |  |  |  |  |
| ra-ResponseWindowSize-r13 |  | sf80 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r13 |  | sf120 |  |  |  |  |  |
| rar-HoppingConfig-r13 |  | off |  |  |  |  |  |
| edt-Parameters-r15 |  | Not present |  |  |  |  |  |
| edt-Parameters-r15 SEQUENCE { |  |  |  |  |  | EDT |  |
| edt-LastPreamble-r15 |  | 29 |  |  |  |  |  |
| edt-SmallTBS-Enabled-r15 |  | False |  |  |  |  |  |
| edt-TBS-r15 |  | b408 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| RACH-CE-LevelInfo-r13[3] SEQUENCE { |  |  |  |  |  |  |  |
| PreambleMappingInfo-r13[3] SEQUENCE { |  |  |  |  |  |  |  |
| firstPreamble-r13 |  | 30 |  |  |  |  |  |
| lastPreamble-r13 |  | 44 |  |  |  |  |  |
|  |  | 43 |  |  |  | EDT |  |
| } |  |  |  |  |  |  |  |
| ra-ResponseWindowSize-r13 |  | sf180 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r13 |  | sf200 |  |  |  |  |  |
| rar-HoppingConfig-r13 |  | off |  |  |  |  |  |
| edt-Parameters-r15 |  | Not present |  |  |  |  |  |
| edt-Parameters-r15 SEQUENCE { |  |  |  |  |  | EDT |  |
| edt-LastPreamble-r15 |  | 44 |  |  |  |  |  |
| edt-SmallTBS-Enabled-r15 |  | False |  |  |  |  |  |
| edt-TBS-r15 |  | b408 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| RACH-CE-LevelInfo-r13[4] SEQUENCE { |  |  |  |  |  |  |  |
| preambleMappingInfo-r13 SEQUENCE { |  |  |  |  |  |  |  |
| firstPreamble-r13 |  | 45 |  |  |  |  |  |
| lastPreamble-r13 |  | 60 |  |  |  |  |  |
|  |  | 59 |  |  |  | EDT |  |
| } |  |  |  |  |  |  |  |
| ra-ResponseWindowSize-r13 |  | sf320 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r13 |  | sf480 |  |  |  |  |  |
| rar-HoppingConfig-r13 |  | off |  |  |  |  |  |
| edt-Parameters-r15 |  | Not present |  |  |  |  |  |
| edt-Parameters-r15 SEQUENCE { |  |  |  |  |  | EDT |  |
| edt-LastPreamble-r15 |  | 60 |  |  |  |  |  |
| edt-SmallTBS-Enabled-r15 |  | False |  |  |  |  |  |
| edt-TBS-r15 |  | b408 |  |  |  |  |  |
| mac-ContentionResolutionTimer-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| edt-SmallTBS-Subset-r15 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| EDT | For Early Data Transmission testing |

#### –	Rach-ConfigDedicated-DEFAULT

Table 4.6.3-12A: Rach-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Rach-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| ra-PreambleIndex | 52 |  |  |
| ra-PRACH-MaskIndex | 0 |  |  |
| } |  |  |  |