3GPP TSG-RAN WG5 Meeting #90-e	R5-211465

# Electronic meeting, 22nd Feb - 05th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1693 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SIB1 and SIB10 for Rel-16 NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To update the default contents of SIB1 and SIB10 for Rel-16 NPN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the default contents of SIB1 and SIB10 for Rel-16 NPN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2 and 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-210371->R5-210371r1: Updated the SIB10 and RRC IE tables R5-210371r1->R5-210372r2: - Further updates to Table 4.6.3-84AB: NPN-IdentityInfoList - Defined conditions CAG, SNPN for Table 4.6.3-84AB: NPN-IdentityInfoList. R5-211465: This is the final version of R5-210371r2 |  |  |  |  |  |  |  |  |

### 4.6.2	System information blocks

<start of modified section>

#### –	SIB10

Table 4.6.2-12: SIB10

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB10-r16 ::= SEQUENCE { |  |  |  |
| hrnn-List-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF HRNN-r16 { |  |  |  |
| HRNN-r16[1] SEQUENCE { |  | 1 entry |  |
| hrnn-r16 | “3gppTest” | charstring “3gppTest” converted to OCTETSTRING |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

<end of modified section>

### 4.6.3	Radio resource control information elements

<start of modified section>

#### –	CellAccessRelatedInfo

Table 4.6.3-16: CellAccessRelatedInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo ::= SEQUENCE { |  |  |  |
| plmn-IdentityList | PLMN-IdentityInfoList |  |  |
| cellReservedForOtherUse | Not present |  |  |
| npn-IdentityInfoList-r16 | NPN-IdentityInfoList-r16 |  | SNPN |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SNPN | Standalone NPN cell |

#### –	NPN-Identity

Table 4.6.3-84AA: NPN-Identity

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPN-Identity-r16 ::= CHOICE { |  |  |  |
| pni-npn-r16 SEQUENCE { |  |  | CAG |
| PLMN-Identity[1] | PLMN-Identity |  |  |
| cag-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF CAG-IdentityInfo-r16 { | 1 entry |  |  |
| cag-Identity-r16[1] |  |  |  |
| manualCAGselectionAllowed-r16 | true |  |  |
| } |  |  |  |
| snpn-r16 SEQUENCE { |  |  | SNPN |
| plmn-Identity-r16 | PLMN-Identity |  |  |
| nid-List-r16[1] | FFS | NID is coded as a BITSTRING based on TS 38.508-1 Table [FFS]. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CAG | PNI-NPN |
| SNPN | Standalone NPN cell |

#### –	NPN-IdentityInfoList

Table 4.6.3-84AB: NPN-IdentityInfoList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPN-IdentityInfoList-r16 ::= SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-IdentityInfo-r16 { | 1 entry |  |  |
| NPN-IdentityInfo-r16[1] SEQUENCE { |  |  |  |
| npn-IdentityList-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF NPN-Identity-r16 { | 1 entry |  |  |
| NPN-Identity-r16[1] | NPN-Identity with condition CAG |  | CAG |
|  | NPN-Identity with condition SNPN |  | SNPN |
| } |  |  |  |
| trackingAreaCode-r16 | TrackingAreaCode |  |  |
| ranac-r16 | RAN-AreaCode |  |  |
| cellIdentity-r16 | CellIdentity |  |  |
| cellReservedForOperatorUse-r16 | notReserved |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CAG | PNI-NPN |
| SNPN | Standalone NPN cell |

<end of modified section>