3GPP TSG-RAN5 Meeting #90-e	R5-211500

# Online,  22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 1916 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 8.1.5.1.1 to add UE capability nr-HO-ToEN-DC-r16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As CR R5-206429 raised at RAN5#89-e was not implementable, a new CR need to be submitted to implement the changes. - In order to support Inter-RAT handover from NR to EN-DC feature in R16,a new UE capability nr-HO-ToEN-DC-r16 should be added in the InterRAT-Parameters IE - The “i entries” in supportedBandListEUTRA is meaningless as the IE is “not checked”, so “i entries” is assumed to be a typo and should be deleted. - change some mandatory IEs from ‘not checked’ to ‘supported’ |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the test case 8.1.5.1.1 to add the UE capability nr-HO-ToEN-DC-r16 in UE-NR-Capability Table 8.1.5.1.1.3.3-3 - Delete “i entries” from the value of supportedBandListEUTRA. - change some mandatory IEs from ‘not checked’ to ‘supported’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE capability nr-HO-ToEN-DC-r16 will not be supported and the wrong value of IE supportedBandListEUTRA will cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-210028 |  |  |  |  |  |  |  |  |

-------- Start of changes --------

8.1.5.1.1	UE capability transfer / Success

8.1.5.1.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

8.1.5.1.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clauses 5.6.1.3 and 5.6.1.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.6.1.3]

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

…

1>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.6.1.4]

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR and eutra-nr-only flag) as defined in TS 36.331, where applicable.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

8.1.5.1.1.3	Test Description

8.1.5.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A on NR Cell 1.

8.1.5.1.1.3.2	Test procedure sequence

Table 8.1.5.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including RAT-Type nr. | <-- | UECapabilityEnquiry | - | - |
| 2 | Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? | --> | UECapabilityInformation | 1 | P |

8.1.5.1.1.3.3	Specific message contents

Table 8.1.5.1.1.3.3-1: UECapabilityEnquiry (step 1, Table 8.1.5.1.1.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |
| ue-CapabilityRAT-RequestList SEQUENCE (SIZE (1.. maxRAT-CapabilityContainers)) OF CapabilityRAT-Request { | - |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| criticalExtensionsFuture |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-2: UECapabilityInformation (step 2, Table 8.1.5.1.1.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF CapabilityRAT-Request { |  |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| ueCapabilityRAT-Container | UE-NR-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| criticalExtensionsFuture | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-3: UE-NR-Capability (Table 8.1.5.1.1.3.3-2)

| Derivation Path: 38.331 [12], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |
| accessStratumRelease | Checked | "rel-15" or higher |  |
| pdcp-Parameters SEQUENCE { |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |
| profile0x0000 | Not checked |  |  |
| profile0x0001 | Not checked |  |  |
| profile0x0002 | Not checked |  |  |
| profile0x0003 | Not checked |  |  |
| profile0x0004 | Not checked |  |  |
| profile0x0006 | Not checked |  |  |
| profile0x0101 | Not checked |  |  |
| profile0x0102 | Not checked |  |  |
| profile0x0103 | Not checked |  |  |
| profile0x0104 | Not checked |  |  |
| } |  |  |  |
| maxNumberROHC-ContextSessions | Not checked |  |  |
| uplinkOnlyROHC-Profiles | Not checked |  |  |
| continueROHC-Context | Not checked |  |  |
| outOfOrderDelivery | Checked |  | pc_outOfOrderDelivery |
| shortSN | Checked |  | pc_shortSN |
| pdcp-DuplicationSRB | Not checked |  |  |
| pdcp-DuplicationMCG-OrSCG-DRB | Not checked |  |  |
| } |  |  |  |
| rlc-Parameters SEQUENCE { |  |  |  |
| am-WithShortSN | Checked |  | pc_am_WithShortSN |
| um-WithShortSN | Checked |  | pc_um_WithShortSN |
| um-WithLongSN | Checked |  | pc_um_WIthLongSN |
| } |  |  |  |
| mac-Parameters SEQUENCE { |  |  |  |
| mac-ParametersCommon SEQUENCE { |  |  |  |
| lcp-Restriction | Not checked |  |  |
| dummy | Not checked |  |  |
| lch-ToSCellRestriction | Not checked |  |  |
| recommendedBitRate | Not checked |  |  |
| recommendedBitRateQuery | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| phy-Parameters SEQUENCE { |  |  |  |
| phy-ParametersCommon SEQUENCE { |  |  |  |
| csi-RS-CFRA-ForHO | Checked |  | pc_csi_RS_CFRA_ForHO |
| dynamicPRB-BundlingDL | Not checked |  |  |
| sp-CSI-ReportPUCCH | Not checked |  |  |
| sp-CSI-ReportPUSCH | Not checked |  |  |
| nzp-CSI-RS-IntefMgmt | Not checked |  |  |
| type2-SP-CSI-Feedback-LongPUCCH | Not checked |  |  |
| precoderGranularityCORESET | Not checked |  |  |
| dynamicHARQ-ACK-Codebook | supported |  |  |
| semiStaticHARQ-ACK-Codebook | Not checked |  |  |
| spatialBundlingHARQ-ACK | Not checked |  |  |
| dynamicBetaOffsetInd-HARQ-ACK-CSI | Not checked |  |  |
| pucch-Repetition-F1-3-4 | Not checked |  |  |
| ra-Type0-PUSCH | Checked |  | pc_ra_Type0_PUSCH |
| dynamicSwitchRA-Type0-1-PDSCH | Not checked |  |  |
| dynamicSwitchRA-Type0-1-PUSCH | Not checked |  |  |
| pdsch-MappingTypeA | supported |  | pc_pdsch_MappingTypeA |
| pdsch-MappingTypeB | Checked |  | pc_pdsch_MappingTypeB |
| interleavingVRB-ToPRB-PDSCH | Checked |  | pc_interleavingVRB_ToPRB_PDSCH |
| interSlotFreqHopping-PUSCH | Not checked |  |  |
| type1-PUSCH-RepetitionMultiSlots | Not checked |  |  |
| type2-PUSCH-RepetitionMultiSlots | Not checked |  |  |
| pusch-RepetitionMultiSlots | Checked |  | pc_pusch_RepetitionMultiSlots |
| pdsch-RepetitionMultiSlots | Checked |  | pc_pdsch_RepetitionMultiSlots |
| downlinkSPS | Checked |  | pc_downlinkSPS |
| configuredUL-GrantType1 | Checked |  | pc_configuredUL_GrantType1 |
| configuredUL-GrantType2 | Checked |  | pc_configuredUL_GrantType2 |
| pre-EmptIndication-DL | Not checked |  |  |
| cbg-TransIndication-DL | Not checked |  |  |
| cbg-TransIndication-UL | Not checked |  |  |
| cbg-FlushIndication-DL | Not checked |  |  |
| dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Not checked |  |  |
| rateMatchingResrcSetSemi-Static | Not checked |  |  |
| rateMatchingResrcSetDynamic | Not checked |  |  |
| bwp-SwitchingDelay | Not checked |  |  |
| dummy | Not checked |  |  |
| maxNumberSearchSpaces | n10 |  |  |
| rateMatchingCtrlResrsSetDynamic | Not checked |  |  |
| maxLayersMIMO-Indication | Not checked |  |  |
| } |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | supported |  |  |
| pucch-F3-WithFH | supportedd |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked | CSI-RS-IM-ReceptionForFeedback |  |
| csi-RS-ProcFrameworkForSRS | Not checked | CSI-RS-ProcFrameworkForSRS |  |
| csi-ReportFramework | Not checked | CSI-ReportFramework |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFR1 SEQUENCE { |  |  |  |
| pdcchMonitoringSingleOccasion | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| pdsch-256QAM-FR1 | Checked |  | pc_pdsch_256QAM_FR1 |
| pdsch-RE-MappingFR1-PerSymbol | Not checked |  |  |
| pdsch-RE-MappingFR1-PerSlot | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFR2 SEQUENCE { |  |  |  |
| dummy | Not checked |  |  |
| pdsch-RE-MappingFR2-PerSymbol | Not checked |  |  |
| pCell-FR2 | Not checked |  |  |
| pdsch-RE-MappingFR2-PerSlot | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rf-Parameters SEQUENCE { |  |  |  |
| supportedBandListNR SEQUENCE (SIZE (1..maxBands)) OF BandNR { |  |  |  |
| BandNR SEQUENCE { |  |  |  |
| bandNR | Checked | Checked for 'maxBands' entries of FreqBandIndicatorNR[i] | pc_nrBandx ('x' being the band number/type related PICS listed in TS 38.508-2) |
| modifiedMPR-Behaviour | Not checked |  |  |
| mimo-ParametersPerBand | Not checked | MIMO-ParametersPerBand[i] |  |
| extendedCP | Not checked |  |  |
| multipleTCI | supported |  |  |
| bwp-WithoutRestriction | Not checked |  |  |
| bwp-SameNumerology | Not checked |  |  |
| bwp-DiffNumerology | Not checked |  |  |
| crossCarrierScheduling-SameSCS | Not checked |  |  |
| pdsch-256QAM-FR2 | Checked |  | pc_pdsch_256QAM_FR2 |
| pusch-256QAM | Checked |  | pc_pusch_256QAM_FR1 |
| ue-PowerClass | Not checked |  |  |
| rateMatchingLTE-CRS | Not checked |  |  |
| channelBWs-DL-v1530 CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| channelBWs-UL-v1530[i] CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| maxUplinkDutyCycle-PC2-FR1 | Not checked |  |  |
| pucch-SpatialRelInfoMAC-CE | Not checked |  |  |
| powerBoosting-pi2BPSK | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| supportedBandCombinationList SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination { |  |  |  |
| BandCombinationSEQUENCE { |  |  |  |
| bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters { |  |  |  |
| BandParameters CHOICE |  |  |  |
| eutra SEQUENCE { |  |  |  |
| bandEUTRA | Not checked | FreqBandIndicatorEUTRA |  |
| ca-BandwidthClassDL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| ca-BandwidthClassUL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| } |  |  |  |
| nr SEQUENCE { |  |  |  |
| bandNR | Not checked | FreqBandIndicatorNR |  |
| ca-BandwidthClassDL-NR | Not checked | CA-BandwidthClassNR |  |
| ca-BandwidthClassUL-NR | Not checked | CA-BandwidthClassNR |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombination | Not checked | FeatureSetCombinationId |  |
| ca-ParametersEUTRA | Not checked | CA-ParametersEUTRA |  |
| ca-ParametersNR | Not checked | CA-ParametersNR |  |
| mrdc-Parameters | Not checked | MRDC-Parameters |  |
| supportedBandwidthCombinationSet | Not checked |  |  |
| powerClass-v1530 | Not checked |  |  |
| } |  |  |  |
| appliedFreqBandListFilter | Not checked | FreqBandList |  |
| supportedBandCombinationList-v1540 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540 { |  |  |  |
| BandCombination-v1540 SEQUENCE { |  |  |  |
| bandList-v1540 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540 { | Not checked | BandParameters-v1540 |  |
| BandParameters-v1540 SEQUENCE { |  |  |  |
| srs-CarrierSwitch CHOICE { |  |  |  |
| nr SEQUENCE [ |  | NR bands |  |
| srs-SwitchingTimesListNR SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR | Not checked |  |  |
| } |  |  |  |
| eutra SEQUENCE { |  | EUTRA bands |  |
| srs-SwitchingTimesListEUTRA SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-TxSwitch-v1540 SEQUENCE { |  |  |  |
| supportedSRS-TxPortSwitch | Not checked |  |  |
| txSwitchImpactToRx | Not checked |  |  |
| txSwitchWithAnotherBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ca-ParametersNR-v1540[i] | Not checked | CA-ParametersNR-v1540 |  |
| } |  |  |  |
| } |  |  |  |
| srs-SwitchingTimeRequested | Not checked |  |  |
| } |  |  |  |
| measAndMobParameters SEQUENCE { |  |  |  |
| measAndMobParametersCommon SEQUENCE { |  |  |  |
| supportedGapPattern | Not checked |  |  |
| ssb-RLM | supported |  |  |
| ssb-AndCSI-RS-RLM | Not checked |  |  |
| eventB-MeasAndReport | Not checked |  |  |
| handoverFDD-TDD | Not checked |  |  |
| eutra-CGI-Reporting | Not checked |  |  |
| nr-CGI-Reporting | Not checked |  |  |
| independentGapConfig | Checked |  | pc_independentGapConfig |
| periodicEUTRA-MeasAndReport | Not checked |  |  |
| handoverFR1-FR2 | Not checked |  |  |
| maxNumberCSI-RS-RRM-RS-SINR | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF-MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Not checked |  |  |
| handoverLTE | Not checked |  |  |
| handover-eLTE | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Not checked |  |  |
| handoverLTE | Not checked |  |  |
| handover-eLTE | Not checked |  |  |
| maxNumberResource-CSI-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF-MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Not checked |  |  |
| handoverLTE | Not checked |  |  |
| handover-eLTE | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF-MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Not checked |  |  |
| handoverLTE | Not checked |  |  |
| handover-eLTE | Not checked |  |  |
| } |  |  |  |
| fr1-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked |  |  |
| csi-RS-ProcFrameworkForSRS | Not checked |  |  |
| csi-ReportFramework | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Not checked |  |  |
| handoverLTE | Not checked |  |  |
| handover-eLTE | Not checked |  |  |
| maxNumberResource-CSI-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { | Not checked |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked |  |  |
| csi-RS-ProcFrameworkForSRS | Not checked |  |  |
| csi-ReportFramework | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Not checked |  |  |
| handoverLTE | Not checked |  |  |
| handover-eLTE | Not checked |  |  |
| maxNumberResource-CSI-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSets SEQUENCE { |  |  |  |
| featureSetsDownlink SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink { |  |  |  |
| FeatureSetDownlink SEQUENCE { | Not checked |  |  |
| featureSetListPerDownlinkCC | Not checked |  |  |
| intraBandFreqSeparationDL | Not checked |  |  |
| scalingFactor | Not checked |  |  |
| crossCarrierScheduling-OtherSCS | Not checked |  |  |
| scellWithoutSSB | Not checked |  |  |
| csi-RS-MeasSCellWithoutSSB | Not checked |  |  |
| dummy1 | Not checked |  |  |
| type1-3-CSS | Not checked |  |  |
| pdcch-MonitoringAnyOccasions | Not checked |  |  |
| dummy2 | Not checked |  |  |
| ue-SpecificUL-DL-Assignment | Not checked |  |  |
| searchSpaceSharingCA-DL | Not checked |  |  |
| timeDurationForQCL SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| dummy3 | Not checked |  |  |
| dummy4 | Not checked |  |  |
| dummy5 | Not checked |  |  |
| dummy6 | Not checked |  |  |
| dummy7 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsDownlinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC { |  |  |  |
| FeatureSetDownlinkPerCCSEQUENCE { |  |  |  |
| supportedSubcarreirSpacingDL | Not checked | SubcarrierSpacing |  |
| supportedBandwidthDL | Not checked | SupportedBandwidth |  |
| channelBW-90mhz | Not checked |  |  |
| maxNumberMIMO-LayersPDSCH | Not checked | MIMO-LayersDL |  |
| supportedModulationOrderDL | Not checked | ModulationOrder |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsUplink SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink { |  |  |  |
| FeatureSetUplink SEQUENCE { |  |  |  |
| featureSetListPerUplinkCC | Not checked |  |  |
| scalingFactor | Not checked |  |  |
| crossCarrierScheduling-OtherSCS | Not checked |  |  |
| intraBandFreqSeparationUL | Not checked |  |  |
| searchSpaceSharingCA-UL | Not checked |  |  |
| dummy1 | Not checked |  |  |
| supportedSRS-Resources | Not checked | SRS-Resources |  |
| twoPUCCH-Group | Not checked |  |  |
| dynamicSwitchSUL | Not checked |  |  |
| simultaneousTxSUL-NonSUL | Not checked |  |  |
| pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| dummy2 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsUplinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC { |  |  |  |
| FeatureSetUplinkPerCCSEQUENCE { |  |  |  |
| supportedSubcarrierSpacingUL | Not checked | SubcarrierSpacing |  |
| supportedBandwidthUL | Not checked | SupportedBandwidth |  |
| channelBW-90mHz | Not checked |  |  |
| mimo-CB-PUSCH SEQUENCE { |  |  |  |
| maxNumberMIMO-LayersCB-PUSCH | Checked | MIMO-LayersUL | pc_nrMIMO_CB_PUSCH |
| maxNumberSRS-ResourcePerSet | Not checked |  |  |
| } |  |  |  |
| maxNumberMIMO-LayersNonCB-PUSCH | Checked | MIMO-LayersUL | pc_nrMIMO_NonCB_PUSCH |
| supportedModulationOrderUL | Not checked | ModulationOrder |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsDownlink-v1540 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1540 { |  |  |  |
| FeatureSetDownlink-v1540 SEQUENCE { |  |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-DL | Not checked |  |  |
| additionalDMRS-DL-Alt | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-DL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-DL | Not checked |  |  |
| pdcch-MonitoringAnyOccasionsWithSpanGap SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| pdsch-SeparationWithGap | Not checked |  |  |
| pdsch-ProcessingType2 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| pdsch-ProcessingType2-Limited SEQUENCE { |  |  |  |
| differentTB-PerSlot-SCS-30kHz | Not checked |  |  |
| } |  |  |  |
| dl-MCS-TableAlt-DynamicIndication | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsUplink-v1540 SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1540 { |  |  |  |
| FeatureSetUplink-v1540 SEQUENCE { |  |  |  |
| zeroSlotOffsetAperiodicSRS | Not checked |  |  |
| pa-PhaseDiscontinuityImpacts | Not checked |  |  |
| pusch-SeparationWithGap | Not checked |  |  |
| pusch-ProcessingType2 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| ul-MCS-TableAlt-DynamicIndication | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsUplinkPerCC-v1540 SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1540 { |  |  |  |
| FeatureSetUplinkPerCC-v1540 SEQUENCE { |  |  |  |
| mimo-NonCB-PUSCH SEQUENCE { |  |  |  |
| maxNumberSRS-ResourcePerSet | Not checked |  |  |
| maxNumberSimultaneousSRS-ResourceTx | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombinations SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination { |  |  |  |
| FeatureSetCombination SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand { |  |  |  |
| FeatureSetsPerBand SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet { |  |  |  |
| FeatureSetCHOICE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| downlinkSetEUTRA | Not checked | FeatureSetEUTRA-DownlinkId |  |
| uplinkSetEUTRA | Not checked | FeatureSetEUTRA-UplinkId |  |
| } |  |  |  |
| nr SEQUENCE { |  |  |  |
| downlinkSetNR | Not checked | FeatureSetDownlinkId |  |
| uplinkSetNR | Not checked | FeatureSetUpinkId |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| fdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| dummy | Not checked |  |  |
| interRAT-Parameters SEQUENCE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| supportedBandListEUTRA SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA | Not checked |  |  |
| eutra-ParametersCommon SEQUENCE { |  |  |  |
| mfbi-EUTRA | Not checked |  |  |
| modifiedMRP-BehaviorEUTRA | Not checked |  |  |
| multiNS-Pmax-EUTRA | Not checked |  |  |
| rs-SINR-MeasEUTRA | Not checked |  |  |
| ne-DC | Not checked |  |  |
| nr-HO-ToEN-DC-r16 | Checked |  | pc_interRAT_NR_ToENDC |
| } |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| inactiveState | Not checked |  |  |
| delayBudgetReporting | Not checked |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |
| sdap-Parameters SEQUENCE { |  |  |  |
| as-ReflectiveQoS | Checked |  | pc_as_ReflectiveQoS |
| } |  |  |  |
| overheatingInd | Not checked |  |  |
| ims-Parameters SEQUENCE { |  |  |  |
| ims-ParametersCommon SEQUENCE { |  |  |  |
| voiceOverEUTRA-5GC | Not checked |  |  |
| } |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked | CSI-RS-IM-ReceptionForFeedback |  |
| csi-RS-ProcFrameworkForSRS | Not checked | CSI-RS-ProcFrameworkForSRS |  |
| csi-ReportFramework | Not checked | CSI-ReportFramework |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Not checked |  |  |
| handoverLTE | Not checked |  |  |
| handover-eLTE | Not checked |  |  |
| maxNumberResource-CSI-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and intraAndInterF-MeasAndReport is supported in both modes, then support of intraAndInterF-MeasAndReport will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and intraAndInterF-MeasAndReport is only supported in one mode, then support of intraAndInterF-MeasAndReport will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate.) Note 2: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and eventA-MeasAndReport is supported in both modes, then support of eventA-MeasAndReport will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and eventA-MeasAndReport is only supported in one mode, then support of eventA-MeasAndReport will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 3: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signaled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 4: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and skipUplinkTxDynamic is supported in both modes, then support of skipUplinkTxDynamic will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and skipUplinkTxDynamic is only supported in one mode, then skipUplinkTxDynamic will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 5: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and logicalChannelSR-DelayTimer is supported in both modes, then support of logicalChannelSR-DelayTimer will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and logicalChannelSR-DelayTimer is only supported in one mode, then support of logicalChannelSR-DelayTimer will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 6: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and pc_longDRX_Cycle is supported in both modes, then support of pc_longDRX_Cycle will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and pc_longDRX_Cycle is only supported in one mode, then support of pc_longDRX_Cycle will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 7: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and pc_shortDRX_Cycle is supported in both modes, then support of pc_shortDRX_Cycle will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and pc_shortDRX_Cycle is only supported in one mode, then support of pc_shortDRX_Cycle will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. |  |  |  |