3GPP TSG-RAN5 Meeting #90-e	R5-211516

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0203 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for support of multiple GPS signals |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2021-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the case where a UE supports multiple GPS signals, previous CRs have missed some text that needs modification. 1. In Table 6.1.3.1-1, the note indicates “multiple signals per GNSS” – this is incorrect as it only applies to GPS and not other GNSSs. Note that this text is correct in other places. 2. The GANSS auxiliary information applies for more sub-tests than just 1, 3, and 4 in the case where the UE supports multiple GPS signals. 3. In Table 6.1.3.3-1, the note indicates “multiple signals per GNSS” – this is incorrect as it only applies to GPS and not other GNSSs. Note that this text is correct in other places. 4. LPP TS 36.355 is now TS 37.355. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. In Table 6.1.3.1-1, the note is corrected and clarified and merged into the main table. 2. The GANSS auxiliary information is made more general so that it can be applied to all applicable sub-tests. 3. In Table 6.1.3.3-1, the note is corrected. 4. LPP TS 36.355 is updated to 37.355. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the case where a UE supports multiple GPS signals, errors will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.3.1, 6.1.3.2, 6.1.3.3, 6.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Similar CR for RF |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | One revision of R5-210051 |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[3]	Void

[4]	Void

[5]	Void

[6]	3GPP TS 37.571-1: “User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance”.

[7]	3GPP TS 37.571-2: “User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance”.

[8]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[9]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[10]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[11]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[12]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[13]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[14]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[15]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[16]	3GPP TS 25.331: “Radio Resource Control (RRC); Protocol specification”

[17]	STANAG 4294: NATO STANAG 4294. Navstar Global Positioning System (GPS) System Characteristics.

[18]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[19]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[20]	3GPP TS 36.508: "Common test environments for User Equipment (UE) conformance testing".

[21]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[22]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[23]	Bluetooth Special Interest Group (SIG): “Bluetooth Core Specification v4.2”, December 2014.

[24]	3GPP TS 38.508-1: "User Equipment (UE) conformance specification; Part 1: Common test environment".

## Next changed section

### 6.1.3	Assistance Data

This subclause defines the GNSS scenarios and assistance data IEs which shall be available for use where required in A-GNSS signalling test cases defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9.

#### 6.1.3.1	Default Assistance Data for TS 37.571-2 subclauses 6.2.1 to 6.2.3

The assistance data listed in subclause 6.1.3.1 are the assistance data elements pushed by the SS in some tests defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3. During the test the UE may request additional assistance data as specified in the tests and the SS shall then provide any other assistance data available as defined in subclause 6.1.3.

Table 6.1.3.1-1: GNSS assistance data to be provided to the UE

| GNSS Assistance Data IE to be provided to the UE | Mode used in test case |  |
| --- | --- | --- |
|  | UE-based | UE-assisted |
| GPS reference time | Yes for sub-tests 3, 4, 8, 10 | Yes for sub-tests 3, 4, 8, 10 |
| GPS reference UE position | Yes for sub-tests 3, 4, 8, 10 | No |
| GPS navigation model | Yes for sub-tests 3, 4, 8, 10 | No |
| GPS ionospheric model | Yes for sub-tests 3, 4, 8, 10 | No |
| GPS UTC model | Yes for sub-test 4, 8 | Yes for sub-test 4, 8 |
| GPS acquisition assistance | No | Yes for sub-tests 3, 4, 8, 10 |
| GANSS reference time | Yes for sub-tests 1, 2, 9 | Yes for sub-tests 1, 2, 9 |
| GANSS reference UE position | Yes for sub-tests 1, 2, 9 | No |
| GANSS ionospheric model | Yes for sub-test 2, 8 | No |
| GANSS additional ionospheric model | Yes for sub-test 1, 9, 10 | No |
| GANSS Time Models | Yes for sub-test 4, 8, 10 | No |
| GANSS navigation model | Yes for sub-test 2, 8 | No |
| GANSS additional navigation models | Yes for sub-tests 1, 4, 9, 10 | No |
| GANSS reference measurement information | No | Yes for sub-tests 1, 2, 4, 8, 9, 10 |
| GANSS auxiliary information | Yes for sub-tests 1, 4 (for GLONASS). Yes for sub-test 3 (for multiple GPS signals). Yes for sub-tests 4, 8, 10 if the UE supports multiple GPS signals | Yes for sub-tests 1, 4 (for GLONASS). Yes for sub-test 3 (for multiple GPS signals). Yes for sub-tests 4, 8, 10 if the UE supports multiple GPS signals |

#### 6.1.3.2	Assistance Data values for TS 37.571-2 subclauses 6.2.1 to 6.2.3

Where assistance data is required on a per-satellite basis, or where the values of the data also varies with time it is specified in comma-separated-variable files in the GNSS data sig zip file specified in Annex B. These files specify the values to be used for each satellite, indexed by satellite PRN or SV ID, and, where applicable, the values to be used indexed by both time and satellite PRN or SV ID.

Assistance data that is marked as “time varying” and the GPS TOW msec or GANSS TOD field are only specified and used in 1 second increments. Interpolation between these values shall not be used.

The accuracy of the GPS TOW msec or GANSS TOD and assistance data that is marked as “time varying” in the provided assistance data shall be within +/- 2 s relative to the GNSS time in the system simulator. In the case that assistance data is required but satellite signals are not required then this clause does not apply.

Assistance data Information Elements and fields that are not specified shall not be used.

The information elements detailed below are fully defined in TS 25.331 [16]

Assistance Data GPS Reference Time

# GPS Reference Time (Fields occurring once per message)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GPS Week | weeks | 1669 |  |
| GPS Week Cycle Number |  | 1 | Rel-10 onwards |
| GPS TOW msec | msec | 1860000 ms. Start time. Add integer number of 1 seconds as required. (Note) |  |
| UE Positioning GPS Reference Time Uncertainty |  | 125 (2.127 seconds) |  |
| Note: GPS TOW msec This is the value of GPS TOW msec when the GNSS scenario is started in the GNSS simulator. The value of GPS TOW msec to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current GPS TOW msec” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.2. In the case that the (hardware) GPS simulator is switched off or not present then the value of GPS TOW msec given above may be used. |  |  |  |

# Assistance Data GPS Reference UE Position

# GPS Reference UE Position

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Latitude sign |  | 0 |
| Degrees Of Latitude | degrees | 35.744287 |
| Degrees Of Longitude | degrees | 139.680176 |
| Altitude Direction |  | 0 |
| Altitude | m | 300 |
| Uncertainty semi-major | m | 3000 |
| Uncertainty semi-minor | m | 3000 |
| Orientation of major axis | degrees | 0 |
| Uncertainty Altitude | m | 500 |
| Confidence | % | 68 |

# Assistance Data GPS Navigation Model

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GPS Navigation Model (Fields occurring once per satellite)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SatID | - | PRNs: 1, 11, 17, 20, 23, 28 |
| Satellite Status |  | 0 (Note) |
| Note: For consistency Satellite Status is also given in file: Sig GPS Navigation model.csv |  |  |

# GPS Ephemeris and Clock correction Information Elements (Fields occurring once per satellite)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| C/A or P on L2 |  | See file: Sig GPS Navigation model.csv |
| URA Index |  | See file: Sig GPS Navigation model.csv |
| SV Health |  | See file: Sig GPS Navigation model.csv |
| IODC | - | See file: Sig GPS Navigation model.csv |
| L2 P Data Flag |  | See file: Sig GPS Navigation model.csv |
| SF 1 Reserved | - | See file: Sig GPS Navigation model.csv |
| SF 2 Reserved | - | See file: Sig GPS Navigation model.csv |
| SF 3 Reserved | - | See file: Sig GPS Navigation model.csv |
| SF 4 Reserved | - | See file: Sig GPS Navigation model.csv |
| TGD | sec | See file: Sig GPS Navigation model.csv |
| toc | sec | See file: Sig GPS Navigation model.csv |
| af2 | sec/sec2 | See file: Sig GPS Navigation model.csv |
| af1 | sec/sec | See file: Sig GPS Navigation model.csv |
| af0 | sec | See file: Sig GPS Navigation model.csv |
| Crs | meters | See file: Sig GPS Navigation model.csv |
| n | semi-circles/sec | See file: Sig GPS Navigation model.csv |
| M0 | semi-circles | See file: Sig GPS Navigation model.csv |
| Cuc | radians | See file: Sig GPS Navigation model.csv |
| e | - | See file: Sig GPS Navigation model.csv |
| Cus | radians | See file: Sig GPS Navigation model.csv |
| (A)1/2 | meters1/2 | See file: Sig GPS Navigation model.csv |
| toe | sec | See file: Sig GPS Navigation model.csv |
| Fit Interval Flag |  | See file: Sig GPS Navigation model.csv |
| AODO | sec | See file: Sig GPS Navigation model.csv |
| Cic | radians | See file: Sig GPS Navigation model.csv |
| OMEGA0 | semi-circles | See file: Sig GPS Navigation model.csv |
| Cis | radians | See file: Sig GPS Navigation model.csv |
| i0 | semi-circles | See file: Sig GPS Navigation model.csv |
| Crc | meters | See file: Sig GPS Navigation model.csv |
|  | semi-circles | See file: Sig GPS Navigation model.csv |
| OMEGAdot | semi-circles/sec | See file: Sig GPS Navigation model.csv |
| Idot | semi-circles/sec | See file: Sig GPS Navigation model.csv |

# Assistance Data GPS Ionospheric Model

# GPS Ionospheric Model

| Information Element | Units | Value/remark |
| --- | --- | --- |
| 0 | seconds | 4.6566129 10E-9 |
| 1 | sec/semi-circle | 1.4901161 10E-8 |
| 2 | sec/(semi-circle)2 | -5.96046 10E-8 |
| 3 | sec/(semi-circle)3 | -5.96046 10E-8 |
| 0 | seconds | 79872 |
| 1 | sec/semi-circle | 65536 |
| 2 | sec/(semi-circle)2 | -65536 |
| 3 | sec/(semi-circle)3 | -393216 |

# Assistance Data GPS UTC model

# GPS UTC Model

| Information Element | Units | Value/remark |
| --- | --- | --- |
| A1 | sec/sec | 0 |
| A0 | seconds | 0 |
| tot | seconds | 249856 |
| WNt | weeks | 133 |
| tLS | seconds | 15 |
| WNLSF | weeks | 158 |
| DN | days | 7 |
| tLSF | seconds | 16 |

# Assistance Data GPS Almanac

# GPS Almanac (Fields occurring once per message)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| WNa | weeks | 1669 |  |
| Complete Almanac Provided |  | TRUE | Rel-10 onwards |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 31 |

# GPS Almanac (Fields occurring once per satellite)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| DataID | - | See file: Sig GPS Almanac.csv |
| SatID | - | PRNs: 1, 2, 3…21, 22, 23, 25, 26, 27…30, 31, 32 |
| e | dimensionless | See file: Sig GPS Almanac.csv |
| toa | sec | See file: Sig GPS Almanac.csv |
| i | semi-circles | See file: Sig GPS Almanac.csv |
| OMEGADOT | semi-circles/sec | See file: Sig GPS Almanac.csv |
| SV Health |  | See file: Sig GPS Almanac.csv |
| A1/2 | meters1/2 | See file: Sig GPS Almanac.csv |
| OMEGA0 | semi-circles | See file: Sig GPS Almanac.csv |
| M0 | semi-circles | See file: Sig GPS Almanac.csv |
|  | semi-circles | See file: Sig GPS Almanac.csv |
| af0 | seconds | See file: Sig GPS Almanac.csv |
| af1 | sec/sec | See file: Sig GPS Almanac.csv |

# Assistance Data GPS Acquisition Assistance

# GPS Acquisition Assist - Information Elements appearing once per message

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GPS TOW msec | msec | 1860000 ms. Start time. Add integer number of 1 seconds as required. (Note) |
| UE Positioning GPS Reference Time Uncertainty |  | 125 (2.127 seconds) |
| Note: GPS TOW msec This is the value of GPS TOW msec when the GNSS scenario is started in the GNSS simulator. The value of GPS TOW msec to be used in the Acquisition Assistance IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. In the case that the (hardware) GPS simulator is switched off or not present then the value of GPS TOW msec given above may be used. |  |  |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GPS Acquisition Assist - Information Elements appearing once per satellite

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| SatID | - | PRNs: 1, 11, 17, 20, 23, 28. |  |
| Doppler (0th order term) | Hz | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| Doppler (1storder term) | Hz/s | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| Doppler Uncertainty | Hz | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| Code Phase | chips | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| Integer Code Phase | - | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| GPS Bit number | - | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| Code Phase Search Window | chips | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| Azimuth | Degrees | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| Elevation | Degrees | Time varying. See file: Sig GPS Acquisition assist .csv (Note) |  |
| Azimuth LSB | Degrees | Time varying. See file: Sig GPS Acquisition assist .csv (Note) | Rel-10 onwards |
| Elevation LSB | Degrees | Time varying. See file: Sig GPS Acquisition assist .csv (Note) | Rel-10 onwards |
| Note: This field is “Time varying” and its value depends on the “current GPS TOW msec”. The value of this field to be used shall be determined by taking the “current GPS TOW msec” value and selecting the field value in the Sig GPS Acquisition assist.csv file corresponding to the value of “current GPS TOW msec”. |  |  |  |

# Assistance Data GANSS reference time

# GANSS reference time: sub-test 1

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Day |  | 5844 |  |
| GANSS Day Cycle Number |  | 0 | Rel-10 onwards |
| GANSS TOD | Seconds | 12645 s. Start time. Add integer number of 1 seconds as required. (Note) |  |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) |  |
| GANSS Time ID |  | 2 (GLONASS) |  |
| Note: GANSS TOD This is the value of GANSS TOD when the GNSS scenario is started in the GNSS simulator. The value of GANSS TOD to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current GANSS TOD” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.3. In the case that the (hardware) GNSS simulator is switched off or not present then the value of GANSS TOD given above may be used. |  |  |  |

# GANSS reference time: sub-test 2

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Day |  | 4515 |  |
| GANSS Day Cycle Number |  | 0 | Rel-10 onwards |
| GANSS TOD | Seconds | 1860. Start time. Add integer number of 1 seconds as required. (Note) |  |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) |  |
| GANSS Time ID |  | Not present (Galileo) |  |
| Note: GANSS TOD This is the value of GANSS TOD when the GNSS scenario is started in the GNSS simulator. The value of GANSS TOD to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current GANSS TOD” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.3. In the case that the (hardware) GNSS simulator is switched off or not present then the value of GANSS TOD given above may be used. |  |  |  |

# GANSS reference time: sub-test 9

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Day |  | 2191 |  |
| GANSS Day Cycle Number |  | 0 | Rel-12 onwards |
| GANSS TOD | Seconds | 1846. Start time. Add integer number of 1 second as required. (Note) |  |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) |  |
| GANSS Time ID |  | 3 (BDS system time) |  |
| Note: GANSS TOD This is the value of GANSS TOD when the GNSS scenario is started in the GNSS simulator. The value of GANSS TOD to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current GANSS TOD” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.3. In the case that the (hardware) GNSS simulator is switched off or not present then the value of GANSS TOD given above may be used. |  |  |  |

# Assistance Data GANSS reference UE position

# GANSS reference UE position

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Latitude sign |  | 0 |
| Degrees Of Latitude | degrees | 35.744287 |
| Degrees Of Longitude | degrees | 139.680176 |
| Altitude Direction |  | 0 |
| Altitude | m | 300 |
| Uncertainty semi-major | m | 3000 |
| Uncertainty semi-minor | m | 3000 |
| Orientation of major axis | degrees | 0 |
| Uncertainty Altitude | m | 500 |
| Confidence | % | 68 |

# Assistance Data GANSS ionospheric model

# GANSS ionospheric model

| Information Element | Units | Value/remark |
| --- | --- | --- |
| ai0 |  | 64.4 |
| ai1 |  | 0 |
| ai2 |  | 0 |
| Storm Flag 1 |  | 0 |
| Storm Flag 2 |  | 0 |
| Storm Flag 3 |  | 0 |
| Storm Flag 4 |  | 0 |
| Storm Flag 5 |  | 0 |

# Assistance Data GANSS additional ionospheric model

# GANSS additional ionospheric model (QZSS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Data Id |  | 00 |
| 0 | Seconds | 4.6566129 10E-9 |
| 1 | sec/semi-circle | 1.4901161 10E-8 |
| 2 | sec/(semi-circle)2 | -5.96046 10E-8 |
| 3 | sec/(semi-circle)3 | -5.96046 10E-8 |
| 0 | Seconds | 79872 |
| 1 | sec/semi-circle | 65536 |
| 2 | sec/(semi-circle)2 | -65536 |
| 3 | sec/(semi-circle)3 | -393216 |

# GANSS additional ionospheric model (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Data Id |  | 01 |
| 0 | Seconds | 4.6566129 10E-9 |
| 1 | sec/semi-circle | 1.4901161 10E-8 |
| 2 | sec/(semi-circle)2 | -5.96046 10E-8 |
| 3 | sec/(semi-circle)3 | -5.96046 10E-8 |
| 0 | Seconds | 79872 |
| 1 | sec/semi-circle | 65536 |
| 2 | sec/(semi-circle)2 | -65536 |
| 3 | sec/(semi-circle)3 | -393216 |

# Assistance Data GANSS time model

# GANSS time model (GLONASS)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Time Model Reference Time | s | 1860 (s) |  |
| TA0 | Seconds | 0 |  |
| GNSS_TO_ID |  | 0 (GPS) |  |
| Delta_T | Seconds | 15 | Rel-10 onwards |

# GANSS time model (Galileo)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Time Model Reference Time | s | 1860 (s) |  |
| TA0 | Seconds | 0 |  |
| GNSS_TO_ID |  | 0 (GPS) |  |
| Delta_T | Seconds | 0 | Rel-10 onwards |

# GANSS time model (BDS)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| GANSS Time Model Reference Time | s | 1860 (s) |  |
| TA0 | Seconds | 0 |  |
| GNSS_TO_ID |  | 0 (GPS) |  |
| Delta_T | Seconds | 13 | Rel-12 onwards |

# Assistance Data GANSS navigation model

# GANSS navigation model (Galileo)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Non-Broadcast Indication |  | Not present |

# Satellite Information (Galileo)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# Satellite Information (Fields occurring once per satellite) (Galileo)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SatID |  | SV IDs: 5, 10, 11, 18, 19, 20 |
| SV Health |  | 0 (Note) |
| IOD |  | 0 (Note) |
| Note: For consistency SV Health and IOD are also given in file: Sig GANSS Navigation model.csv |  |  |

# GANSS Clock Model (Fields occurring once per satellite) (Galileo)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Satellite clock model |  |  |
| toc | seconds | See file: Sig GANSS Navigation model.csv |
| af2 | sec/sec2 | See file: Sig GANSS Navigation model.csv |
| af1 | sec/sec | See file: Sig GANSS Navigation model.csv |
| af0 | sec | See file: Sig GANSS Navigation model.csv |
| TGD | sec | See file: Sig GANSS Navigation model.csv |
| SISA |  | See file: Sig GANSS Navigation model.csv |
| Model ID |  | See file: Sig GANSS Navigation model.csv |

# GANSS Orbit Model (Fields occurring once per satellite) (Galileo)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| toe | seconds | See file: Sig GANSS Navigation model.csv |
|  | semi-circles | See file: Sig GANSS Navigation model.csv |
| n | semi-circles/sec | See file: Sig GANSS Navigation model.csv |
| M0 | semi-circles | See file: Sig GANSS Navigation model.csv |
| OMEGAdot | semi-circles/sec | See file: Sig GANSS Navigation model.csv |
| e |  | See file: Sig GANSS Navigation model.csv |
| Idot | semi-circles/sec | See file: Sig GANSS Navigation model.csv |
| sqrtA | meters1/2 | See file: Sig GANSS Navigation model.csv |
| i0 | semi-circles | See file: Sig GANSS Navigation model.csv |
| OMEGA0 | semi-circles | See file: Sig GANSS Navigation model.csv |
| Crs | meters | See file: Sig GANSS Navigation model.csv |
| Cis | radians | See file: Sig GANSS Navigation model.csv |
| Cus | radians | See file: Sig GANSS Navigation model.csv |
| Crc | meters | See file: Sig GANSS Navigation model.csv |
| Cic | radians | See file: Sig GANSS Navigation model.csv |
| Cuc | radians | See file: Sig GANSS Navigation model.csv |

# Assistance Data GANSS additional navigation models

# GANSS additional navigation models (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Non-Broadcast Indication |  | Not present |

# Satellite Information (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# Satellite Information (Fields occurring once per satellite) (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SatID |  | Slot Numbers: 3, 4, 9, 10, 18, 20 |
| SV Health |  | 000000 (Note) |
| IOD |  | 225 (Note) |
| Note: For consistency SV Health and IOD are also given in file: Sig GANSS Additional Navigation model.csv |  |  |

# GANSS additional Clock Models (Fields occurring once per satellite) (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GLONASS Satellite Clock Model |  |  |
| n(tb) | seconds | See file: Sig GANSS Additional Navigation model.csv |
| n(tb) |  | See file: Sig GANSS Additional Navigation model.csv |
| n | seconds | See file: Sig GANSS Additional Navigation model.csv |

# GANSS additional orbit models (Fields occurring once per satellite) (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GLONASS Earth-Centered, Earth-fixed Parameters |  |  |
| En | days | See file: Sig GANSS Additional Navigation model.csv |
| P1 | minutes | See file: Sig GANSS Additional Navigation model.csv |
| P2 |  | See file: Sig GANSS Additional Navigation model.csv |
| M |  | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers/sec | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers/sec2 | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers/sec | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers/sec2 | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers/sec | See file: Sig GANSS Additional Navigation model.csv |
|  | kilometers/sec2 | See file: Sig GANSS Additional Navigation model.csv |

# GANSS additional navigation model (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Non-Broadcast Indication |  | Not present |

# Satellite Information (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# Satellite Information (Fields occurring once per satellite) (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SatID |  | SV IDs: 1, 2, 7, 18, 21, 27 |
| SV Health |  | 0 (Note) |
| IOD |  | 0 (Note) |
| Note: For consistency SV Health and IOD are also given in file: Sig GANSS Navigation model.csv |  |  |

# GANSS additional Clock Model (Fields occurring once per satellite) (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Satellite clock model |  |  |
| AODC |  | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| toc | seconds | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| a0 | sec | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| a1 | sec/sec | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| a2 | sec/sec2 | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| TGD1 | sec | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |

# GANSS additional Orbit Models (Fields occurring once per satellite) (BDS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| AODE |  | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| URA Index |  | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| toe | seconds | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| A1/2 | meters1/2 | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| e |  | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
|  | semi-circles | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| n | semi-circles/sec | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| M0 | semi-circles | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| 0 | semi-circles | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| _dot | semi-circles/sec | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| i0 | semi-circles | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| Idot | semi-circles/sec | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| Cuc | radians | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| Cus | radians | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| Crc | meters | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| Crs | meters | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| Cic | radians | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |
| Cis | radians | See file: Sig GANSS Additional Navigation model subtest 9_10.csv |

# Assistance Data GANSS reference measurement information

# GANSS reference measurement information: sub-test 1, 4 (Fields occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GANSS reference measurement information: sub-test 1, 4 (Fields occurring once per satellite)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| SatID |  | Slot Numbers: 3, 4, 9, 10, 18, 20 |  |
| Doppler (0th order term) | m/s | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) |  |
| Doppler (1storder term) | m/s2 | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) |  |
| Doppler Uncertainty | m/s | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) |  |
| Code Phase | ms | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) |  |
| Integer Code Phase | ms | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) |  |
| Code Phase Search Window |  | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) |  |
| Azimuth | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) |  |
| Elevation | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) |  |
| Azimuth LSB | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) | Rel-10 onwards |
| Elevation LSB | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest1_4.csv (Note) | Rel-10 onwards |
| Note: For sub-test 1: this field is “Time varying” and its value depends on the “current GANSS TOD”. The value of this field to be used shall be determined by taking the “current GANSS TOD” value and selecting the field value in the Sig GANSS reference measurement information subtest1_4.csv file corresponding to the value of “current GANSS TOD”. For sub-test 4: this field is “Time varying” and its value depends on the “current GPS TOW msec”. The value of this field to be used shall be determined by taking the “current GPS TOW msec” value and selecting the field value in the Sig GANSS reference measurement information subtest1_4.csv file corresponding to the value of “current GPS TOW msec”. |  |  |  |

# GANSS reference measurement information: sub-test 2, 8 (Fields occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GANSS reference measurement information: sub-test 2, 8 (Fields occurring once per satellite)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| SatID |  | SV IDs: 5, 10, 11, 18, 19, 20. |  |
| Doppler (0th order term) | m/s | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) |  |
| Doppler (1storder term) | m/s2 | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) |  |
| Doppler Uncertainty | m/s | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) |  |
| Code Phase | ms | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) |  |
| Integer Code Phase | ms | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) |  |
| Code Phase Search Window |  | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) |  |
| Azimuth | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) |  |
| Elevation | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) |  |
| Azimuth LSB | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) | Rel-10 onwards |
| Elevation LSB | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest2.csv (Note) | Rel-10 onwards |
| Note: This field is “Time varying” and its value depends on the “current GANSS TOD”. The value of this field to be used shall be determined by taking the “current GANSS TOD” value and selecting the field value in the Sig GANSS reference measurement information subtest2.csv file corresponding to the value of “current GANSS TOD”. |  |  |  |

# GANSS reference measurement information: sub-test 9, 10 (Fields occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GANSS reference measurement information: sub-test 9, 10 (Fields occurring once per satellite)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| SatID |  | SV IDs: 1, 2, 7, 18, 21, 27. |  |
| Doppler (0th order term) | m/s | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) |  |
| Doppler (1storder term) | m/s2 | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) |  |
| Doppler Uncertainty | m/s | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) |  |
| Code Phase | ms | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) |  |
| Integer Code Phase | ms | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) |  |
| Code Phase Search Window |  | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) |  |
| Azimuth | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) |  |
| Elevation | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) |  |
| Azimuth LSB | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) | Rel-12 onwards |
| Elevation LSB | Degrees | Time varying. See file: Sig GANSS reference measurement information subtest9.csv (Note) | Rel-12 onwards |
| Note: This field is “Time varying” and its value depends on the “current GANSS TOD”. The value of this field to be used shall be determined by taking the “current GANSS TOD” value and selecting the field value in the Sig GANSS reference measurement information subtest9.csv file corresponding to the value of “current GANSS TOD”. |  |  |  |

# Assistance Data GANSS almanac

# GANSS almanac: sub-test 1, 4 (Fields occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Week Number | Weeks | N/A |

# Satellite Information GLO-KP: sub-tests 1, 4

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 24 |

# GANSS almanac: sub-test 1, 4 (Fields occurring once per satellite)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| NA | Days | See file: Sig GANSS Almanac subtest1_4.csv |
| nA | - | Slot Numbers: 1, 2, 3 …….22, 23, 24 |
| HnA | - | See file: Sig GANSS Almanac subtest1_4.csv |
| nA | semi-circles | See file: Sig GANSS Almanac subtest1_4.csv |
| tnA | seconds | See file: Sig GANSS Almanac subtest1_4.csv |
| inA | semi-circles | See file: Sig GANSS Almanac subtest1_4.csv |
| TnA | sec/orbit period | See file: Sig GANSS Almanac subtest1_4.csv |
| T_DOTnA | sec/orbit period2 | See file: Sig GANSS Almanac subtest1_4.csv |
| nA |  | See file: Sig GANSS Almanac subtest1_4.csv |
| nA | semi-circles | See file: Sig GANSS Almanac subtest1_4.csv |
| nA | seconds | See file: Sig GANSS Almanac subtest1_4.csv |
| CnA |  | See file: Sig GANSS Almanac subtest1_4.csv |
| MnA |  | See file: Sig GANSS Almanac subtest1_4.csv |

# GANSS almanac: sub-test 2, 8 (Fields occurring once per message)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| Week Number | Weeks | 645 |  |
| Complete Almanac Provided |  | TRUE | Rel-10 onwards |

# GANSS almanac: sub-test 2, 8 (Field occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Toa |  | 147000 |
| IODa |  | 0 |

# Satellite Information KP: sub-test 2, 8

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 29 |

# GANSS almanac: sub-test 2, 8 (Fields occurring once per satellite)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SV ID |  | SV IDs: 1, 2, 3…21, 22, 23, 25, 26, 27…30 |
| e |  | See file: Sig GANSS Almanac subtest2.csv |
| i | semi-circles | See file: Sig GANSS Almanac subtest2.csv |
| OMEGADOT | semi-circles/sec | See file: Sig GANSS Almanac subtest2.csv |
| SV Health KP |  | See file: Sig GANSS Almanac subtest2.csv |
| delta A1/2 | meters1/2 | See file: Sig GANSS Almanac subtest2.csv |
| OMEGA0 | semi-circles | See file: Sig GANSS Almanac subtest2.csv |
| M0 | semi-circles | See file: Sig GANSS Almanac subtest2.csv |
|  | semi-circles | See file: Sig GANSS Almanac subtest2.csv |
| af0 | Seconds | See file: Sig GANSS Almanac subtest2.csv |
| af1 | sec/sec | See file: Sig GANSS Almanac subtest2.csv |

# GANSS almanac: sub-test 9, 10 (Fields occurring once per message)

| Information Element | Units | Value/remark | Release |
| --- | --- | --- | --- |
| Week Number | Weeks | 313 |  |
| Complete Almanac Provided |  | TRUE | Rel-12 onwards |

# GANSS almanac: sub-test 9, 10 (Field occurring once per message)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Toa |  | 147456 |
| IODa |  | 0 |

# Satellite Information KP: sub-test 9, 10

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 30 |

# GANSS almanac: sub-test 9, 10 (Fields occurring once per satellite)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| SV ID |  | SV IDs: 1, 2, 3, …, 29, 30 |
| e |  | See file: Sig GANSS Almanac subtest9.csv |
| i | semi-circles | See file: Sig GANSS Almanac subtest9.csv |
| OMEGADOT | semi-circles/sec | See file: Sig GANSS Almanac subtest9.csv |
| SV Health KP |  | See file: Sig GANSS Almanac subtest9.csv |
| delta A1/2 | meters1/2 | See file: Sig GANSS Almanac subtest9.csv |
| OMEGA0 | semi-circles | See file: Sig GANSS Almanac subtest9.csv |
| M0 | semi-circles | See file: Sig GANSS Almanac subtest9.csv |
|  | semi-circles | See file: Sig GANSS Almanac subtest9.csv |
| af0 | Seconds | See file: Sig GANSS Almanac subtest9.csv |
| af1 | sec/sec | See file: Sig GANSS Almanac subtest9.csv |

# Assistance Data GANSS auxiliary information

# GANSS auxiliary information (Fields occurring once per message) (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS-ID-3 |  | Present (GLONASS) |

# Aux Info List (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

# GANSS auxiliary information (Fields occurring once per satellite) (GLONASS)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Sat ID |  | Slot Numbers: 3, 4, 9, 10, 18, 20 |
| Signals Available |  | G1 |
| Channel Number |  | 5, 6, -2, -7, -3, 2 |

# GANSS auxiliary information (Fields occurring once per message) (multiple GPS signals)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS-ID-1 |  | Present (Modernized GPS) |

Aux Info List (multiple GPS signals)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Number of satellites | - | 6 |

GANSS auxiliary information (Fields occurring once per satellite) (multiple GPS signals)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| Sat ID |  | PRNs: 1, 11, 17, 20, 23, 28 |
| Signals Available |  | As supported by the UE |

Assistance Data GANSS ID

GANSS ID: sub-test 1, 4

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS ID |  | 3 (GLONASS) |

GANSS ID: sub-test 2, 8

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS ID |  | Not present (Galileo) |

GANSS ID: sub-test 3

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS ID |  | 1 (Modernized GPS) |

GANSS ID: sub-test 9, 10

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GANSS ID |  | 4 (BDS) |

#### 6.1.3.3	Default Assistance Data for TS 37.571-2 subclauses 7 and 9

This subclause defines the GNSS assistance data elements which shall be provided to the UE in certain tests in TS 37.571-2 [7] subclauses 7 and 9 in the LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message. The GNSS assistance data provided depends on the mode being used in the test case, the assistance data supported by the UE and the GNSS(s) supported by the UE. GNSS assistance data IEs not supported by the UE shall not be sent. GNSS assistance data IEs supported by the UE but not listed below shall not be sent.

Table 6.1.3.3-1: Default GNSS assistance data to be provided to the UE

| GNSS Assistance Data IE supported by the UE | Mode used in test case |  |  |
| --- | --- | --- | --- |
|  | UE-based | UE-assisted. GNSS-Acquisition Assistance supported by the UE | UE-assisted. GNSS-Acquisition Assistance not supported by the UE |
| GNSS-Reference Time | Yes | Yes | Yes |
| GNSS-ReferenceLocation | Yes | No | Yes |
| GNSS-IonosphericModel | Yes | No | No |
| GNSS-TimeModelList | Yes(1) | No | Yes(1) |
| GNSS-NavigationModel | Yes | No | Yes |
| GNSS-AcquisitionAssistance | No | Yes | No |
| GNSS-Almanac | No | No | Yes |
| GNSS-UTC-Model | Yes(2) | Yes(2) | Yes(2 |
| GNSS-AuxiliaryInformation | Yes(3) | Yes(3) | Yes(3) |
| Note1: Only if more than one GNSS supported by the UE Note2: Only if GLONASS and at least one other GNSS supported by the UE. Note3: Only if GLONASS supported by the UE, and/or if the UE supports multiple GPS signals. |  |  |  |

#### 6.1.3.4	Assistance Data values for TS 37.571-2 subclauses 7 and 9

Where assistance data is required on a per-satellite basis, or where the values of the data also varies with time it is specified in comma-separated-variable files in the GNSS data zip file specified in Annex B. These files specify the values to be used for each satellite, indexed by satellite SV ID, and, where applicable, the values to be used indexed by both time and satellite SV ID.

Assistance data that is marked as “time varying” and the gnss-TimeOfDay field are only specified and used in 1 second increments. Interpolation between these values shall not be used.

The accuracy of the gnss-TimeOfDay and assistance data that is marked as “time varying” in the provided assistance data shall be within +/- 2 s relative to the GNSS time in the system simulator. In the case that assistance data is required but satellite signals are not required then this clause does not apply.

Assistance data Information Elements and fields that are not specified shall not be used.

The information elements detailed below are fully defined in TS 37.355 [8]